3GPP TSG-RAN2 Meeting # 120	R2-2212723

# Toulouse, France, Nov 14th - Nov 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3727 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC for NR Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In MAC spec, when the max number of consecutive DTX was detected, it will indicates HARQ-based sidelink RLF detection to RRC: #### 5.22.1.3.3 HARQ-based Sidelink RLF detection …… The Sidelink HARQ Entity shall for each PSFCH reception occasion associated to the PSSCH transmission: 1> if PSFCH reception is absent on the PSFCH reception occasion: 2> increment numConsecutiveDTX by 1; 2> if numConsecutiveDTX reaches sl-maxNumConsecutiveDTX: 3> indicate HARQ-based Sidelink RLF detection to RRC.. However, in RRC spec, the RLF is judged based on “upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached” , as below: #### 5.8.9.3 Sidelink radio link failure related actions The UE shall: 1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or 1> upon T400 expiry for a specific destination; or 1> upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or ……. The current MAC and RRC spec is not aligned. When RRC receives HARQ-based Sidelink RLF detection from MAC layer, it is hard to determine whether the maximum number of consecutive HARQ DTX for a specific destination has been reached since in RRC layer it is hard to link the “HARQ-based sidelink RLF” with the “the maximum number of consecutive HARQ DTX for a specific destination has been reached”. |
| Summary of change: |  | In subclause 5.8.9.3, change “upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached” to “upon indication of HARQ-based Sidelink RLF detection from MAC entity” Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR sidelink Inter-operability: If network implements the change but not the UE, there is no inter-operability issue. If the UE implements the change but not the network, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The current MAC and RRC spec is not aligned. - It is hard to link the “HARQ-based sidelink RLF” with the “the maximum number of consecutive HARQ DTX for a specific destination has been reached” in RRC layer, it may result that UE cannot judge whether RLF is happened once it receives HARQ-based sidelink RLF from MAC layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

5.8.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication of HARQ-based Sidelink RLF detection from MAC entity; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in clause 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

End of Change