3GPP TSG-RAN WG2 Meeting #120	 R2-2212739

Toulouse, France, 14th – 18th November 2022

Agenda Item:    8.13.7

Source:         Xiaomi

Title:         	Discussion on the SON/MDT enhancement for NPN

Document for:	Discussion and Decision

Introduction

Currently, the data collection enhancement in NR for SON/MDT purpose is only supported in the PLMN scenario while that in the NPN scenario is not supported. In R18, the SON/MDT enhancement for NPN is to be supported for the network optimization. The agreements achieved in the last meeting on the SON/MDT enhancement for NPN scenario are as follows [1]:

1	SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

2	Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case.

3	RAN2 prioritizes the use cases of RLF report and logged MDT enhancement for NPN.

In this contribution, we provide our considerations on the support of SON/MDT enhancement for NPN.

Discussion

In the logged MDT for PLMN, the MDT PLMN list can be configured in the logged MDT configuration to indicate a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval.

To support the logged MDT for NPN, the similar mechanism can also be applied so as network can control UE to collect the MDT measurements in certain NPNs, i.e. the existing LoggedMeasurementConfiguration can be extend to include the MDT NPN list, only when the registered NPN is part of this set of NPNs, UE can performs measurement logging as well as the availability indication and logged MDT report retrieval.

Proposal 1: Introduce the MDT NPN list in the logged MDT configuration to indicate the NPNs where measurement collection and log reporting is allowed.

Furtherly, in addition to the restriction of MDT PLMN list, the logging area can also be indicated in the logged MDT configuration, consisting of the list of target cells/tracking areas where UE is allowed to perform measurement logging within the configured MDT PLMN list.

Similar to the logged MDT for PLMN, to support the measurement logging in NPN, network can also indicate the target NPN cells/tracking areas list to restrict the logging to specific area within the configured MDT NPN list.

Proposal 2：Introduce the NPN related information in the logging area configuration to restrict the measurement logging to specific NPN cell/tracking areas.

Currently, if there is no suitable cell to camp on, UE will enter to any cell selection state, and provide the OOC indication to network (e.g. anyCellSelectionDetected, noSuitableCellFound) to identify the coverage hole. In NPN, UE operating in SNPN access mode or with CAG-only indication can only camp on the cell which belongs to SNPN or UE allowed CAG ID during the mobility in IDLE/INACTIVE/CONNECTED mode. Otherwise, UE will enter to any cell selection state. In other words, a UE may experience the out of coverage situation due to the access mode. In order to identify the coverage hole in NPN, the OOC indication can be supported for NPN and UE can report the NPN access mode for the OOC analysis.

Observation 1: For UE operating in SNPN access or with CAG-only indication, it may experience OCC due to access mode limitation.

Proposal 3: UE access mode (e.g. SNPN access mode or with CAG-only indication) can be reported to network for OOC analysis.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation 1: For UE operating in SNPN access or with CAG-only indication, it may experience OCC due to access mode limitation.

Proposal 1: Introduce the MDT NPN list in the logged MDT configuration to indicate the NPNs where measurement collection and log reporting is allowed.

Proposal 2：Introduce the NPN related information in the logging area configuration to restrict the measurement logging to specific NPN cell/tracking areas.

Proposal 3: UE access mode (e.g. SNPN access mode or with CAG-only indication) can be reported to network for OOC analysis.

References

R2-220xxx, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online.