3GPP TSG-RAN WG2 Meeting #120	R2-2212778

# Toulousse, France, 14th- 18th November, 2022

Agenda Item:	8.6.3.1

Source:	Ericsson

Title:	Triggering neighbour cell measurements prior to RLF

Document for:	Discussion, Decision

# 1	Introduction

In Release 18, a work item on IoT NTN was approved in [1] including the objective highlighted below on mobility enhancements:

4.1.2	Mobility enhancements

The following mobility enhancements objectives are listed.

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

- Define UE RRM core requirements for the above mobility enhancement features [RAN4].

In this paper we discuss how neighbour cell measurements can be triggered before RLF so that time to re-establish connection can be shortened for NB-IoT and LTE-M devices in NTN.

# 2	Discussion

## 2.1	Neighbour cell measurements prior to RLF

### 2.1.1	Background

In Release 17, there was a WI to introduce additional enhancements for NB-IoT and LTE-M UEs. The following was one of the objectives [2]:

- Specify signalling for neighbour cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

UE performs a cell search to initiate the RRC connection re-establishment procedure when RLF is declared. The procedure allows the retrieval of the UE context and the recovery of undelivered data. Considering that it may be beneficial for a UE to reduce the time it takes to perform RRC reestablishment to another cell to avoid service interruption, RAN2 introduced a mechanism in Rel-17 to assist an NB-IoT UE to trigger cell search when it is likely to declare RLF. It is expected that such assistance can facilitate performing the cell search faster and thus reduce the time it takes to re-establish an RRC connection especially when inter-frequency cells are considered. An important aspect that was considered is the criteria defined so that neighbour cell measurements are triggered only when RLF is to be declared due to “mobility” This was to avoid any impact on the battery life performance of UEs that are “stationary”. Network provides such criteria, based on a combination of serving cell quality threshold and variance, to a UE regarding when to start measurements to accelerate cell search once RLF is declared. Legacy relaxed monitoring criteria is used as a baseline to address the variance part of the criteria.

The mechanism introduced in Rel-17 is to assist an NB-IoT UE to trigger cell search when it is likely to declare RLF so that it can facilitate performing the cell search faster and thus reduce the time it takes to re-establish an RRC connection.

Network broadcasts separate thresholds for intra- and inter-frequency measurements, i.e., s-MeasureIntra and s-MeasureInter respectively, in system information (SIB3-NB). Intra- or inter-frequency neighbour cell measurements are performed when NRSRP is below the broadcast threshold and UE is not in relaxed neighbour cell monitoring state, i.e., UE is mobile or at the cell edge.

In Rel-17 intra- or inter-frequency neighbour cell measurements are performed when NRSRP is below the broadcast threshold and UE is not in relaxed neighbour cell monitoring state, i.e., UE is mobile or at the cell edge.

Relaxed neighbour cell monitoring state in connected mode is similar to relaxed monitoring state in idle mode and the objective is to estimate whether the UE is stationary or has low mobility. Network may provide such criteria in addition to the thresholds provided in SIB3-NB considering that these measurements may unnecessarily increase power consumption for stationary devices once triggered. The criteria is provided in neighCellMeasCriteria. It is based on a combination of serving cell quality and variance so that these neighbour cell measurements are only triggered by UEs that are not stationary. The following parameters were specified in System Information (SIB3-NB):

- t-MeasureDeltaP: duration of time where the UE is not required to perform neighbour cell measurements in RRC_CONNECTED.

- s-MeasureDeltaP: change threshold in serving NRSRP to trigger neighbour cell measurements in RRC_CONNECTED.

UE can perform the measurements only when timer T326 is running. This timer is (re-)started with the value t-MeasureDeltaP when following conditions are fulfilled:

- Upon transition to RRC_CONNECTED, unless the UE was in relaxed monitoring state in RRC_IDLE.

- After a measurement in RRC_CONNECTED, when the drop in NRSRP is over a threshold. NRSRPRef-NRSRP-nrsPowerOffsetNonAnchor>sMeasureDeltaP

The reference NRSRP is updated with the current NRSRP value whenever the timer starts running.

In NB-IoT, handover mechanism is not supported so mobility can only be triggered by RLF in connected mode. This is not the case for LTE-M, which is why this optional mechanism that provides indication from the network for UEs to trigger neighbor cell measurements in order to reduce the time it takes to re-establish RRC connection was specified only for NB-IoT. It is a feature specified for NB-IoTs in terrestrial networks that enhances the RLF/RRC connection re-establishment mechanism which is supported by both NB-IoT and LTE-M UEs.

In Rel-17, the feature specified for NB-IoTs in terrestrial networks (TN), enhances the RLF/RRC connection re-establishment mechanism that is supported by both NB-IoT and LTE-M UEs.

In Rel-18, the objective is to introduce a mechanism for both NB-IoT and LTE-M UEs in NTN based on the mechanism specified for NB-IoT UEs in TN. The intention is to enhance the RLF/RRC connection re-establishment mechanism which is supported by both NB-IoT and LTE-M in NTN.

In Rel-18, the objective is to introduce a mechanism for both NB-IoT and LTE-M UEs in NTN based on the mechanism specified for NB-IoT UEs in TN.

### 2.1.2	Enhancements in NTN

In Release 17, the criteria mentioned in section 2.1.1 above were introduced based on the following:

- As the UE moves towards the cell border, there is a clear difference in RSRP compared to the cell centre.

- UEs can be either static or mobile.

Regarding the first bullet; such observation may not be made in NTN cells [4], where the difference in signal strength is small between two beams in a region where there is overlap. Smaller margins would impose tighter power thresholds (s-MeasureDeltaP, s-MeasureIntra and s-MeasureInter) that could lead to triggering neighbour cell measurements unnecessarily.

In NTN, difference in signal strength is small between the cell edge and the cell centre.

Regarding the second bullet; a non-terrestrial network is characterized as an always-moving entity (e.g., the satellite or NTN payload), so from network perspective, all UEs can be considered as mobile regardless of whether a UE is in fact physically stationary. For instance, a static IoT device such as a sensor installed on a lamp post would observe that serving cell stops serving the area at some point in time similar to a mobile device. Therefore, the criteria specified in Release 17 are not sufficient for NTN scenarios and new criteria need to be considered to trigger neighbour cell measurements.

In NTN, all UEs can be considered mobile due to the always-moving NTN payload.

In RAN2#119-e, RAN2 agreed to “continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g., the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)”. In the same meeting, RAN2 also agreed that “IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g., triggers) for both NB-IoT and eMTC.”. Considering that Rel-17 mechanism is already agreed as the baseline mechanism for both NB-IoT and LTE-M, and the benefits of time and location-based criteria with respect to the RSRP based criteria in NTN we propose the following:

Introduce time-based criteria to trigger neighbour cell measurements in connected mode for both NB-IoT and LTE-M in NTN.

In NTN, a UE may have information on the characteristics of the serving cell, e.g., its stop serving time, cell reference location, cell radius or satellite ephemeris. For instance, in the quasi-fixed earth cell scenario, UE knows when the serving cell will stop serving using the parameter t-service broadcast in System Information. This information can be complemented with a network configurable distance threshold to decide when UE may trigger neighbour cell measurements.

In NTN, network informs UEs about the characteristics of a cell (e.g., serving time, footprint) via system information broadcast.

Introduce distance-based criteria to trigger neighbour cell measurements in connected mode for both NB-IoT and LTE-M at least for quasi-fixed earth cells in NTN.

# 3	Conclusions

In this paper we discussed how neighbour cell measurements can be triggered before RLF so that time to re-establish connection can be shortened for NB-IoT and LTE-M devices in NTN. We made the following observations:

Observation 1	The mechanism introduced in Rel-17 is to assist an NB-IoT UE to trigger cell search when it is likely to declare RLF so that it can facilitate performing the cell search faster and thus reduce the time it takes to re-establish an RRC connection.

Observation 2	In Rel-17 intra- or inter-frequency neighbour cell measurements are performed when NRSRP is below the broadcast threshold and UE is not in relaxed neighbour cell monitoring state, i.e., UE is mobile or at the cell edge.

Observation 3	In Rel-17, the feature specified for NB-IoTs in terrestrial networks (TN), enhances the RLF/RRC connection re-establishment mechanism that is supported by both NB-IoT and LTE-M UEs.

Observation 4	In Rel-18, the objective is to introduce a mechanism for both NB-IoT and LTE-M UEs in NTN based on the mechanism specified for NB-IoT UEs in TN.

Observation 5	In NTN, difference in signal strength is small between the cell edge and the cell centre.

Observation 6	In NTN, all UEs can be considered mobile due to the always-moving NTN payload.

Observation 7	In NTN, network informs UEs about the characteristics of a cell (e.g., serving time, footprint) via system information broadcast.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce time-based criteria to trigger neighbour cell measurements in connected mode for both NB-IoT and LTE-M in NTN.

Proposal 2	Introduce distance-based criteria to trigger neighbour cell measurements in connected mode for both NB-IoT and LTE-M at least for quasi-fixed earth cells in NTN.

# References

RP-221806, WID on IoT NTN enhancements, MediaTek, RAN#96, June 6-9, 2022

RP-220529, Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN#95e, March 2022.

TS 36.331, Radio Resource Control (RRC) protocol specification, V17.1.0, June 2022.

TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0, June 2021.