3GPP TSG-RAN WG2 Meeting #120                                  R2- 2212787

Toulouse, France, 14th – 18th November, 2022

Agenda Item	: 8.5.4.1 (FS_NR_XR_enh)

Source	: LG Electronics Inc.

Title	: Discussion on BSR enhancement for timing information in XR

Document for	: Discussion and Decision

Introduction

In the RAN2#119bis-e meeting [1], it is agreed to consider delay information as a feedback information for XR traffic.

Delay information consists of at least “remaining time”. 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

In this contribution, we show our views on delay information and the BSR enhancement for delay information.

Discussion

Delay information reporting

According to XR SID [2], the UL XR traffic including pose/control update and video stream has relatively stringent PSDB requirements:

Many of the XR and CG use cases are characterised by quasi-periodic traffic (with possible jitter) with high data rate in DL (i.e., video steam) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB). Hence, there is a need to study and potentially specify possible solutions to better support such challenging services.

That is, if the UL data is not delivered to the network in time, the UL data would be discarded even though the UE receives the corresponding uplink grant to transmit the UL data. In this case, the packet delivery success rate for XR traffic would be degraded, causing frozen XR service or display and deteriorated user experience. Thus, satisfying strict PSDB requirement for XR service should be considered, i.e., XR data should be delivered before exceeding PSDB requirement.

Observation 1. If UL data for pose/control/video stream is not arrived in time, XR service or display would be frozen and user experience would be terribly deteriorated.

In order to schedule the XR traffic before the required PSDB, it is agreed that delay information is useful for the XR traffic. However, whether to provide the delay information using dynamic reporting (e.g., via BSR) is remained as FFS:

2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

In the current MAC specification, the BSR is not always triggered when the new data is generated. In addition, the current BSR only includes the amount of available UL data for each LCG. That is, even though the network knows the PSDB for each PDU set, when the network receives the BSR, the network knows only the amount of UL data but does not know how much time is left for the UL data until the required PSDB. In this case, if the uplink scheduling is not allocated within the required PSDB, e.g., due to the network scheduling policy, the XR performance would be degraded as well as the UL resource would be wasted since the corresponding data is dropped.

Therefore, the PSDB information is not sufficient and the BSR enhancement is needed for delay information reporting. In addition, the delay information should be defined as remaining time (e.g., remaining PSDB for the PDU set) in order to let the network know how much time is left for the UL data until the required PSDB.

Proposal 1. RAN2 study on the BSR enhancement in order to imply the remaining time information.

BSR format with remaining time information

However, whether to include the remaining time in the BSR format is another issue. Regarding the BSR format with the remaining time information, followings could be considered:

- Option 1: In the new BSR MAC CE format, the remaining time field is explicitly included. For example, when each buffered data has different remaining time, the BSR format includes one or more the remaining time field(s) and how much data is buffered for each remaining time(s).

- Option 2: In the new BSR MAC CE format, the amount of UL data only with short remaining time (e.g., remaining time is less than the predetermined value) is included.

When the remaining PDB is explicitly provided in the BSR MAC CE format (i.e., Option 1), the network may have flexibility to schedule the uplink grant considering the remaining time until the exceeding the PDB. However, in Option 1, the BSR MAC CE format should include multiple remaining time fields and the multiple Buffer size fields for each remaining time, in order to distinguish how much data is buffered for which remaining time value, as agreed in RAN2#119bis-e meeting. Therefore, the Option 1 would be complicated as well as the size of the BSR MAC CE would be large. In addition, when the BSR is retransmitted without rebuilding the MAC PDU, e.g., due to HARQ retransmission, the value of remaining time would not be accurate, which causes the misalignment between the uplink grant scheduling and the PSDB requirement of the UL data. Therefore, it should be carefully reviewed whether the exact value of remaining time is really needed in the BSR format.

In our view, in order to simply define the BSR format with delay information, new BSR format should only include the information on amount of data which is needed to be transmitted immediately (i.e., Option 2). For example, The option 2 may include the amount of data with short remaining time for each LCG, which is similar to the current BSR format. In this case, the remaining time information could be implicitly indicated, i.e., by configuring the value to determine whether the uplink data has short remaining PDB.

Proposal 2. RAN2 should study on new BSR format to include the amount of available data with short remaining time.

New triggering condition

In the current specification, BSR is triggered when the data is newly generated in a high-priority logical channel in an LCG (for Regular BSR), periodically (for Periodic BSR), or when there is padding bits (for Padding BSR). However, given that the BSR is not always triggered when the new data is arrived at the UE, current BSR triggering condition is not enough to serve the delay-sensitive traffic. For example, when the additional data with strict PDB is generated in the non-empty logical channel or in the logical channel with relatively low priority, the uplink scheduling for this additional data would be delayed since the Regular/Periodic BSR may not be triggered right away and the network may not know information on this additional data in time. As a result, the UE may receive the UL grant after the required PSDB of the UL data, which causes data loss of XR traffic.

Observation 2. Current BSR triggering condition is not enough to support the delay-stringent XR traffic.

Therefore, in order to support the delay stringent XR data, new BSR triggering condition should be studied in order to request the uplink scheduling in time. For example, when the remaining time of buffered data is lower than the predetermined value, the UE may trigger the BSR in order to request the uplink resource immediately.

Proposal 3. RAN2 should study on new BSR trigger condition (e.g., when the remaining time of buffered data is short) in order to request the uplink resource scheduling in time.

Conclusion

In this contribution, we show our views on BSR with delay information for XR traffic.

Observation 1. If UL data for pose/control/video stream is not arrived in time, XR service or display would be frozen and user experience would be terribly deteriorated.

Observation 2. Current BSR triggering condition is not enough to support the delay-stringent XR traffic

Based on the above observations, followings are proposed.

Proposal 1. RAN2 study on the BSR enhancement in order to imply the remaining time information.

Proposal 2. RAN2 should study on new BSR format to include the amount of available data with short remaining time

Proposal 3. RAN2 should study on new BSR trigger condition (e.g., when the remaining time of buffered data is short) in order to request the uplink resource scheduling in time.

Reference

[1] R2-2210802, Report from session on LTE legacy, 71 GHz, DCCA, Multi-SIM, RAN slicing, QoE and XR [2] RP-220285, Revised SID : Study on XR Enhancements for NR