3GPP TSG-RAN WG2 #120	R2-2212796

Toulouse, France, 14 - 18 Nov, 2022

Source:	NTT DOCOMO, INC.

Title:	Assistance information for cell reselection

Document for:	Discussion and decision

Agenda Item:	8.3.4

- Introduction

In RAN2#119bis-e, Network Energy Saving solutions were discussed and continued in post-meeting email discussion. Based on company input, the Rapporteur summary [1] of the remaining issues on cell selection/reselection are:

List of remaining issues on cell selection/reselection: (De)prioritize NES cells by NES capable UEs: - Whether de-prioritization is sufficient for NES cells, or even prioritization of NES cells need to be supported - Applicability of existing mechanisms, e.g. frequency priorities, cell offset to (de)prioritize cells - Potential new mechanism description, and potential specification impacts - Mechanisms to incentivize and disincentivize NES-capable UEs from camping on cells according to their NES states

This contribution presents our view on UE assistance information in cell selection/re-selection to de-prioritize the cell from a DTX/DRX perspective.

- Discussion

The DTX / DRX methodology is an effective way to reduce wasted power consumption by defining a time period during which the network is asleep, thereby reducing idle time. However, it should be considered that user traffic cannot be sent or received while the network is asleep, especially with periodic DTX / DRX methods, which can have a negative impact on the user experience as delay and jitter on user traffic. Some user traffic types, especially IoT devices, may send and receive periodically, even though the amount of traffic may be small. There are also user applications, even if not IoT devices, that do not like delay and jitter. In such cases, the NW should configure the NW DTX/DRX periodicity to be smaller value so that the impact of NW DTX/DRX will be smaller.

Observation 1. If the user application performs periodic traffic and prefers not to have delay or jitter, the NW should configure the DTX/DRX periodicity to be smaller value so that the impact of NW DTX/DRX will be smaller.

On the other hand, there may be some UEs that do not have a problem even if the network is asleep for a longer period of time. For NES cells composed of such UEs, there is a possibility to gain more NES gain without compromising the user experience by setting the NW DRX/DTX pattern according to the preference from the UEs. In such cases, the NW can configure the NW DTX/DRX periodicity to be larger value so that the NES gain of NW DTX/DRX will be larger.

Observation 2 If the user application can be very receptive to delay and jitter, the NW can configure the DTX/DRX periodicity to be larger value so that the NES gain of NW DTX/DRX will be larger.

Therefore, by defining DRX/DTX pattern according to user traffic, both User experience and NES gain can be achieved. For this purpose, it is useful for UEs which operate applications to send a request to the NW about the preferred NW DTX/DRX pattern (i.e., DTX periodicity and DRX periodicity) as UEAssistanceInformation.

Based on the notified preferred DTX/DRX pattern, the NW can de-prioritize the cell which DTX/DRX pattern is not met with preferred DRX/DTX pattern reported from UE to prevent user experience degradation.

Proposal 1. RAN2 to confirm it is useful for UEs which operate applications to send a request to the NW about the preferred NW DTX/DRX pattern (i.e., DTX periodicity and DRX periodicity) as UEAssistanceInformation in order to de-prioritize the cell which DTX/DRX pattern is not met with preferred by UE.

- Summary and proposal

Observation 1. If the user application performs periodic traffic and prefers not to have delay or jitter, the NW should configure the DTX/DRX periodicity to be smaller value so that the impact of NW DTX/DRX will be smaller.

Observation 2 If the user application can be very receptive to delay and jitter, the NW can configure the DTX/DRX periodicity to be larger value so that the NES gain of NW DTX/DRX will be larger.

Proposal 1. RAN2 to confirm it is useful for UEs which operate applications to send a request to the NW about the preferred NW DTX/DRX pattern (i.e., DTX periodicity and DRX periodicity) as UEAssistanceInformation in order to de-prioritize the cell which DTX/DRX pattern is not met with preferred by UE.

- References

[1] R2-2211066, “Report of [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less (Huawei),” Huawei, HiSilicon, RAN2#120