3GPP TSG-RAN WG2 Meeting #120						  R2-2212836

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.24.1.1

Source:			MediaTek Inc.

Title:	On size reduction benefit in new signaling proposal of FBG5

Document for:	Discussion and Decision

- Introduction

In RAN2#119bis-e meeting companies discussed the capability signaling proposal from the RAN4 LS [1] but did not achieve any agreement [2].

This document intends to help aligning companies view on legacy fallback rules, also to discuss the benefit of new signaling proposal and potential problems for the new IE proposal from RAN4.

- Discussion

  - RAN2 interpretation of RAN4 fallback requirement for intra-FBG

Given a compliant UE supports intra-band continuous CA_n46M by BCS0, CA_n46N by BCS0 and CA_n46O by BCS0 in the Table 1 below, and all other capabilities except for CC bandwidth within the feature set are the same for all CCs.

Table 1: FBG3 configurations for the intra-band CCA of n46 [3]

| NR CA config. | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Channel BWs (MHz) | Max. aggregated BW (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46M | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |

Then the UE compiles the CA capabilities for supportedBnadCombinationList by respecting current fallback band combination definition in the Table 2, it advertises at least following band combinations since CA#2 is obviously not a fallback of CA#1:

CA#1 = CA_n46O, BCS0, FS: {[60], [40], [40], [40], [40]}

CA#2 = CA_n46N, BCS0, FS: {[80], [40], [40], [40]}

FS stands for Feature Set.

Table 2: Fallback band combination definition [5]

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL. A PC5 band combination that would result from another PC5 band combination (parent band combination) by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. A fallback band combination supports the same channel bandwidth(s) for each carrier as its parent band combination(s).

When the network wants to adopt the parent band combination CA#1, that requires the UE to support the fallback CA#2’ = CA_n46N, undefined BCS, FS: {[60], [40], [40], [40]} because the network can establish CA#1 by adding SCell one by one. The derived fallback CA#2’ is just an intermediate configuration between PCell only (initial state) and CA#1 (final state) and still belongs to the CA bandwidth class N (4CC). Therefore we understand that such a UE implementation is still complaint to the RAN4 fallback requirement in the Table 3. Same understanding holds even the UE does not advertise the CA#2, but the CA#1 only.

Table 3: Fallback requirement note in RAN4 CA configurations tables [3]

NOTE 2:	It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

- The principles how a fallback is determined and configured from a parent CA are already clarified in the RAN2 fallback definitions. Within an FBG, RAN4 fallback requirement works no confliction with RAN2 fallback definitions in practical deployment today.

Same way is applied to proceed the fallback case for the CA bandwidth class M. In practical CA operation, two possible fallbacks could be derived by the UE and the network:

CA#3’ = CA_n46M, undefined BCS, FS: {[80], [40], [40]}

(fallback from CA#2 by releasing 1CC)

CA#3’’ = CA_n46M, BCS0, FS: {[60], [40], [40]}	(fallback from CA#1 by releasing 2CC)

It is noteworthy that same situation (undefined BCS) is seen in the derived CA#3’. Not even its maximum aggregated bandwidth (160MHz) could be accommodated by the RAN4 defined value (140MHz) in the CA_n46M, BCS0, but it is still within the support range of the CA bandwidth class M defined in the Table 4:

Table 4: NR CA bandwidth classes [3]

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |
| NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. |  |  |  |

- Although the BCS of a derived fallback could be undefined in the RAN4 CA configurations table but it is still technically accommodated for the bandwidth class where it belongs.

For the sake of discussion progress, we will support to send LS to RAN4 if companies really think it is necessary. But we think it is enough to inform the RAN2 observations for reference only since there is nothing broken in RAN2 fallback definitions. It is up to RAN4 whether to tidy up the inconsistency / ambiguity between different rows (BCS level) in their CA configurations table.

- For cross-WG alignment of fallback rules, RAN2 to consider sending LS to RAN4 for the observation: UE would need to support an undefined BCS of a certain CA if it is a fallback derived from a parent CA by following RAN4 fallback requirement within an FBG.

  - FBG5 example

Given a UE supports following 4 intra-band FBG5 CA band combinations:

FBG5-CA1: n260R6 {[100MHz][100MHz][100MHz][100MHz][100MHz][100MHz]}

FBG5-CA2: n260R5 {[200MHz][100MHz][100MHz][100MHz][100MHz]}

FBG5-CA3: n260R4 {[200MHz][200MHz][100MHz][100MHz]}

FBG5-CA4: n260R3 {[200MHz][200MHz][200MHz]}

All possible BCSs are shown in the Table 5 and the one-to-one relationship marked by the same bright colors. These light-colored cells are marked as the derived fallbacks from their parent CAs in bright colors, and the case the same BCS has two or more different light colors indicates that it can be derived from two or more parent CAs.

Table 5: Bandwidth combination sets for bandwidth classes R6 to R2 in FBG5

| Aggre. BW | R6 |  | R5 |  | R4 |  | R3 |  | R2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 200 | 100 | 200 | 100 | 200 | 100 | 200 | 100 | 200 |  | 100 |
| 1200 | 6 |  |  |  |  |  |  |  |  |  |  |
| 1100 | 5 | 1 |  |  |  |  |  |  |  |  |  |
| 1000 | 4 | 2 | 5 |  |  |  |  |  |  |  |  |
| 900 | 3 | 3 | 4 | 1 |  |  |  |  |  |  |  |
| 800 | 2 | 4 | 3 | 2 | 4 |  |  |  |  |  |  |
| 700 | 1 | 5 | 2 | 3 | 3 | 1 |  |  |  |  |  |
| 600 |  | 6 | 1 | 4 | 2 | 2 | 3 |  |  |  |  |
| 500 |  |  |  | 5 | 1 | 3 | 2 | 1 |  |  |  |
| 400 |  |  |  |  |  | 4 | 1 | 2 | 2 |  |  |
| 300 |  |  |  |  |  |  |  | 3 | 1 |  | 1 |
| 200 |  |  |  |  |  |  |  |  |  |  | 2 |

  - Legacy signaling with fallback rule

    - Other capabilities except for CC BW within the feature set are the same for all CCs

By current fallback rules, the UE would only report 4 supported CA band combinations CA1, CA2, CA3, and CA4 if all other capability parameters except for CC bandwidth within the feature set are the same for all CCs. The signaling size is small by benefiting from skipping all fallbacks (those CAs in the light-colored cell of the Table 4; e.g., n260R5: {5x100} is a fallback of the FBG5-CA1; n260R2: {1x200 + 1x100} is a fallback of not only the FBG5-CA2, but also the FBG5-CA3, and so on) as CA band combinations and FeatureSetCombinations shown in the Table 6.

Table 6: CABCs in the supportedBandCombinationList and their FeatureSetCombinations

| FBG5-CA1 |  |  |  |  |  | FBG5-CA2 |  |  |  |  | FBG5-CA3 |  |  |  | FBG5-CA4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandNR = n260 ca-BandwidthClassDL = R6 |  |  |  |  |  | bandNR = n260 ca-BandwidthClassDL = R5 |  |  |  |  | bandNR = n260 ca-BandwidthClassDL = R4 |  |  |  | bandNR = n260 ca-BandwidthClassDL = R3 |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | CC1 | CC2 | CC3 | CC4 | CC5 | CC1 | CC2 | CC3 | CC4 | CC1 | CC2 | CC3 |
| 100 | 100 | 100 | 100 | 100 | 100 | 200 | 100 | 100 | 100 | 100 | 200 | 200 | 100 | 100 | 200 | 200 | 200 |

    - Other capabilities are not all the same

But for practical production, there are UE design constraints to make other capabilities are not the same for all CCs (i.e., greater diversity in UE capabilities). A typical example is MIMO capability, given that there are 12 MIMO layers in total can be shared by all CCs, the UE would explicitly advertise all fallbacks with different capabilities for maximizing the interoperability and data performance. Assume worst case that all CCs are with different capabilities from each other, the combination of feature sets could be very complicated because it needs to contain all possible permutations like the fallback combinations shown in the Table 8. (For FBG5-CA6 and FBG5-CA7, the UE even needs to repeat the parent FS itself with different MIMO capability parameters.)

Table 7: Legend of the Table 8 in MIMO part.

|  | FSDLperCC w/ 2-layer MIMO |
| --- | --- |
|  | FSDLperCC w/ 4-layer MIMO |

Table 8: A worst case for reporting all fallbacks with different capabilities

| FBG5-CA5 |  |  |  |  |  | FBG5-CA6 |  |  |  |  | FBG5-CA7 |  |  |  | FBG5-CA8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandNR = n260 ca-BandwidthClassDL = R6 |  |  |  |  |  | bandNR = n260 ca-BandwidthClassDL = R5 |  |  |  |  | bandNR = n260 ca-BandwidthClassDL = R4 |  |  |  | bandNR = n260 ca-BandwidthClassDL = R3 |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | CC1 | CC2 | CC3 | CC4 | CC5 | CC1 | CC2 | CC3 | CC4 | CC1 | CC2 | CC3 |
| 100 | 100 | 100 | 100 | 100 | 100 | 200 | 100 | 100 | 100 | 100 | 200 | 200 | 100 | 100 | 200 | 200 | 200 |
| 100 | 100 | 100 | 100 | 100 |  | 200 | 100 | 100 | 100 | 100 | 200 | 200 | 100 | 100 | 200 | 200 |  |
| 100 | 100 | 100 | 100 |  |  | 200 | 100 | 100 | 100 |  | 200 | 200 | 100 | 100 | 200 |  |  |
| 100 | 100 | 100 |  |  |  | 200 | 100 | 100 | 100 |  | 200 | 200 | 100 |  |  |  |  |
| 100 | 100 |  |  |  |  |  | 100 | 100 | 100 | 100 |  | 200 | 100 | 100 |  |  |  |
| 100 |  |  |  |  |  | 200 | 100 | 100 |  |  | 200 | 200 |  |  |  |  |  |
|  |  |  |  |  |  |  | 100 | 100 | 100 |  |  | 200 | 100 |  |  |  |  |
|  |  |  |  |  |  | 200 | 100 |  |  |  |  |  | 100 | 100 |  |  |  |
|  |  |  |  |  |  |  | 100 | 100 |  |  | 200 |  |  |  |  |  |  |
|  |  |  |  |  |  | 200 |  |  |  |  |  |  | 100 |  |  |  |  |
|  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |  |  |

NOTE: The UE takes Feature Set way to advertise fallback in this example.

- For intra-band contiguous CA, the size of capability signaling mainly comes from the greater diversity in the UE capabilities because most fallbacks are not skipped.

  - New signaling proposal from RAN4

The characteristics of the new IE are quoted in the Table 9. The main idea is to use the maximum aggregated bandwidth as a boundary condition for determining how many intra-band contiguous CA band combinations are supported across different bandwidth classes. Considering there are only two CC bandwidth candidates (100MHz and 200MHz), the UE could obtain a determinate result if all boundary conditions are clear.

Table 9: New signaling proposal from RAN4 LS [1]

- The new IE is optional for a UE to signal. When the IE is not signalled, legacy operation is assumed: - the UE can still communicate to the network the maximum aggregated BW limitation using the existing framework. - The network understands that the UE supports the legacy fallback BW classes. - The new IE applies to intra-band carrier contiguous aggregation as well as an intra-band contiguous carrier aggregation component within an inter-band carrier aggregation. The new IE is separately applicable to each, UL, and DL. - When signalled for an explicitly supported BW class in FBG5: - It is in addition to the existing signaling for that BW class. - The network understands that the UE has independent maximum limits on number of CCs and max. aggregated bandwidth for that band. For example, when the UE indicates explicit support for R12 and a max. aggregated bandwidth of 1600Mhz using the new IE, it not only means the max. aggregated bandwidth 1600MHz applies to 12 CCs, but also applies to lower order classes, i.e., 11CCs, 10CCs, and so on. - The IE conveys the max. aggregated bandwidth value for each FeatureSetListPerUplink(Downlink)CC. for example, in each FeatureSetUplink(Downlink). - A band may have multiple values of max. aggregated bandwidth associated with different FeatureSetListPerUplink(Downlink)CC.

For the case other UE capabilities except for CC bandwidth within the feature set are the same for all CCs, it is proposed to allow the UE to advertise only 1 CA band combination with a new IE “MaximumAggregatedBW” (i.e., the FBG5-CA9 below) in the Table 10 to represent the support of 4 supported CA band combinations in the Table 6.

FBG5-CA9: n260R6 {MaximumAggregatedBW = 600MHz, [200MHz][200MHz][200MHz][200MHz][200MHz][200MHz]}

Table 10: Signaling example for CA n260R6 with the new IE

| FBG5-CA9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandNR = n260 ca-BandwidthClassDL = R6 |  |  |  |  |  |  |
| MaximumAggregatedBW = 600MHz |  |  |  |  |  |  |
|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 |
|  | Max. BW: 200MHz | Max. BW: 200MHz | Max. BW: 200MHz | Max. BW: 200MHz | Max. BW: 200MHz | Max. BW: 200MHz |

By the CA capability advertisement in the Table 10, the network understands that the UE supports at most 6 aggregated CCs and the maximum allowed CC bandwidth is 200MHz for each, but it cannot support 6 simultaneous CCs aggregation with maximum bandwidth for each because the total bandwidth 1200MHz is greater than the reported maximum aggregated bandwidth 600MHz of these 6 CCs (of the same FeatureSetDownlink). For this 6 CCs case, there is roughly 75% signaling reduction (from 4 to 1) in the BC level of capability reporting. The reduction gain is expected to be more significant as the capability signaling used for higher bandwidth classes (more CCs) in FBG5.

However, for the case other UE capabilities within the feature set are not the same for all CCs, individual CC is with different capabilities from each other and shall be explicitly advertised so that CA band combinations at the same aggregated bandwidth level are no longer able to be represented by mere one CA band combination along with the new IE.

- The signaling reduction advantage of the new IE proposal is unneglectable ONLY for the case other capabilities except for CC bandwidth within the feature set are the same for all CCs.

We also found that following CA band combinations reporting would mean same thing as FBG5-CA9 does to the network (i.e., the network determines the same resulting CA band combinations set {FBG5-CA1, FBG5-CA2, FBG5-CA3, FBG5-CA4} from them as from the FBG5-CA9.):

- FBG5-CA10: n260R6 {MaximumAggregatedBW = 600MHz, [200MHz][200MHz][200MHz][100MHz][100MHz][100MHz]}

- FBG5-CA11: n260R6 {MaximumAggregatedBW = 600MHz, [200MHz][200MHz][200MHz][200MHz][100MHz][100MHz]}

- FBG5-CA12: n260R6 {MaximumAggregatedBW = 600MHz, [200MHz][200MHz][200MHz][200MHz][200MHz][100MHz]}

From viewpoint of RF design, the UE claims to support 200MHz channel bandwidth for a CC then it shall be able to support 100MHz as well. So, it is natural to distinguish them from each other by taking the quantities of 200MHz CC as the indication for maximum allowed number of 200MHz CC that UE supports. And such an interpretation is also well aligned with current usage of the legacy capability parameter supportedBandwidthDL/UL.

However, we do not know how to treat these CA band combinations (FBG5-CA10, FBG5-CA11, and FBG5-CA12) in RRM (38.133) and conformance test (38.521/38.523) areas. Do they share the same RRM and conformance testing requirements?

- In certain conditions, the new IE proposal introduces additional CA band combination equivalences. RAN2 could have identification and explanation in signaling level but do not know the justification for RAN1, RAN4, and RAN5.

- RAN2 to discuss whether and how to avoid additional CA band combination equivalence introduced by the new IE proposal.

Furthermore, if the UE has additional capability constraint: at most two 200MHz CCs are supported, the new signaling would need to let the network know FBG5-CA4 is no longer supported. The capability is supposed to be like the content in the Table 11.

Table 11: Signaling example for CA n260R6 with aggregated BW and CC number boundary

| FBG5-CA13 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandNR = n260 ca-BandwidthClassDL = R6 |  |  |  |  |  |  |
| MaximumAggregatedBW = 600MHz |  |  |  |  |  |  |
|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 |
|  | Max. BW: 200MHz | Max. BW: 200MHz | Max. BW: 100MHz | Max. BW: 100MHz | Max. BW: 100MHz | Max. BW: 100MHz |

- The resulting set of supported band combinations is determined not only by the maximum aggregated bandwidth, but also by the quantity of CC with dominant bandwidth (i.e., 200MHz), within the same feature set. The CA band combinations in the resulting set are the parent band combinations in the same aggregated bandwidth level, then legacy fallback rule can be applied as usual.

According to the understanding abovementioned, we can simply formulate the procedure to produce the reported CA band combination as the pseudo code in the Table 12: (The reported capability parameters are marked in blue)

Table 12: Formulation of new IE proposal for FBG5 (UE side)

if (capabilities except for CCBW within the FS are the same for all CCs) {

reportedMaxAggreBW = UEMaxAggreBWForFBG5CA;					//600MHz

reportedCA-BWClassDL = NUM2BWCLASS(reportedMaxAggreBW / 100);	//R6

if (UE has quantity constraint in CC with dominant bandwidth) {

NrofCCforMaxBW = UEConstraintFor200MHzCC;					//2

} else {

//Because 200MHz BW contains 100MHz

NrofCCforMaxBW = reportedMaxAggreBW / 100;					//[6]

}

NrofCCforMinBW = (reportedMaxAggreBW / 100) – NrofCCforMaxBW;

reportedFeatureSet = {[200MHz]^NrofCCforMaxBW, [100MHz]^NrofCCforMinBW};

}

- RAN2 to discuss and agree that new IE proposal has no co-existence issue with legacy fallback rule ONLY in the case other capabilities except for CC bandwidth within the feature set are the same for all CCs of FBG5 CA band combination.

- Summary

In previous sections we made the following observations and proposals:

- The principles how a fallback is determined and configured from a parent CA are already clarified in the RAN2 fallback definitions. Within an FBG, RAN4 fallback requirement works no confliction with RAN2 fallback definitions in practical deployment today.

- Although the BCS of a derived fallback could be undefined in the RAN4 CA configurations table but it is still technically accommodated for the bandwidth class where it belongs.

- For cross-WG alignment of fallback rules, RAN2 to consider sending LS to RAN4 for the observation: UE would need to support an undefined BCS of a certain CA if it is a fallback derived from a parent CA by following RAN4 fallback requirement within an FBG.

- For intra-band contiguous CA, the size of capability signaling mainly comes from the greater diversity in the UE capabilities because most fallbacks are not skipped.

- The signaling reduction advantage of the new IE proposal is unneglectable ONLY for the case other capabilities except for CC bandwidth within the feature set are the same for all CCs.

- In certain conditions, the new IE proposal introduces additional CA band combination equivalences. RAN2 could have identification and explanation in signaling level but do not know the justification for RAN1, RAN4, and RAN5.

- RAN2 to discuss whether and how to avoid additional CA band combination equivalence introduced by the new IE proposal.

- The resulting set of supported band combinations is determined not only by the maximum aggregated bandwidth, but also by the quantity of CC with dominant bandwidth (i.e., 200MHz), within the same feature set. The CA band combinations in the resulting set are the parent band combinations in the same aggregated bandwidth level, then legacy fallback rule can be applied as usual.

- RAN2 to discuss and agree that new IE proposal has no co-existence issue with legacy fallback rule ONLY in the case other capabilities except for CC bandwidth within the feature set are the same for all CCs of FBG5 CA band combination.

- References

- R2-2209347	LS on new contiguous BW classes for legacy networks	RAN4 (To: RAN2)

- R2-2211009	Summary of email discussion [AT119bis-e][010][NR17] FBG5 BW Classes (Qualcomm)

- TS 38.101-1 V17.7.0 (2022-09)

- TS 38.101-2 V17.7.0 (2022-09)

- TS 38.306 V17.2.0 (2022-09)