3GPP TSG-RAN WG2 Meeting #120		R2-2212922

Toulouse, France, 14th – 18th November, 2022

Title:	Draft Reply LS on provision of GNSS Positioning integrity requirement

Response to:	R2-2211137 (S2-2209966) “LS on GNSS integrity requirement provisioning”

Release:	Release 17

Work Item:	NR_pos_enh-Core

Source:	RAN2

To:	SA2

Cc:	SA1

Contact Person:

Name:	Xiang Pan

E-mail Address:	panxiang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 would like to thank SA2 for their LS on GNSS integrity requirement provisioning. Regarding what parameters can be provided by LCS client/AF, or UE, it is decided and specified in Rel-17 that:

- Only UE-based GNSS positioning integrity was supported and the LMF shall send the Target Integrity Risk to UE to derive the Protection Level.

- The parameter IRallocation should be freely chosen by the LCS client and provided to LMF. Combing the knowledge of Residual Risk, LMF further generates a reasonable range of Integrity Risk (IR) and transmits to UE in the form of assistance data.

- Time-To-Alert is needed for LMF as the maximum allowable elapsed time from when the Error exceeds the Bound until a DNU flag must be issued.

- From RAN2’s perspective, it is LCS client/AF or UE, who request GNSS positioning integrity requirements, that should be obliged to evaluation the system. That is, the operation to compare PL and AL, including to refer to the optional achievable target integrity risk, can be done by the LCS client/AF or UE. Therefore, there is no need to provide GMLC or AMF (further forwarding to LMF) with AL.

In conclusion, RAN2 believes that, at least for R17 GNSS positioning integrity, the necessary parameters that need to be provided by the LCS client/AF or UE to the GMLC or AMF (further forwarding to LMF) are Target Integrity Risk, Time To Alert and IRallocation. RAN2 also thinks it is up to SA2’s decision whether LCS client/AF, or UE needs to expose Alert Limit to the GMLC or AMF.

2. Actions:

To SA WG2

ACTION: 	RAN2 kindly asks SA2 to take above clarifications into account and proceed with the design work accordingly.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #121	27 Feb – 3 March 2023			Athens

TSG RAN WG2 Meeting #122	22 May – 26 May 2023			Korea