3GPP TSG-RAN WG2 Meeting #120	R2-2213182

# Toulouse, France, 14-18 November 2022	was R3-226795

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0358 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction between CPC Cancel and SN Release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Ericsson, Lenovo, ZTE, Google Inc., Intel Corporation) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-17, it is not clear how the CPA or CPC is cancelled when the SN is released by MN, i.e., MR-DC is released. No matter it is the MN or SN who triggers the release of MR-DC, the configured CPAC anyway needs to be cancelled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes to clarify that CPA/CPC with target SN(s) should be cancelled when the MN receives SN Release Request Ack message or SN Release Required message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact under funtional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issue may happen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.1, 10.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-226795. |  |  |  |  |  |  |  |  |

### 10.4.1	EN-DC

The Secondary Node Release procedure may be initiated either by the MN or by the SN and is used to initiate the release of the UE context at the SN. The recipient node of this request can reject it, e.g., if a SN change procedure is triggered by the SN.

In case of CPA or CPC, this procedure may be initiated either by the MN or the target SN, and it is used to cancel all the prepared PSCells at the target SN and initiate the release of related UE context at the target SN.

It does not necessarily need to involve signalling towards the UE, e.g., in case of the RRC connection re-establishment due to Radio Link Failure in MN.

MN initiated SN Release

Figure 10.4.1-1: SN Release procedure – MN initiated

Figure 10.4.1-1 shows an example signalling flow for the MN initiated Secondary Node Release procedure when SN Release is confirmed by SN.

1.	The MN initiates the procedure by sending the SgNB Release Request message. If applicable, the MN provides data forwarding addresses to the SN.

2.	The SN confirms SN Release by sending the SgNB Release Request Acknowledge message. If appropriate, the SN may reject SN Release, e.g. if the SN change procedure is triggered by the SN.

NOTE 1a:	If CPA or CPC is configured, upon reception of the SgNB Release Request Acknowledge message the MN cancels all CPAC with the target candidate SN(s).

3/4.	If required, the MN indicates in the RRCConnectionReconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

NOTE 1:	If data forwarding is applied, timely coordination between steps 1 and 2 may minimize gaps in service provision, this is however regarded to be an implementation matter.

5.	For bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 2:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated SN Release

Figure 10.4.1-2: SN Release procedure – SN initiated

Figure 10.4.1-2 shows an example signalling flow for the SN initiated Secondary Node Release procedure.

1.	The SN initiates the procedure by sending the SgNB Release Required message which may contain inter-node message to support delta configuration.

2.	If applicable, the MN provides data forwarding addresses to the SN in the SgNB Release Confirm message. The SN may start data forwarding and stop providing user data to the UE as early as it receives the SgNB Release Confirm message.

NOTE 3a:	If CPA or CPC is configured, upon reception of the SgNB Release Required message the MN cancels all CPAC with the target candidate SN(s).

3/4.	If required, the MN indicates in the RRCConnectionReconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

NOTE 3:	If data forwarding is applied, timely coordination between steps 2 and 3 may minimize gaps in service provision. This is however regarded to be an implementation matter.

5.	For bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 4:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### 10.4.2	MR-DC with 5GC

The SN Release procedure may be initiated either by the MN or by the SN and is used to initiate the release of the UE context and relevant resources at the SN. The recipient node of this request can reject it, e.g., if an SN change procedure is triggered by the SN.

In case of CPA or CPC, this procedure may be initiated either by the MN or the target SN, and it is used to cancel all the prepared PSCells at the target SN and initiate the release of related UE context at the target SN.

MN initiated SN Release

Figure 10.4.2-1: SN release procedure - MN initiated

Figure 10.4.2-1 shows an example signalling flow for the MN initiated SN Release procedure.

1.	The MN initiates the procedure by sending the SN Release Request message.

2.	The SN confirms SN Release by sending the SN Release Request Acknowledge message. If appropriate, the SN may reject SN Release, e.g., if the SN change procedure is triggered by the SN.

NOTE 0a:	If CPA or CPC is configured, upon reception of the SN Release Request Acknowledge message the MN cancels all CPAC with the target candidate SN(s).

2a.	When applicable, the MN provides forwarding address information to the SN.

NOTE 0:	The MN may send the Xn-U Address Indication message to provide forwarding address information before step 2.

3/4.	If required, the MN indicates in the MN RRC reconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

NOTE 1:	If data forwarding is applied, timely coordination between steps 1 and 2 may minimize gaps in service provision, this is however regarded to be an implementation matter.

5.	If PDCP termination point is changed to the MN for bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 1a:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the PDU Session path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated SN Release

Figure 10.4.2-2: SN release procedure - SN initiated

Figure 10.4.2-2 shows an example signalling flow for the SN initiated SN Release procedure.

1.	The SN initiates the procedure by sending the SN Release Required message which may contain inter-node message to support delta configuration.

2.	If data forwarding is requested, the MN provides data forwarding addresses to the SN in the SN Release Confirm message. The SN may start data forwarding and stop providing user data to the UE as early as it receives the SN Release Confirm message.

NOTE 2a:	If CPA or CPC is configured, upon reception of the SN Release Required message the MN cancels all CPAC with the target candidate SN(s).

3/4.	If required, the MN indicates in the MN RRC reconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

NOTE 2:	If data forwarding is applied, timely coordination between steps 2 and 3 may minimize gaps in service provision. This is however regarded to be an implementation matter.

5.	If PDCP termination point is changed to the MN for bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 3:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the PDU Session path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.