3GPP TSG RAN WG2 Meeting #121          	      	                	                R2-2300440

Athens, GR, 27th Feb – 3rd Mar, 2023

Agenda Item:	8.1.4

Source:	Intel Corporation

Title:	Initial access signalling for NCR-MT

Document for:	Discussion and Decision

# Introduction

It was agreed in RAN3 #118 meeting that solution 3 will be used by NCR for authorization.

Take Solution 3 as the basis for NCR management. FFS on any additional aspects.

In this contribution, we first discuss the RAN2 impact of solution 3, e.g. signaling impact in air interface.

# Initial Access

Example of Solution 3 is captured in TR38.867 [1], copied as below:

Furthermore, RAN2 further agreed an NCR-support indication will be introduced in SIB1:

- Introduce an NCR-support indication in SIB1 per PLMN; whether it is also per NPN is FFS

Based on this indication, one NCR-MT is able to join a gNB that supports NCR via initial access procedure. To support gNB to select an AMF that supports NCR, an early identification is needed over RRCSetupComplete message, indicating the joined device is an NCR-MT.

Proposal 1: Introduce an early identification “NCR-NodeIndication” in RRCSetupComplete message.

# Conclusion

In this contribution, we discussed potential RAN2 impact during NCR-MT initial accecss.

We have following observations and proposals:

Proposal 1: Introduce an early identification “NCR-NodeIndication” in RRCSetupComplete message.

# References

- TR38.867, Study on NR network-controlled repeaters