3GPP TSG-RAN WG2 Meeting #121	R2-2300959

Athens, Greece, 27 February-3 March, 2023

Agenda item:	 8.13.8

Source:		Lenovo

Title:		SON enhancements for CPAC

Document for:		Discussion and Decision

1	Introduction

In RAN2#119-e meeting [1], agreements for MRO for CPAC were achieved:

Agreement:

For CPAC failure relevant MRO, RAN2 prioritize the discussion on NR-DC, while other scenarios can be further discussed if time permits.

In RAN2#120 meeting [2], further agreements were achieved:

Agreement:

1	RAN2 confirms the CPA/CPC scenarios agreed by RAN3 and discuss corresponding UE impacts.

2	SCGFailureInformation is enhanced to support CPAC MRO (i.e, no need to introduce new reports/message).

In this paper, we would further discuss the details of SON enhancements for CPAC in NR-DC.

2	Discussion

In R16, intra-SN CPAC without MN involvement is introduced, to further enhance mobility robustness, MN or SN initiated inter-SN CPAC and intra-SN CPAC with MN involvement are specified in R17. In general, a CPAC procedure means a CPA or CPC procedure, the legacy SN addition and SN change procedure can be taken as a baseline to support CPAC.

Compared with legacy PSCell change/addition, the most difference in a CPAC procedure is that it is the UE to decide whether/when to perform CPAC execution e.g. perform PSCell change/addition towards a target PSCell in which the corresponding CPAC execution condition is fulfilled, after receiving CPAC configuration. In a CPAC procedure, the following phases are included:

- CPAC configuration: The network provides the CPAC configuration to the UE. The CPAC configuration includes the configuration of CPAC candidate PSCell(s) generated by the candidate SN(s), CPA execution condition(s) generated by MN, or CPC execution condition(s) generated by MN in case of MN initiated CPC or by SN in case of SN initiated CPC. The network may update CPAC configuration before CPAC execution.

- CPAC Evaluation: After receiving the latest CPAC configuration, the UE shall perform conditional configuration evaluation. If the execution condition of a CPAC candidate PSCell is fulfilled, the UE shall consider the candidate PSCell as a target PSCell and initiate CPAC execution.

- CPAC execution: The UE shall apply the configuration generated by the target PSCell, and perform PSCell change/addition towards the target PSCell.

In a CPA procedure, the following failure case i/ii may happen:

- Case i: an SCG failure occurs during CPA execution phase, i.e. the UE fails to access the target PSCell in which the CPA execution condition is fulfilled;

- Case ii: an SCG failure occurs shortly after successful CPA execution, i.e. an SCG failure occurs shortly in the target PSCell after a successful PSCell addition to the target PSCell.

Similarily, the following failure case 1-3 may happen in a CPC procedure:

- Case 1: an SCG failure occurs before CPC is executed, i.e. source SCG failure occurs after being configured with CPC configuration and before CPC is executed;

- Case 2: an SCG failure occurs during CPC execution phase, i.e. the UE fails to access the target PSCell in which the CPC execution condition is fulfilled;

- Case 3: an SCG failure occurs shortly after successful CPC execution, i.e. an SCG failure occurs shortly in the target PSCell after a successful PSCell change from a source PSCell to the target PSCell.

Failure in a CPAC procedure would impact system performance, the above failure cases should be considered for MRO for CPAC in NR-DC.

Observation 1: The following failure cases should be considered for MRO for CPAC:

- an SCG failure occurs before CPC is executed;

- an SCG failure occurs during CPAC execution phase;

- an SCG failure occurs shortly after successful CPAC execution.

In R17, to support MRO for SCG failure, besides failure type and measurement results in MCG and SCG, the SCG Failure Information message is enhanced to include other SCG failure related information i.e. previousPSCellId, failedPSCellId, timeSCGFailure and RA information. In RAN2#120 meeting, it has agreed that SCGFailureInformation is enhanced to support MRO for CPAC (i.e, no need to introduce new reports/message).

Observation 2: SCG Failure Information message can be enhanced to support MRO for CPAC.

In case of an SCG failure occurs before CPC execution, to enable the network decide whether the CPC execution condition or candidate PSCell(s) are configured properly or not, it is necessary for the UE to report the time elapsed between the SCG failure in source SCG and the latest CPC configuration is received in the SCG Failure Information message. For example, if this reported time duration is long, network may understand that SCG failure may be caused due to strict CPC execution condition, network may reduce the RSRP/RSRQ/SINR trigger threshold for CPC or TTT value for CPC evaluation.

Proposal 1: The UE reports the time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

In case of an SCG failure occurs during CPAC execution phase or an SCG failure occurs shortly after successful CPAC execution, since it is the UE decides when CPAC execution is triggered, it is necessary for the UE to report the time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell in the SCG Failure Information message.

Proposal 2: The UE reports the time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

Additionally, in R17, timeSCGFailure IE is introduced in the SCG Failure Information message to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. For a CPAC procedure, this IE can be reused to indicate the time elapsed since the CPAC execution towards the target PSCell until the SCG failure, i.e. the time elapsed since executing the last PSCell addition/change towards the target PSCell until the PSCell addition/change failure or a RLF occurring shortly after the successful PSCell addition/change.

Proposal 3: The UE reports the time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

Considering legacy PSCell addition/change procedure and CPA/CPC procedure have different configurations, it is beneficial for the network to know the type of PSCell addition/change when performing MRO. After discussion in RAN2#120 meeting, it is FFS that information to differentiate CAPC from conventional SCG failure is needed for CPAC MRO (e.g. by implicit or explicit indication). The explicit sloution is better, i.e. the UE reports the type of PSCell addition/change (e.g. CPA/CPC or legacy PSCell addition/change), especially for the case that no CPAC specific informatin is reported in the SCG Failure Information message.

Proposal 4: The UE reports the type of PSCell addition/change (e.g. CPA/CPC or legacy PSCell addition/change) to differentiate CAPC from conventional SCG failure.

To analyze whether CPAC configuration is set properly, e.g. whether CPAC execution condition(s) are configured properly, or whether configured CPAC candidate PSCell(s) are suitable, it is beneficial for the network to know the configured CPAC configuration(s) for MRO analysis, especially when the network does not have CPAC related parameters e.g. when UE context is released. It is beneficial of the UE to report the latest configured CPAC configuration including the latest CPAC execution condition(s) and the list of latest CPAC candidate PSCells in the SCG Failure Information message.

Proposal 5: The UE reports the latest configured CPAC configuration including the CPAC execution condition(s) and the list of CPAC candidate PSCells.

Besides latest radio measurement results, an indication about whether a measured neighbour cell included in the measurement results was configured as a CPAC candidate PSCell or not can be reported in the SCG Failure Information message, to assist the network to analyse whether the configured CPAC candidate PSCell(s) are suitable or how to modify the CPAC candidate PSCell(s) for optimisation.

Proposal 6: The UE reports an indication about whether a measured neighbour cell was configured as a CPAC candidate PSCell or not.

Furthermore, since at most two triggering event can be configured for one CPAC candidate PSCell, if two triggering events are configured for one CPAC candidate PSCell, CPAC execution is performed only when both two triggering events are fulfilled. When failure (e.g. CPAC execution failure or radio link failure) occurs, it may mean at least one trigger event is not suitable, to let network know more information for modifying CPAC execution condition(s), the UE can report the first fulfilled CPAC triggering event e.g., A3 or A5, and the time elapsed between two CPAC triggering events if both the two CPAC triggering events are fulfilled.

Proposal 7: The UE reports the first fulfilled CPAC triggering event and the time elapsed between two CPAC triggering events if both the two CPAC triggering events are fulfilled.

3	Conclusion

In this contribution, the details of SON enhancements for CPAC in NR-DC are discussed. We have the following observations and proposals:

Observation 1: The following failure cases should be considered for MRO for CPAC:

- an SCG failure occurs before CPC is executed;

- an SCG failure occurs during CPAC execution phase;

- an SCG failure occurs shortly after successful CPAC execution.

Observation 2: SCG Failure Information message can be enhanced to support MRO for CPAC.

Proposal 1: The UE reports the time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

Proposal 2: The UE reports the time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

Proposal 3: The UE reports the time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

Proposal 4: The UE reports the type of PSCell addition/change (e.g. CPA/CPC or legacy PSCell addition/change) to differentiate CAPC from conventional SCG failure.

Proposal 5: The UE reports the latest configured CPAC configuration including the CPAC execution condition(s) and the list of CPAC candidate PSCells.

Proposal 6: The UE reports an indication about whether a measured neighbour cell was configured as a CPAC candidate PSCell or not.

Proposal 7: The UE reports the first fulfilled CPAC triggering event and the time elapsed between two CPAC triggering events if both the two CPAC triggering events are fulfilled.

Reference

[1] Chairman notes for RAN2#119-e meeting

[2] Chairman notes for RAN2#120 meeting