3GPP TSG-RAN WG2 #121 	R2-2300986

Athens Greece, 27th Feb.–3rd March 2023

Agenda Item:	8.1.3

Source:	NEC, Rakuten Mobile Inc.

Title:	Discussion on remaining RAN2 aspects for NCR supporting

Document for:	Discussion, Decision

Introduction

The Rel-18 work item on NR network-controlled repeaters (NCR) was discussed in RAN2#120 meeting. Following agreements are achieved [1]:

Agreement: - Introduce an NCR-support indication in SIB1 per PLMN; whether it is also per NPN is FFS - WA: RRC_INACTIVE is optionally supported without any specific enhancements. - gNB cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation - NCR-MT indicates the maximum number of supported DRB in UE capability, values {1, 16}. If absent, the NCR-MT does not support DRB. - SRB2 is mandatory feature for NCR-MT. - On NCR-Fwd ON/OFF: - When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. - After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB. - Release to RRC_IDLE is FFS. - On NCR-MT RLF: - After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment; - If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF; - During RRC re-establishment procedure, NCR-Fwd is OFF. - NCR-MT mandatorily support cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE. - In Rel-18, NCR-MT does not support handover and RRM measurements in RRC_CONNECTED. - For reporting the capabilities of NCR-MT, the existing UECapabilityEnquiry and UECapabilityInformation messages are reused. - In NCR-MT capability discussion, to focus on mandatory features that are required for NCR-MT. - All existing optional features are considered as applicable to NCR-MT unless explicitly excluded (Same as IAB-MT). FFS on taking IAB specified features as a baseline for future discussion. - NPN capable NCR-MT should consider cellReservedForOtherUse for determination of an NPN-only cell.

This contribution provides our views on some remaining RAN2 aspects for NCR supporting.

Discussion

NCR identification

One remaining issue is NCR identification. There are two alternatives captured in TR38.867 [2]:

- Alternative 1: report an NCR indicator in Msg5 (in addition to sending any NCR-related radio capability)

- Alternative 2: report an NCR indicator (implicitly or explicitly) in UE's radio capability signaling

RAN3#118 meeting achieved the agreement to take Solution 3 (Clause 8.1.3 in [2]) as the basis for NCR management. That means the gNB shall select an AMF which supports NCR function and forward the NCR indicator to the AMF. Therefore, an explicit NCR identification reported in Msg5 is necessary.

RAN2 agree to report an explicit NCR indicator in Msg5 (in addition to sending any NCR-related radio capability).

Release to RRC_IDLE is FFS

During last meeting, some companies doubted whether there is a scenario to release NCR-MT from RRC_CONNECTED to RRC_IDLE. Therefore we think two aspects need to be discussed here, one is the potential scenario to release a NCR-MT from RRC_CONNECTED mode to RRC_IDLE mode, the other is the NCR-Fwd ON/OFF status when NCR-MT is released to RRC_IDLE.

As RAN2#120 has agreed that gNB cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to, then the radio condition (e.g., RSPR) of control link and backhaul link are highly relevant, i.e, the control link and the backhaul link can share the same large-scale properties of the channel.

- NCR is a kind of network node deployed by operators, sometimes it would be difficult to do a perfect cell planning for NCR deployments. Then, one possible scenario is that an NCR is deployed at a poor radio condition where both control and backhaul link radio condition are not good enough to achieve a satisfied access link performance. Additionally, to provide a satisfied performance of the access link, different operators may have individual requirements on the radio condition of control link and backhaul link (for example, an NCR-specific minimum required RX level in the cell, details are shown in "NCR-specific minimum required RX level" section), which is different from UE points of view.

- In our view, one possible scenario to release an NCR-MT from RRC_CONNECTED mode to RRC_IDLE mode is that the NCR-specific requirement of the minimum required RX level in the cell is not met by an NCR-MT (i.e., the backhaul link performance does not fulfil the requirements).

One possible scenario to release an NCR-MT from RRC_CONNECTED mode to RRC_IDLE mode is that the NCR-specific requirement of the minimum required RX level in the cell is not met by an NCR-MT (i.e., the backhaul link performance does not fulfil the requirements).

- In such a case, NCR-MT in RRC_IDLE mode means that the NCR is out of service, i.e., the NCR-Fwd should be OFF.

When NCR-MT is released from RRC_CONNECTED mode to RRC_IDLE mode due to the NCR-specific requirement of the minimum required RX level in the cell is not met (i.e., the backhaul link performance does not fulfil the requirements), the NCR is out of service, i.e., the NCR-Fwd should be OFF.

Another possible scenario is the case when there is no UE connecting to the gNB cell via the NCR. When there is no UE connecting to the gNB cell via the NCR, our understanding is that the NCR-Fwd would be at least asked to amplify and forward cell-specific signals and channels to keep the NCR is in-service. In such a case, it is technically possible to release the NCR-MT to RRC_IDLE, i.e, neither exchange UE-specific side control information nor manage both control and backhaul links. However, it will result in a quiet long delay when the NCR re-starts its amplifying and forwarding behaviors for newcomers who want to connect to the gNB cell via the NCR. For example, when it is detected that a new UE is initiating RRC connection to the gNB cell via the NCR, the delay of sending the required side control information would be quite long due to the NCR-MT initial access delay and the NCR-MT context setup delay. Considering RAN2#120 has agreed to optionally support RRC_INACTIVE without any specific enhancements (as the WA), we think suspending NCR-MT to RRC_INACTIVE is better than releasing NCR-MT to RRC_IDLE. Therefore we see there is no benefits to release NCR-MT to RRC_IDLE mode while keeping the NCR in-service.

Since RRC_INACTIVE mode is optionally supported by NCR-MT, there is no benefits to release NCR-MT to RRC_IDLE mode while keeping the NCR in-service.

NCR-Fwd should be OFF when NCR-MT is in RRC_IDLE mode.

- NCR-specific minimum required RX level

As we analyzed above, the control link and the backhaul link can share the same large-scale properties of the channel since gNB cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to. Then to provide a satisfied performance of the backhaul and access links, different operators may have individual requirements on the NCR-specific minimum required RX level in the cell applying for NCR-MT (i.e., the minimum requirements on backhaul link performance), which is different from UE’s minimum required RX level in the cell, as simplify depicted in Fig.1.

Fig.1 NCR-specific minimum required RX level

Such an NCR-specific minimum required RX level is quite beneficial for easily deploying NCRs while obtaining a guaranteed performance of the NCR backhaul and access links. Therefore, our proposal is

RAN2 agree to support NCR-specific minimum required RX level in Rel-18.

One potential straight-forward approach is to define an NCR-specific q-RxLevMinNCR and specify that Qrxlevmin applying in NCR-MT cell selection criterion S is obtained from q-RxLevMinNCR instead of q-RxLevMin, like supplementary uplink case (i.e., obtained from q-RxLevMinSUL).

A straight-forward approach for supporting NCR-specific minimum required RX level is to define an NCR-specific q-RxLevMinNCR and specify that Qrxlevmin applying in NCR-MT cell selection criterion S is obtained from q-RxLevMinNCR.

With the above approach, the NCR supporting cell needs to broadcast the NCR-specific minimum required RX level in the cell, q-RxLevMinNCR. Considering the number of NCR under a gNB cell is limited, there might be a concern on the signaling overhead. Then another potential approach is to apply the NCR-specific minimum required RX level for NCR-MT in RRC_CONNECTED mode by releasing the NCR-MT to RRC_IDLE if the network confirmed that the NCR-specific minimum required RX level is not fulfilled, as we explained in the last section. With this approach, an explicit indication and/or other necessary information for NCR-MT accessing (e.g., the requirement on NCR-specific minimum required RX level, timer for forbidding NCR-MT access, etc.) may be needed.

Another potential approach for supporting NCR-specific minimum required RX level is to apply the NCR-specific minimum required RX level for NCR-MT in RRC_CONNECTED mode by releasing the NCR-MT to RRC_IDLE if the network confirmed that the NCR-specific minimum required RX level is not fulfilled.

In our view, both approaches (shown in observation 4 and observation 5) are feasible. Considering it is the last meeting for NCR-MT WI, we slightly prefer to go forward with the approach shown in observation 4.

RAN2 agree to define an NCR-specific q-RxLevMinNCR and specify that Qrxlevmin applying in NCR-MT cell selection criterion S is obtained from q-RxLevMinNCR.

Summary

This contribution provides our views on remaining RAN2 aspects for NCR supporting.

Observations

Observation 1 One possible scenario to release an NCR-MT from RRC_CONNECTED mode to RRC_IDLE mode is that the NCR-specific requirement of the minimum required RX level in the cell is not met by an NCR-MT (i.e., the backhaul link performance does not fulfil the requirements).

Observation 2 When NCR-MT is released from RRC_CONNECTED mode to RRC_IDLE mode due to the NCR-specific requirement of the minimum required RX level in the cell is not met (i.e., the backhaul link performance does not fulfil the requirements), the NCR is out of service, i.e., the NCR-Fwd should be OFF.

Observation 3 Since RRC_INACTIVE mode is optionally supported by NCR-MT, there is no benefits to release NCR-MT to RRC_IDLE mode while keeping the NCR in-service.

Observation 4 A straight-forward approach for supporting NCR-specific minimum required RX level is to define an NCR-specific q-RxLevMinNCR and specify that Qrxlevmin applying in NCR-MT cell selection criterion S is obtained from q-RxLevMinNCR.

Observation 5 Another potential approach for supporting NCR-specific minimum required RX level is to apply the NCR-specific minimum required RX level for NCR-MT in RRC_CONNECTED mode by releasing the NCR-MT to RRC_IDLE if the network confirmed that the NCR-specific minimum required RX level is not fulfilled.

Proposals

Proposal 1 RAN2 agree to report an explicit NCR indicator in Msg5 (in addition to sending any NCR-related radio capability).

Proposal 2 NCR-Fwd should be OFF when NCR-MT is in RRC_IDLE mode.

Proposal 3 RAN2 agree to support NCR-specific minimum required RX level in Rel-18.

Proposal 4 RAN2 agree to define an NCR-specific q-RxLevMinNCR and specify that Qrxlevmin applying in NCR-MT cell selection criterion S is obtained from q-RxLevMinNCR.

- References

[1] R2-2213009, Report from NC Repeater breakout session

[2] TR38.867