3GPP TSG-RAN WG2 Meeting #121	                                   R2-2301064

Athens, Greece, February 27th-March 3rd, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on cell DTX and DRX mechanism for NES

Agenda item:		8.3.2

Document for: 		Discussion and Decision

- Introduction

In RAN#98, a new WID[1] of Network energy savings for NR has been agreed. And the objective about DTX/DRX is the following:

2. Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this paper, we will focus on the cell DTX/DRX mechanism, and discuss the impact on RAN2 specification.

- Discussion

In the beginning, we discuss the gNB behavior for power saving. In RAN2#119B meeting, some agreements were reached as below:

Proposal 6: RAN2 discuss expected gNB DTX / DRX behavior with details of different TX / RX types: • Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX / DRX OFF duration. • Example 2: gNB is expected to turn off its transmission / reception only for data traffic during Cell DTX / DRX OFF duration (i.e. gNB will still transmit / receive reference signals). • Example 3: gNB is expected to turn off its dynamic transmission / reception during Cell DTX / DRX OFF duration (i.e. gNB is expected to still perform periodic transmission / reception, including SPS, CG-PUSCH, SR, RACH, and SRS). • Example 4: gNB is expected to only transmit reference signals (e.g. CSI-RS for measurement). => RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX. => For the purpose of our discussion we will focus on a single UE behavior at any point in time. FFS if we allow multiple configuration of NW DRX/DTX behaviors.

From the network perspective, if gNB reduces the transmission or reception, gNB could enter into the deep sleep mode, and realize the power saving. Under the low load, Example 1 could save network power as much as possible. Example 2 could obtain the less NES gain than Example 1 due to the transmission or reception of reference signals. Example 3 could minimize the impact on UE experience, but brings with small NES gain. And Example 4 could be considered as a subset of Example 2.

Firstly, if gNB turns off the transmission or reception, the possible impacts to idle/inactive UE on the performance would be discussed. For SSB, according to WID, SSB is transmitted regularly, hence, for legacy UE in idle or inactive mode, UE behavior, such as the measurement, cell selection, wouldn’t be affected. For RO, gNB could choose to keep the reception on sparse RO and turn off the transmission, if gNB receives the preamble from, gNB could turn on the transmission for RACH timely, in this way, the impacts on RACH could be minimized.

Observation 1: If gNB could keep the reception on RO and turn off the transmission, the impacts on RACH could be minimized.

Secondly, if gNB turns off the transmission or reception, the possible impacts to connected UE on the performance would be discussed. For reference signal, the missing CSI-RS may cause the failure in radio link monitoring or beam detection for UE, and missing SRS may delay the scheduling for gNB. However, the maximum periodicity of CSI-RS and SRS could reach 320ms and 1280ms separately, the long periodicity of reference signal could decrease the probability that sleep state of gNB overlaps with CSI-RS or SRS occasion, the rare missing transmission or reception of CSI-RS or SRS wouldn’t cause the performance degradation significantly on UE and gNB. For the retransmission, it is up to gNB’s implementation to turn off transmission or reception after finishing the retransmission. For SPS and CG, the premise of stopping transmission on SPS or reception on CG for gNB is that the transmission for time-intensive traffic has been finished or UE with time-intensive traffic has been handovered to neighbor cells, thus, stopping transmission on SPS or reception on CG wouldn’t cause the interruption for time-intensive traffic. For SR or RACH, the probability of collision of SR or RACH sent by sparse UE with discontinuous reception in gNB is small. In summary, under low load, the impact of turning off transmission or reception on performance degradation is very small. In other words, if gNB believes that the large NES obtain could be exchanged by the little performance degradation, gNB could turn off its transmission or reception.

Observation 2: Under low load, if gNB could turning off its transmission including CSI-RS, SPS, and/or turning off its reception including SRS, SR, CG, it could bring with large power saving gain for gNB and cause the little performance degradation on gNB and UE.

As in WID, UE behavior could align with gNB behavior. Furtherly, UE especial DRX configuration is determined by UE traffic, and it is impracticable for gNB to reconfigure DRX configuration dynamically. Hence, cell DTX/DRX is introduced to inform UE about the gNB behavior for power saving. In RAN2#120 meeting, some agreements about DTX/DRX were reached as following:

Agreements

1 	Clarify previous agreement to: periodic cell DTX/DRX pattern is configured by UE-specific RRC.   Periodic cell DTX/DRX can be activated/deactivated by L1/L2 signalling and UE-specific RRC signaling.

2 	Capture in TR 38.864 that both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern, per the agreement in 119b-e.

3	Cell DTX and Cell DRX modes can be configured and operated separately (e.g. one RRC configuration set for DL and the other set for UL).  Cell DTX/DRX can also be configured and operated together.

4	It is up to NW whether legacy UEs can access cells with Cell DTX/DRX

5	Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA.  No additional RAN2 impacts or enhancements are foreseen.

6	Whether to support multiple Cell DTX/DRX configurations can be discussed later in the normative phase.

7	At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration.  Details related to UE behaviour can be discussed during WI phase

8	From RAN2 perspective DTX/DRX is feasible

As above, gNB behavior for NES could be summarized as no DL transmission, no UL reception or no DL transmission and UL reception. It is flexible to configure Cell DTX and Cell DRX separately to suit kinds of gNB behavior for power saving. If gNB turn off all transmission and reception, both Cell DTX and Cell DRX are informed to UE. If gNB turn off only transmission, Cell DTX configuration are informed to UE. If gNB turn off only reception, Cell DRX configuration are informed to UE.

Proposal 1: Cell DTX and Cell DRX configuration can be configured separately.

In general, gNB would adopt different Cell DTX/DRX configuration under different load. For example, under low load, gNB would turn off all transmission and reception for a relatively long time, and corresponding the Cell DTX and DRX configuration has long off duration; under light load, gNB would turn off its dynamic transmission / reception for a relatively short time, and corresponding the Cell DTX and DRX configuration has short off duration. Hence, there is a need to support multiple DTX/DRX configuration in a cell.

Proposal 2: Multiple DTX/DRX configuration should be supported in a cell.

But, for a moment, only one DTX/DRX configuration takes effect, and UE aligns with this DTX/DRX configuration. Considering the DTX/DRX configuration is per cell, a common L1 signaling is used to dynamically indicate the DTX/DRX configuration that is going to take effect in order to decrease the signaling overhead. UE should know the timing information of DTX/DRX, such as start time, the duration and period. Considering the limited bits in L1 signaling, it is impossible to carry much bits of timing information in L1 signaling. Hence, the easy way that RRC configuration + a common L1 signaling enabling could be considered. RRC broadcasts the supported multiple DTX/DRX configuration including the duration and period, and a common L1 signaling enables one of them.

In order to support enable one of DTX/DRX configurations dynamically, the semi-static configuration of the start time isn’t applicable. Hence, start time could be carried in common L1 signalling.

Proposal 3a: RRC configures the supported multiple DTX/DRX configuration including the duration and period.

Proposal 3b: A common L1 signaling enables one of DTX/DRX configurations.

Proposal 3c: A common L1 signaling carries start time of enabled DTX/DRX configurations.

As the above analysis, in order to obtain the largest NES gain, gNB could turn off its transmission during off duration of cell DTX and or reception during off duration of cell DRX. Therefore, if a DTX configuration is enabled, UE could stop DL reception during off duration. If a DRX configuration is enabled, UE could stop UL transmission during off duration.

Proposal 4a: If a DTX configuration that gNB is expected to turn off its transmission is enabled, R18 UE in connected mode could stop DL reception during off duration.

Proposal 4b: If a DRX configuration that gNB is expected to turn off its reception is enabled, R18 UE in connected mode could stop UL transmission during off duration.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: If gNB could keep the reception on RO and turn off the transmission, the impacts on RACH could be minimized.

Observation 2: Under low load, if gNB could turning off its transmission including CSI-RS, SPS, and/or turning off its reception including SRS, SR, CG, it could bring with large power saving gain for gNB and cause the little performance degradation on gNB and UE.

Proposal 1: Cell DTX and Cell DRX configuration can be configured separately.

Proposal 2: Multiple DTX/DRX configuration should be supported in a cell.

Proposal 3a: RRC configures the supported multiple DTX/DRX configuration including the duration and period.

Proposal 3b: A common L1 signaling enables one of DTX/DRX configurations.

Proposal 3c: A common L1 signaling carries start time of enabled DTX/DRX configurations.

Proposal 4a: If a DTX configuration that gNB is expected to turn off its transmission is enabled, R18 UE in connected mode could stop DL reception during off duration.

Proposal 4b: If a DRX configuration that gNB is expected to turn off its reception is enabled, R18 UE in connected mode could stop UL transmission during off duration.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR