3GPP TSG-RAN WG2 #121b-e                                          R2-2303182

online, April, 2023

Source: 			OPPO

Title: 	SON on fast MCG recovery

Agenda item:		7.13.8

Document for: 	Discussion and Decision

- Introduction

In the RAN2#120 meeting, agreements on the fast MCG recovery MRO have been made as follows:

Agreements

1	For fast MCG recovery MRO, prioritize NR-DC scenario. if time allows, study whether the same solution can be extended for others DC scenarios.

2	Consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316). The “upon fast MCG recovery case” is FFS.

3	RLF report is enhanced to support fast MCG recovery MRO.

4	Fast MCG recovery failure cause shall be included for fast MCG recovery optimization. FFS details

Subsequently, in the last RAN2#121 meeting, a FFS have been left as follows:

FFS the scenario to “SCG deactivation before fast MCG recovery (when UE detects MCG failure)”

In this paper, we would like to present our views on supporting the SON enhancements for fast MCG recovery.

- Discussion

2.1 the mechanism of fast MCG recovery

When R15 MR-DC is employed, if RLF towards the PCell occurs, even if the SCG bearer is still workable, the UE has no choice but to re-establish the RRC, i.e., perform the RACH procedure to send the RRC Re-establishment Request msg towards the network, which takes time to recovery the MCG connection. Fast MCG recovery was introduced in R16 to take advantage of SCG bearer (split SRB1 or SRB3) to transmit the UL MCGFailureInformation msg towards the network, when the MCG is failed. Such mechanism allows the UE to try its best to avoid performing RRC Re-establishment towards the MCG, if the SCG bearer quality is still good, by reconfiguring the MCG using the SCG bearer. Hopefully, the SN receiving the report of MCG link failure information forwards the information to the MN, and then the MN feedbacks the RRCReconfiguration msg including a new MCG configuration to the SN. Finally, the UE is able to restore the MR-DC state after receiving the RRCReconfiguration msg from the SN, which is faster than performing RRC Re-establishment.

Note that the UE also maintains a new timer T316 for fast MCG recovery. Specifically, T316 was triggered upon transmission of the MCGFailureInformation msg via the SCG bearer. It will be continuously running until further reception of the msg RRCRelease, RRCReconfiguration with sync for the PCell, or MobilityFromNRCommand from the SCG. If the UE does not receive any one of the above mentioned msg from the SCG, when T316 is expired finally, the UE will still need to initialize the RRC re-establishment procedure.

Observation 1: fast MCG recovery allows the UE to try its best to avoid having to perform RRC Re-establishment towards the MCG, if the SCG bearer quality is still good, by reconfiguring the MCG using the SCG.

Observation 2: a new timer T316 for fast MCG recovery was introduced. If the UE does not receive RRC msg from the SCG regarding the MCG reconfiguration/release/mobility command, when T316 is expired finally, the UE will still need to initialize the RRC re-establishment procedure.

2.2 The necessity of T316 optimization

Suppose that when the UE experiences the MCG RLF failure, the SCG link is still good, the UE determines to initialize the fast MCG recovery procedure. However, as indicated as follows, after the UE sends the MCGFailureInformation msg towards the SCG, if the SCG link becomes worse, UE cannot receive anything from the SCG, it can do nothing but wait for the expiry of the T316 to initialize RRC Re-establishment procedure. If the duration of the T316 is set unnecessarily long, the UE has to experience long-time no data transmission service.

Figure 1: the SCG link becomes worse after the UE sends the MCGfailureInforamtion msg towards the SCG

However, on the other hand, the MN cannot take the risk of reducing the running duration of T316 with no priori knowledge of the SCG situation around a particular area within the coverage, because if so, under the circumstances of SCG links are good but the T316 expiry, the UE will have to initialize the RRC Re-establishment procedure before reception of the DL RRC Reconfiguration msg from the network, though the MN has already received SCG failure information msg from the SN.

Observation 3: setting the T316 with a too long timer duration may let UE wait long time before initializing the RRC-Reestablishment procedure, if the SCG link becomes worse after the UE sends the MCGFailureInformation msg towards the SCG. On the other hand, setting the T316 with short duration may result in unnecessary initialization of the RRC Re-establishment procedure, though it has already applied the fast MCG recovery procedure.

2.3 T316 optimization using SON

2.3.1 T316 expiry

As indicated above, when the T136 is expired but the SCG link is still good, the UE has to stop the fast MCG recovery procedure, but to initialize the RRC Re-establishment procedure. One way of enabling the network to further optimize the T316 setting (enlarge the T316 value) in such cases could be report the related information in the RRCReestablishmentRequest msg. For example, the ReestablishmentCause IE included in the RRCReestablishmentRequest could be extended to embrace the ‘T316 expiry’. Upon reception of such RRCReestablishmentRequest msg, the new serving cell could notify the previous connected cell where the fast MCG recovery is failed of the related information via the msg on the Xn interface.

Proposal 1: RAN2 to agree to report the fast MCG Recovery failure related information in the RRCReestablishmentRequest msg.

2.3.1 SCG failure/deactivation during fast MCG recovery

If a UE has already performed the MCG Recovery procedure but does not receive any further DL RRC msg from the split SRB1 or SRB3 before the expiry of the T316, the UE could store related information when the T316 is expired, and notify the network of the availability of such set of information during re-establishment procedure and send the corresponding RLF report towards the network when the network would like to collect the report from the UE.

In our opinion, at least following information needs to be included in the corresponding RLF report:

- Location information up to the MCG failure occurrence

- Failed PCell ID

- Indication of the fast MCG recovery failure occurrence

- The measurement result of the SCG up to the MCG failure occurrence (expected to be low)

Forward such UE report from the current serving cell to the previous serving cell where the MCG failure occurred could help it configure a more proper T316 timer, i.e., reduce the T316 value, towards UEs in proximity to the location indicated in the report.

Proposal 2: RAN2 to at least include following information in the RLF report for optimization of the T316 if SCG is failed or deactivated during fast MCG recovery:

- Location information up to the MCG failure occurrence

- Failed PCell ID

- Indication of the fast MCG recovery failure occurrence

- The measurement result of the SCG

2.3.2 SCG deactivation before fast MCG recovery

Regarding the last RAN2#121 meeting left FFS, the network could deactivate the UE SCG bearer before UE detects of the MCG failure for many reasons, such as power saving, user data high demand vanishes, etc. Even if the network could know that the MCG link failed just soon after the deactivation of the UE SCG bearer, still the network’s decision on whether or not to deactivate the UE SCG bearer should not be influenced, since the SCG on or off should only reflect the UE real-time demand, especially the data volume demand, and this is also the root reason why we introduce the dual connectivity in the 5G NR.

Observation 4: the network could deactivate the UE SCG bearer before UE detects of the MCG failure for many reasons, such as power saving, user data high demand vanishes, etc.

In addition, in our opinion, such scenario is not covered by the fast MCG recovery, which uses the currently available SCG for transmitting MCG recovery related info towards the UE. Correspondingly, SON on the fast MCG recovery should be focused on the optimization of the currently available SCG setting, but not the one that has already been deactivated.

Observation 5: the scenario of SCG being deactivated before fast MCG recovery is not covered by the fast MCG recovery which uses the available SCG for transmitting MCG recovery related info towards the UE, and therefore is out of the current WID scope.

As a result, we propose RAN2 to exclude the scenario of SCG deactivation before fast MCG recovery from SON on fast MCG recovery.

Proposal 3: RAN2 agree to exclude the scenario of SCG deactivation before fast MCG recovery from SON on fast MCG recovery.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following observations and proposals:

Observation 1: fast MCG recovery allows the UE to try its best to avoid having to perform RRC Re-establishment towards the MCG, if the SCG bearer quality is still good, by reconfiguring the MCG using the SCG.

Observation 2: a new timer T316 for fast MCG recovery was introduced. If the UE does not receive RRC msg from the SCG regarding the MCG reconfiguration/release/mobility command, when T316 is expired finally, the UE will still need to initialize the RRC re-establishment procedure.

Observation 3: setting the T316 with a too long timer duration may let UE wait long time before initializing the RRC-Reestablishment procedure, if the SCG link becomes worse after the UE sends the MCGFailureInformation msg towards the SCG. On the other hand, setting the T316 with short duration may result in unnecessary initialization of the RRC Re-establishment procedure, though it has already applied the fast MCG recovery procedure.

Proposal 1: RAN2 to agree to report the fast MCG Recovery failure related information in the RRCReestablishmentRequest msg.

Proposal 2: RAN2 to at least include following information in the RLF report for optimization of the T316 if SCG is failed or deactivated during fast MCG recovery:

- Location information up to the MCG failure occurrence

- Failed PCell ID

- Indication of the fast MCG recovery failure occurrence

- The measurement result of the SCG

Observation 4: the network could deactivate the UE SCG bearer before UE detects of the MCG failure for many reasons, such as power saving, user data high demand vanishes, etc.

Observation 5: the scenario of SCG being deactivated before fast MCG recovery is not covered by the fast MCG recovery, which uses the available SCG for transmitting MCG recovery related info towards the UE and therefore is out of the current WID scope.

Proposal 3: RAN2 agree to exclude the scenario of SCG deactivation before fast MCG recovery from SON on fast MCG recovery.