3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303938

Electronic, 17th April – 26th April 2023

Agenda Item:	7.17.3

Source: 	ASUSTeK

Title:  	Discussion on temporary capability restriction for Dual Tx/Rx Multi-SIM

Document for:	Discussion and Decision

- Introduction

In RAN2#120 meeting, Dual transmission/reception for MUSIM and temporary capability restriction including maximum UL power change for MUSIM have been discussed and some agreements have been made:

A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted: • Transmission and reception capabilities (e.g. MIMO layers) • Measurement capabilities (e.g. gaps) • Supported bandwidth • Supported band-combinations (FFS whether this is CA or DC or both) FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion). FFS whether UE reports some or all of the above or whether we can do something simpler … 2: RAN2 considers that there may be RAN4 impact on the maximum UL power change due to R18 MUSIM. However, RAN2 needs to analyze the power issue more before asking RAN4 specifically. RAN2 considers that the checkpoint on RAN3/4 impacts has been done.

In this contribution, we continue discuss possible solutions for MUSIM procedure regarding temporary UE capability restriction and discuss maximum power change impact of MUSIM procedure.

- Discussion

UE capability restriction indication for dual Tx/Rx MUSIM

In the previous meeting discussion, it is agreed that some UE capabilities can be impacted due to Dual SIM when start/stopping connection with NW B while being in RRC connected mode with NW A:

A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted: • Transmission and reception capabilities (e.g. MIMO layers) • Measurement capabilities (e.g. gaps) • Supported bandwidth • Supported band-combinations (FFS whether this is CA or DC or both) FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion). FFS whether UE reports some or all of the above or whether we can do something simpler

While the band conflict and frequency level-impact was discussed, to further clarify the scenario of applying UE capability restriction and granularity of the temporary restriction, it is also needed to discuss whether the shared UE capabilities can be dynamically switched between PLMNs or not, in time domain-level, when UE operates in dual Tx/Rx communication. For example, the UE can share a part of UE capabilities between NW A and NW B in time-division manner like MUSIM solution for Rel-17. When the UE performs dual Tx/Rx communication to NW B, the part of UE capabilities will be used to communicate with NW B. If NW B uses the UE capabilities only in a periodic manner, the UE can still use the shared UE capabilities to communicate with NW A when NW B is not using the UE capabilities.

Proposal 1: 	RAN2 to discuss whether the UE capability restriction is applied continuously or periodically in NW A before removal of the restriction.

- Conclusion

We have the following proposal for MUSIM enhancements:

Proposal 1: 	RAN2 to discuss whether the UE capability restriction is applied continuously or periodically in NW A before removal of the restriction.

- Reference