3GPP TSG-RAN WG2 Meeting #121-bis electronic	R2-2304206

April 17-26, 2023

Agenda Item:		8.6

Source: 			Session chair (CMCC)

Title: 			Report from SON/MDT session

Document for:	Approval

Organizational:

- LSs – contact companies should flag LSs that need presenting.  Otherwise we will directly note them

- Running CRs will be endorsed to be used as baseline and moved to email discussion.  Further agreements will be captured on that baseline CR.

- Only Email discussions and summary discussions will be treated during e-meetings (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT121][800][SON/MDT] Organizational Hu

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to SON/MDT

    - Share meetings notes and agreements for review and endorsement

## 5.4	SON MDT support for NR

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776).

### 5.4.1	General and stage-2 corrections

Including incoming LSs, TS 37.320 corrections

### 5.4.2	TS 38.314 corrections

### 5.4.3	RRC corrections

R2-2302942	Clarification on RLF Cause	Samsung	discussion	NR_SON_MDT-Core

R2-2302943	Clarification on RLF cause (Option 1)	Samsung	CR	Rel-16	38.331	16.12.0	3972	-	F	NR_SON_MDT-Core

R2-2302952	Clarification on RLF cause (Option 2)	Samsung	CR	Rel-16	38.331	16.12.0	3973	-	F	NR_SON_MDT-Core

R2-2303447	Correction on logging RLM resources in the RLF report	Ericsson, Qualcomm	discussion	Rel-16	38.331	NR_SON_MDT-Core

R2-2303448	Correction on logging RLM resources in the RLF report	Ericsson, Qualcomm	discussion	Rel-17	38.331	NR_SON_MDT-Core

R2-2303449	Correction to the setting of locationInfo in MeasResultSCG-Failure	Ericsson	discussion	Rel-16	38.331	NR_SON_MDT-Core

R2-2303450	Correction to the setting of locationInfo in MeasResultSCG-Failure	Ericsson	discussion	Rel-17	38.331	NR_SON_MDT-Core

R2-2303897	Discussion on location configuration for SON and MDT features	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

## 6.9	SON MDT

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

Tdoc Limitation: 2 tdocs

### 6.9.1	Stage-2

Stage-2 corrections and system level discussions.

R2-2302451	Reply LS on the user consent for trace reporting (S3-231398; contact: Huawei)	SA3	LS in	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3	Cc:RAN2, SA5, SA1, RAN

R2-2302460	LS on Excess Packet Delay Threshold for MDT (S5-232150; contact: Nokia)	SA5	LS in	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3	Cc:RAN2

R2-2302863	Correction to NR M3 measurement	Nokia, Nokia Shanghai Bell	CR	Rel-17	37.320	17.3.0	0124	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2303898	Discussion on the UL PDCP packet average delay measurement of split bearer	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2303899	Stage-2 correction on the UL PDCP packet average delay	Huawei, HiSilicon	CR	Rel-17	37.320	17.3.0	0125	-	F	NR_ENDC_SON_MDT_enh-Core

### 6.9.3	SON Corrections

6.9.4	MDT Corrections

R2-2302611	Correction on timeSinceCHO-Reconfig in TS 38.331	CATT	CR	Rel-17	38.331	17.4.0	3953	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2302612	Correction on SCG failure scenario of MHI in TS 38.331	CATT	CR	Rel-17	38.331	17.4.0	3954	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2302653	Report of new packet loss rate	China Unicom	report	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2303451	Correction to the handling of RLF-Report after successful HO	Ericsson	discussion	Rel-17	38.331	NR_ENDC_SON_MDT_enh-Core

R2-2303452	On including TAC in the SHR	Ericsson	discussion	Rel-17	38.331	NR_ENDC_SON_MDT_enh-Core

R2-2303646	Correction to timeSCGFailure	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.4.0	4020	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2303696	NB-IoT UE location Info in RLF report	Qualcomm Incorporated	discussion	Rel-17

R2-2303717	Correction on UE location information in NB-IoT RLF report	Qualcomm Inc.	CR	Rel-17	36.331	17.4.0	4924	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2304083	38.314 CR for the introduction of packet loss rate with delay threshold	China Unicom, CATT	CR	Rel-17	38.314	17.2.0	0027	-	B	NR_ENDC_SON_MDT_enh-Core

## 7.13	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

Includes LS in’s related to AI/ML for NG-RAN

Time budget: 0.5 TU

Tdoc Limitation: 3 tdocs

### 7.13.1	Organizational

Ls in Rapporteur input.

R2-2302423	LS on MRO for CPC and CPA and fast MCG recovery (R3-230992; contact: Huawei)	RAN3	LS in	Rel-18	NR_ENDC_SON_MDT_enh2-Core	To:RAN2

R2-2302452	Reply LS on user consent of Non-public Network (S3-231399; contact: Vodafone)	SA3	LS in	Rel-18	NR_ENDC_SON_MDT_enh2-Core	To:RAN3	Cc:RAN2, SA5

### 7.13.2	MRO for inter-system handover for voice fallback

R2-2304198	[Pre121bis-e][822][SON/MDT] Summary of agenda item 7.13.2 on MRO for inter-system handover for voice fallback	CATT	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

R2-2302613	Consideration on Inter-system Handover for Voice Fallback	CATT	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303143	Consideration on MRO for inter-system handover for voice fallback	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2303183	Further discussion on MRO of inter-system HO voice fallback	OPPO	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core	Late

R2-2303244	MRO for inter-system handover for voice fallback	Lenovo	discussion	Rel-18

R2-2303453	MRO for inter-system handover for voice fallback	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

R2-2303683	MRO for inter-system handover for voice fallback	Samsung R&D Institute India	discussion

R2-2303694	Data collection for MRO for inter-system handover for voice fallback 	Qualcomm Incorporated 	discussion	Rel-18

R2-2303956	Discussion on MRO for inter-system handover for voice fallback	Huawei, HiSilicon	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

### 7.13.3	MDT override

Will not be treated in #121b

### 7.13.4 	SHR and SPCR

Will not be treated in #121b

### 7.13.5	SON for NR-U

Focus on UE impacts. RAN2/RAN3 progress should be considered.

Will not be treated in #121b online session. Offline email discussion is possible.

R2-2304200	Summary of AI 7.13.5 SON for NR-U (Ericsson)	Ericsson discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

Proposal 1 Only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam.

Proposal 3 For the RA-Report, the enhancements on the handling of the “per RA attempt info list” (i.e. as per Proposal 1) apply only to the last RA procedure in the last BWP prior to the random access success.

Proposal 4 For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs:

a The locationAndBandwidth information of the BWP

b The subcarrierSpacing information of the BWP

c The absoluteFrequencyPointA information of the BWP (logged once for all the BWPs of the cell)

Proposal 8  As baseline, RAN2 assumes the following:

a Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b  The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c The above bullets may be revisited case by case depending on future agreements.

Proposal 10 The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy).

Proposal 13 The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

Proposal 15 The UE should log the following RSSI values in the RLF-Report:

a For RLF, the latest measured RSSI of the NR-U channel of the last serving cell

b For HOF, the latest measured RSSI of the NR-U channel of the source cell, and if available, the latest measured RSSI of the NR-U channel of the target cell

Proposal 20 The UE logs in the SHR the random access information, same as for the RA- and RLF-Report, i.e. including the number of UL LBT failures during HO, and the information on the multiple BWPs in which consistent UL LBT failures was triggered

Proposal 22 RAN2 to postpone discussion of the LBT information in MDT.

Proposal 23 RAN2 to keep discussing in RAN2#122 the proposals in this document (R2-2304200) not yet discussed.

R2-2302857	Discussion on SON for NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2302858	Discussion on storing LBT-FailureRecoveryConfig (Reply LS to R2-2300031)	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303113	SON Enhancement for NR-U	CATT	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303144	Consideration on NR-U related SON	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2303245	Discussion on MRO for NR-U	Lenovo	discussion	Rel-18

R2-2303673	SON/MDT enhancements for NR-U	Samsung R&D Institute India	discussion

R2-2303695	Discussion on NR-U Related Enhancements	Qualcomm Incorporated 	discussion	Rel-18

R2-2303803	SONMDT enhancement for NR-U	CMCC	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core	Late

R2-2304031	Discussion on SON for NR-U	Xiaomi	discussion	Rel-18

R2-2304111	Enhancements of SON reports for NR-U	Ericsson	discussion

### 7.13.6	RACH enhancement

R2-2304196		Summary of 7.13.6 RACH enhancement	Apple	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

=>	1	FFS: Include the actual number of msg3 repetitions in RA report.

2	FFS: Include NSAG priority in RA report.

3b	FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1.

3c	FFS: Include S-NSSAI(s) in RA report.

R2-2302614	RACH enhancement for SON	CATT	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2302856	RA report retrieval	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303145	Consideration on RACH enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2303368	Remaining issues of SON enhancements for RACH	Apple	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303454	RA report enhancement	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

R2-2303670	SON/MDT enhancements for RACH	Samsung R&D Institute India	discussion

R2-2303783	Discussion on RACH enhancement for SON	China Telecom	discussion	Late

R2-2303798	Further considerations on RACH Enhancement	CMCC	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core	Late

R2-2303806	Consideration on the SON enhancements for RACH report	Beijing Xiaomi Software Tech	discussion	Rel-18	Late

R2-2303829	SON enhancement for RA report	Sharp	discussion

R2-2303957	Discussion on RACH enhancement	Huawei, HiSilicon	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

### 7.13.7	SON/MDT enhancements for Non-Public Networks

Will not be treated in #121

R2-2303958	Discussion on SON MDT enhancements for NPN and NR-U	Huawei, HiSilicon	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

### 7.13.8	Other

Will not be treated in #121b

R2-2303182	SON on fast MCG recovery	OPPO	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

R2-2303246	Discussion on MRO for CPAC and fast MCG link recovery	Lenovo	discussion	Rel-18

R2-2303787	Discussion on CPAC failure report	NTT DOCOMO, INC.	discussion

R2-2303799	Further considerations on fast MCG recovery	CMCC	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core	Late