3GPP RAN WG2 Meeting #121bis-e	(draft)R2-2304258

eMeeting April 17th – 26th, 2023

Agenda Item:	7.6.4

Source:	InterDigital (Rapporteur)

Title:	[DRAFT] Report of 	[AT121bis-e][115][IoT NTN Enh] Discontinuous coverage enhancements (Interdigital)

Document for:	Discussion, Decision

# Introduction

This document is intended address contributions on discontinuous coverage enhancements for non-terrestrial networks submitted to AI 7.6.4, as per the following:

[AT121bis-e][115][IoT NTN Enh] Discontinuous coverage enhancements (Interdigital)

Scope: Discuss possible discontinuous coverage enhancements based on R2-2303716 and possibly including proposals from other contributions as well (R2-2302822, R2-2303193)

Intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Deadline for companies' feedback: Tuesday 2023-04-25 06:00 UTC

Deadline for rapporteur's summary (in R2-2304258): Tuesday 2023-04-25 08:00 UTC

Proposals marked "for agreement" in R2-2304258 not challenged until Tuesday 2023-04-25 20:00 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue online in the Wednesday CB session).

# Background

In RAN2#121 the following agreements were made:

Agreements: - RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage. - RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details - RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process) - Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID

# Discussion

## Dedicated RRC signalling for providing satellite information corresponding to discontinuous coverage

The following agreement was reached in RAN2#120:

Agreement: RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage

The following summarizes RAN2#121bis-e contributions discussing this issue:

[2] (CATT) Proposal 2：Providing satellite information corresponding to discontinuous coverage via dedicated signaling (not RRC Release message) is not needed, or at least should be de-prioritized.

[3] (CMCC) Proposal 1: RAN2 discuss to transfer more satellite assistance information via dedicated signaling, SIB segmentation or multiple SIBs.

[10] (Nokia) Proposal 1: RAN2 to discuss the provisioning of additional satellites’ ephemeris via dedicated RRC signalling.

Question 1)	Do you support introduction of dedicated signalling for providing satellite information corresponding to discontinuous coverage to the UE? If so, please give details.

| Company | Yes/No | Additional comments |
| --- | --- | --- |

## Enhancements in paging and eDRX

The following agreement was reached in RAN2#120:

Agreement: RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details

The following summarizes RAN2#121bis-e contributions discussing solution directions:

[1] (Apple) Proposal 1: RAN2 to discuss whether to address the mismatch issue between PTW and actual coverage time.

[2] (CATT) Proposal 4：Enhancements on paging and eDRX should be supported to avoid UE monitoring paging during the UE unreachability periods, and to guarantee the UE reachability during the coverage periods. [2] (CATT) Proposal 5：The calculations related with paging/eDRX (e.g., PH/PO, PH or PTW) should not be changed.

[6] (Huawei) Proposal 1: NW should be able to configure multiple Paging Time Windows during an eDRX cycle.

[7] (Interdigital) Proposal 2: PTW can be adjusted with co-ordination between UE and NW to account for UE unreachability periods. [7] (Interdigital) Proposal 3: RAN2 to down select between the following options for PTW adjustment: - Option 1 (configurable offset) - Option 2 (updated PTW calculation) - Option 3 (UE/NW autonomous adjustment) - Option 4 (other?)

[8] (Lenovo) Proposal 2: RAN2 to consider enhancements to PSM/eDRX for aligning PSM/eDRX duration with coverage interruption period at UE.

[10] (Nokia) Proposal 3: The PSM and eDRX configurations can be configured to align with the estimated UE unreachability period [10] (Nokia) Proposal 4: Network can extend the paging before/after the coverage window if the UE does not respond to paging within the estimated coverage window. [10] (Nokia) Proposal 5: UE can extend the paging monitoring outside the estimated coverage window if radio coverage is available. UE may report to the network to realign the paging monitoring and coverage windows.

[14] (Spreadtrum) Proposal 1: In order to ensure that the UE can be reachable when paging arrives, the PTW should aligns with the time duration of coverage and the following two options can be taken into further consideration: Option 1: both UE and network adjust PTW based on a predefined rule Option 2: network provides multiple PTW info

[16] (Xiaomi) Proposal 2: UE determines the paging cycle only based on UE specific DRX/eDRX configured by AMF if the AMF takes the UE coverage information into consideration when configures the UE specific DRX/eDRX.

[17] (ZTE) Proposal 1a: If legacy eDRX is used for keeping alignment between UE and NW during discontinuous coverage, in order to align the starting time of PTW with the out-of-coverage period or unreachability period, it’s suggested to introduce a configurable offset to shift the starting time of PTW.

The following detailed options are proposed, based on the contributions listed:

- Option 1 (configurable offset) [17], [14]

- Option 2 (updated PTW calculation) [7], [14]

- Option 3 (multiple PTW during an eDRX cycle can be configured) [6], [14]

- Option 4 (UE/NW autonomous adjustment + UE may report to the network to realign the paging monitoring and coverage windows) [10]

- Option 5 (UE determines the paging cycle only based on UE specific DRX/eDRX configured by AMF) [16]

As a first step, the above solutions can be put into 3 solution categories. If one of the below options can be selected, then

- Option A: Updated PTW configuration (options 1, 2, 3)

- Option B: UE autonomous adjustment (option 4)

- Option C: No RAN impact (option 5)

- Other?

Question 2)	Do you support?

- Option A: Updated PTW configuration (details FFS, e.g. configurable offset, updated PTW calculation, multiple PTW configurations)

- Option B: UE autonomous adjustment (details FFS, e.g. extend the paging monitoring, report to NW)

- Option C: No RAN impact (revert the agreement from last meeting)

- Other? (please describe)

| Company | A/B/C/other | Additional comments |
| --- | --- | --- |

## UE assistance information

The following summarizes RAN2#121bis-e contributions discussing UE assistance information and UE unreachability period:

[1] (Apple) Proposal 2: UE assistance information can be used for PTW adjustment. [1] (Apple) Proposal 3: Send an LS to SA2/CT1 on unevenly distributed coverage period caused by unevenly distributed satellite from multiple satellites.

[5] (Google) Proposal 3 A RRC_CONNECTED UE can inform the network of the remaining time that the UE will be within the satellite coverage before entering an unreachability period, using a RRC message (e.g., UEAssistanceInformation).

[7] (InterDigital) Proposal 7: RAN2 to discuss whether it can be assumed that reporting of UE unreachability period in Registration Request can be kept sufficiently (i.e. to support AS based solutions) up to date and takes into consideration UE mobility, or whether this needs to be confirmed with SA2.

[9] (NEC) Proposal 1: Consider to support UE providing assistance information on being out-of-coverage.

[17] (ZTE) Proposal 1b: The out-of-coverage period or unreachability period should be informed to RAN, e.g., from core network node, to assist RAN to provide a more appropriate paging schedule for UE in idle mode. [17] (ZTE) Proposal 2d: The UE in connected mode could provide out-of-coverage period or unreachability period information as an assistance to the network (eNB).

Given that SA2 have already agreed UE reporting of the unreachability period, we may not need to introduce any further RAN information. However, based on the above proposals, we may additionally need RAN assistance information and/or this information may need to be provided to RAN for PTW calculation. In addition we may need to check with SA2 whether the reported UE unreachability is sufficient for RAN needs.

Question 3a)	Do you assume RAN assistance information is needed in addition to UE unreachability period in Registration Request? If so, please indicate what/why.

| Company | Y/N | Additional comments |
| --- | --- | --- |

Question 3b)	Do you agree that clarification from SA2 (i.e. with an LS) is needed regarding using, in RAN, the UE unreachability period reported in Registration Request?

| Company | Y/N | Additional comments |
| --- | --- | --- |

## Enhancements for connected UE

The following agreement was reached in RAN2#120:

Agreement: RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process)

The following summarizes RAN2#121bis-e contributions discussing this issue:

[1] (Apple) Proposal 4: UE follows legacy operation in declaring RLF upon entering coverage gap..

[7] (Interdigital) Proposal 4: For eMTC and NB-IoT: RLM, RLF detection, and RRC re-establishment are suspended during a UE unreachability period. [7] (InterDigital) Proposal 5: For eMTC: To avoid always sending the UE to RRC_IDLE/RRC_INACTIVE during a UE unreachability period, introduce an activation time in RRC Reconfiguration to allow handover between cells occurring before and after a UE unreachability period. [7] (Interdigital) Proposal 6: For eMTC: Consider how discontinuous coverage impacts CHO.

Question 4)	Do you think any enhancement is needed to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap? (Details FFS, e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement)

| Company | Y/N | Additional comments |
| --- | --- | --- |

## RRC Release

Several companies have proposed enhancements to RRC Release in this meeting.

The following summarizes RAN2#121bis-e contributions discussing this:

[2] (CATT) Proposal 3：RRC Release message can be updated to enhance discontinuous coverage.

[3] (CMCC) Proposal 2: The UE can provide the out-of-coverage information to RAN node for e.g, optimization on RRC release or paging. [3] (CMCC) Proposal 4: A new cause value ‘Release due to discontinuous coverage’ is introduced in RRCRelease message.

[10] (Nokia) Proposal 8: RAN2 to discuss support for UE request for RRC connection release based on DC estimation. FFS support for implicit RRC connection release.

[11] (Qualcomm) Proposal 2 Reuse NR MUSIM timer T346g behavior, i.e., upon expiry of the out-of-coverage timer, the UE performs the actions upon leaving RRC_CONNECTED, with release cause 'other'.

[12] (Rakuten) Proposal 3a: eNB to initiate RRC release based on relative UE location with respect to beam footprint, Proper cause value have to be added into spec. [12] (Rakuten) Proposal 3b: UE itself release network based on configured Timing Advance (TA) value communicated by NTN cell. Proper cause value needs to be added in to spec.

[17] (ZTE) Proposal 2a: A new release reason, e.g., ‘Release due to discontinuous coverage’ as that introduced in RAN3, can be introduced in RRC release message for indicating UE to stop the subsequent AS layer processes after it is released to idle mode. [17] (ZTE) Proposal 2b: An AS-NAS interaction (e.g., an indication from AS to NAS) also needs to be introduced for indicating UE to stop the subsequent NAS layer processes after it is released to idle mode due to discontinuous coverage. [17] (ZTE) Proposal 2c: The legacy IE extendedWaitTime can be reused to stop the subsequent NAS layer processes after UE is released to idle mode due to discontinuous coverage. The extension to the value range of extendedWaitTime needs to be discussed.

Question 5)	Do you agree to enhance RRC Release for the discontinuous coverage? (Details FFS, e.g. new cause value, UE timer/trigger)

| Company | Y/N | Additional comments |
| --- | --- | --- |

## Others

Question 6)	Companies may list any other discontinuous coverage enhancements that should be discussed in the table below.

| Company | Additional comments |
| --- | --- |

# Conclusions

<To be generated based on company input>

# References (In alphabetical order by company)

| Reference | TDoc | Title | Company | Proposals |
| --- | --- | --- | --- | --- |
| [1] | R2-2303407 | Support on discontinuous coverage in IoT NTN | Apple | Proposal 1: RAN2 to discuss whether to address the mismatch issue between PTW and actual coverage time. |
|  |  |  |  | Proposal 2: UE assistance information can be used for PTW adjustment. |
|  |  |  |  | Proposal 3: Send an LS to SA2/CT1 on unevenly distributed coverage period caused by unevenly distributed satellite from multiple satellites. |
|  |  |  |  | Proposal 4: UE follows legacy operation in declaring RLF upon entering coverage gap. |
|  |  |  |  | Proposal 5: During T311, UE does not perform cell search if UE is in coverage gap. |
|  |  |  |  | Proposal 6: RAN2 to discuss whether to support discontinuous coverage scenario in inactive state. |
| [2] | R2-2302560 | Discussion on enhancements to discontinuous coverage | CATT | Proposal 1：Some enhancement is needed for HARQ process with HARQ enabling when there is no enough time for ACK/NACK feedback because of the incoming coverage hole. |
|  |  |  |  | Proposal 2：Providing satellite information corresponding to discontinuous coverage via dedicated signaling (not RRC Release message) is not needed, or at least should be de-prioritized. |
|  |  |  |  | Proposal 3：RRC Release message can be updated to enhance discontinuous coverage. |
|  |  |  |  | Proposal 4：Enhancements on paging and eDRX should be supported to avoid UE monitoring paging during the UE unreachability periods, and to guarantee the UE reachability during the coverage periods. |
|  |  |  |  | Proposal 5：The calculations related with paging/eDRX (e.g., PH/PO, PH or PTW) should not be changed. |
|  |  |  |  | Proposal 6：Then enhancement for connected UE upon coming of discontinuous coverage should be de-prioritized. |
| [3] | R2-2303520 | Discussion on the discontinuous coverage for IoT-NTN | CMCC | Proposal 1: RAN2 discuss to transfer more satellite assistance information via dedicated signaling, SIB segmentation or multiple SIBs. |
|  |  |  |  | Proposal 2: The UE can provide the out-of-coverage information to RAN node for e.g, optimization on RRC release or paging. |
|  |  |  |  | Proposal 3: The UE out-of-coverage information can be kept by RAN node when the UE enters into idle mode. |
|  |  |  |  | Proposal 4: A new cause value ‘Release due to discontinuous coverage’ is introduced in RRCRelease message. |
| [4] | R2-2303735 | Enhancements to discontinuous coverage | Ericsson | Proposal 1 Provide measurement assistance information, e.g., PCI or carrier frequency, in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap. |
| [5] | R2-2304081 | Discussion on the UE Unreachability Periods | Google Inc. | Proposal 1 The UE AS notifies the UE NAS at the time when the UE is about to leave the network coverage, and/or when the UE is back to the network coverage. The UE AS may include the information regarding how long the UE would be out of network coverage while notifying the UE NAS of leaving the network coverage. |
|  |  |  |  | Proposal 2 Upon determining that the UE has entered an unreachability period, a RRC_CONNECTED UE transitions to RRC_IDLE immediately without starting T310 or T311. If T310 or T311 has been started already upon the determination, the UE stops T310 or T311 and transitions to RRC_IDLE directly. |
|  |  |  |  | Proposal 3 A RRC_CONNECTED UE can inform the network of the remaining time that the UE will be within the satellite coverage before entering an unreachability period, using a RRC message (e.g., UEAssistanceInformation). |
| [6] | R2-2303963 | Discussion on discontinuous coverage | Huawei, HiSilicon | Proposal 1: NW should be able to configure multiple Paging Time Windows during an eDRX cycle. |
|  |  |  |  | Proposal 2: The network can derive multiple coverage windows and takes them into account when paging a UE. |
|  |  |  |  | Proposal 3: UE enters RRC_IDLE if the remaining serving time is less than a threshold in discontinuous coverage scenario. |
|  |  |  |  | Proposal 4: UE stops the AS idle mode tasks related to TN only when there is no TN cells in the discontinuous coverage. |
| [7] | R2-2303716 | IoT-NTN discontinuous coverage enhancements | Interdigital, Inc. | Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period. |
|  |  |  |  | Proposal 2: PTW can be adjusted with co-ordination between UE and NW to account for UE unreachability periods. |
|  |  |  |  | Proposal 3: RAN2 to down select between the following options for PTW adjustment: |
|  |  |  |  | - Option 1 (configurable offset) |
|  |  |  |  | - Option 2 (updated PTW calculation) |
|  |  |  |  | - Option 3 (UE/NW autonomous adjustment) |
|  |  |  |  | - Option 4 (other?) |
|  |  |  |  | Proposal 4: For eMTC and NB-IoT: RLM, RLF detection, and RRC re-establishment are suspended during a UE unreachability period. |
|  |  |  |  | Proposal 5: For eMTC: To avoid always sending the UE to RRC_IDLE/RRC_INACTIVE during a UE unreachability period, introduce an activation time in RRC Reconfiguration to allow handover between cells occurring before and after a UE unreachability period. |
|  |  |  |  | Proposal 6: For eMTC: Consider how discontinuous coverage impacts CHO. |
|  |  |  |  | Proposal 7: RAN2 to discuss whether it can be assumed that reporting of UE unreachability period in Registration Request can be kept sufficiently (i.e. to support AS based solutions) up to date and takes into consideration UE mobility, or whether this needs to be confirmed with SA2. |
| [8] | R2-2303253 | On mobility and power saving issues for discontinuous coverage | Lenovo | Proposal 1: RAN2 to consider enhancements to UE reporting its prediction of coverage interruption to the network. |
|  |  |  |  | Proposal 2: RAN2 to consider enhancements to PSM/eDRX for aligning PSM/eDRX duration with coverage interruption period at UE. |
|  |  |  |  | Proposal 3: RAN2 to consider enhancements to disable CONNECTED neighbour cell measurement for NB-IoT UE before coverage interruption due to discontinuous coverage. |
|  |  |  |  | Proposal 4: RAN2 to consider enhancements to RRC connection recovery configuration before coverage interruption, so that UE can recovery RRC connection when coverage restores. |
| [9] | R2-2303111 | Considerations on Supporting Discontinuous Coverage | NEC Europe Ltd | Proposal 1: Consider to support UE providing assistance information on being out-of-coverage. |
|  |  |  |  | Proposal 2: Consider enhancements to existing IoT features such as PUR, to support periodical, brief connection in IoT-NTN discontinuous coverage. |
| [10] | R2-2303193 | On RAN impacts for Discontineous coverage enhancements | Nokia, Nokia Shanghai Bell | Proposal 1: RAN2 to discuss the provisioning of additional satellites’ ephemeris via dedicated RRC signalling. |
|  |  |  |  | Proposal 2: RAN2 to include footprint information for the earth-moving cell in discontinuous coverage as an optional field in SIB31. |
|  |  |  |  | Proposal 3: The PSM and eDRX configurations can be configured to align with the estimated UE unreachability period |
|  |  |  |  | Proposal 4: Network can extend the paging before/after the coverage window if the UE does not respond to paging within the estimated coverage window. |
|  |  |  |  | Proposal 5: UE can extend the paging monitoring outside the estimated coverage window if radio coverage is available. UE may report to the network to realign the paging monitoring and coverage windows. |
|  |  |  |  | Proposal 6: RAN2 to discuss how to handle UE movement within the same TA during discontinuous coverage. |
|  |  |  |  | Proposal 7: RAN2 to consider provisioning cell availability information to enable moving and cold start UEs to determine cell search and reselection measurement period(s) to enhance energy-saving potential. |
|  |  |  |  | Connected mode functionality impacts for DC related enhancements |
|  |  |  |  | Proposal 8: RAN2 to discuss support for UE request for RRC connection release based on DC estimation. FFS support for implicit RRC connection release. |
|  |  |  |  | Proposal 9: RAN2 to discuss UE behaviour when the UE has limited remaining GNSS validity duration and the remaining discontinuous coverage time is also short. |
| [11] | R2-2303042 | RRC release procedure in discontinuous coverage | Qualcomm Incorporated | Proposal 1 If the UE is able to predict when the discontinuous coverage starts, reuse NR MUSIM procedure to leave RRC_CONNECTED state where the UE indicates the out-of-coverage to network and starts an out-of-coverage timer. |
|  |  |  |  | Proposal 2 Reuse NR MUSIM timer T346g behavior, i.e., upon expiry of the out-of-coverage timer, the UE performs the actions upon leaving RRC_CONNECTED, with release cause 'other'. |
|  |  |  |  | Proposal 3 Further discuss the details on the values of the out-of-coverage timer and message to carry out-of-coverage information. |
| [12] | R2-2304160 | Discussion on Enhancements related to discontinuous coverage | Rakuten Mobile, Inc | Proposal 1: UEs should be provided with satellite ephemeris data for neighbor cells in 2 groups, “Default Neighbor ephemeris data” shall contain limited amount of neighbors & “Extended Neighbor Ephemeris Data” may contain more than default neighbors till defined by network. |
|  |  |  |  | Proposal 2: NB IoT UEs should use extended satellite ephemeris data to calculate T3212 extended timer value along with T3324 for activating PSM. |
|  |  |  |  | Proposal 3a: eNB to initiate RRC release based on relative UE location with respect to beam footprint, Proper cause value have to be added into spec. |
|  |  |  |  | Proposal 3b: UE itself release network based on configured Timing Advance (TA) value communicated by NTN cell. Proper cause value needs to be added in to spec. |
| [13] | R2-2303052 | Enhancements to discontinuous coverage | Samsung R&D Institute UK | Proposal 1: Measurement assistance information can be provided to control how the UE shall perform idle mode tasks (i.e. whether a UE can power down specific frequencies during discontinuous coverage) in a discontinuous coverage NTN. |
|  |  |  |  | Proposal 2(a): The UE verifies whether it has sufficient coverage time to complete a given RRC procedure (or a new connection establishment). |
|  |  |  |  | Proposal 2(b): The UE initiates a given RRC procedure if it has sufficient coverage time to complete this procedure. Otherwise, the UE will not initiate the procedure or will wait for the next available satellite coverage period. |
|  |  |  |  | Proposal 3: Discontinuous coverage-related information is provided in a re-direct message when being re-directed to a discontinuous coverage network. |
| [14] | R2-2303576 | Discussion on power saving enhancements for supporting discontinuous coverage | Spreadtrum Communications | Proposal 1: In order to ensure that the UE can be reachable when paging arrives, the PTW should aligns with the time duration of coverage and the following two options can be taken into further consideration: |
|  |  |  |  | Option 1: both UE and network adjust PTW based on a predefined rule |
|  |  |  |  | Option 2: network provides multiple PTW info |
|  |  |  |  | Proposal 2: From RAN2 point of view, the existing PSM mechanism needs to be enhanced in order to get better power effect. |
|  |  |  |  | Proposal 3：The PSM starting mechanism needs to be modified to adapt to the scenario of discontinuous coverage. |
|  |  |  |  | Proposal 4: A time threshold needs to be introduced for UE to determine whether to start T3324 timer. |
| [15] | R2-2303476 | Discussion on enhancement to discontinuous coverage for IoT NTN | Transsion Holdings | Proposal 1 RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time. |
|  |  |  |  | Proposal 2 The discontinuous coverage wait timer can be used in AS to avoid RACH congestion. |
|  |  |  |  | Proposal 3 The CN should be aware of the UE’s discontinuous coverage period to CN to avoid paging issues. |
|  |  |  |  | Proposal 4 The satellite assistance information or the discontinuous coverage information predicted by UE can be provided to CN for paging. |
| [16] | R2-2303437 | Enhancements to discontinuous coverage | Xiaomi | Proposal 1: The assistance information of target cells can be provided to UE in advance for UE performing cell selection/reselection when UE returns to coverage and the assistance information could be frequency, PCI and SSB configuration. |
|  |  |  |  | Proposal 2: UE determines the paging cycle only based on UE specific DRX/eDRX configured by AMF if the AMF takes the UE coverage information into consideration when configures the UE specific DRX/eDRX. |
|  |  |  |  | Proposal 3: AMF provides the UE out of coverage period to the gNB. |
|  |  |  |  | Proposal 4: If UE is in discontinuous coverage, UE should go to RRC idle when detects radio link problem. |
| [17] | R2-2302822 | RAN2 enhancements for discontinuous coverage | ZTE Corporation, Sanechips | Proposal 1a: If legacy eDRX is used for keeping alignment between UE and NW during discontinuous coverage, in order to align the starting time of PTW with the out-of-coverage period or unreachability period, it’s suggested to introduce a configurable offset to shift the starting time of PTW. |
|  |  |  |  | Proposal 1b: The out-of-coverage period or unreachability period should be informed to RAN, e.g., from core network node, to assist RAN to provide a more appropriate paging schedule for UE in idle mode. |
|  |  |  |  | Proposal 2a: A new release reason, e.g., ‘Release due to discontinuous coverage’ as that introduced in RAN3, can be introduced in RRC release message for indicating UE to stop the subsequent AS layer processes after it is released to idle mode. |
|  |  |  |  | Proposal 2b: An AS-NAS interaction (e.g., an indication from AS to NAS) also needs to be introduced for indicating UE to stop the subsequent NAS layer processes after it is released to idle mode due to discontinuous coverage. |
|  |  |  |  | Proposal 2c: The legacy IE extendedWaitTime can be reused to stop the subsequent NAS layer processes after UE is released to idle mode due to discontinuous coverage. The extension to the value range of extendedWaitTime needs to be discussed. |
|  |  |  |  | Proposal 2d: The UE in connected mode could provide out-of-coverage period or unreachability period information as an assistance to the network (eNB). |