3GPP TSG-RAN2 Meeting 121bis                                             R2-230xxxx

E-meeting, April, 2023

Agenda Item:	7.9.1

Source:	LG

Title:	[AT121bis-e][418][Relay] 38.300 relay CR draft (LG)

Document for:	Discussion, Decision

Introduction

This is for the following offline discussion

[AT121bis-e][418][Relay] 38.300 relay CR draft (LG)

Scope: Collect comments on the CR outline in R2-2302994.

Intended outcome: Report to CB session and endorseable CR baseline

Deadline: Monday 2023-04-24 2359 UTC

Discussion

Contents for U2U Relay

The rel-18 U2U relay operation has some common parts and different parts compared to the rel-17 U2N relay operation. So, the common operation between the rel-17 U2N relay and the rel-18 U2U relay can be placed combined with the rel-17 U2N description part. The different operations between the rel-17 U2N relay and the rel-18 U2U relay can be placed on a separate sub-clause. We propose that common operations between the rel-18 U2U and the rel-17 U2N can be described in the following sub-clause: General (16.12.1 in TS 38.300), Relay Discovery (16.12.3 in TS 38.300) and Relay Selection/Reselection (16.12.4 in TS 38.300). And the rel-18 U2U operations differentiated from the rel-17 U2N can be described in the newly made sub-clause: L2 UE-to-UE Relay (16.12.2.x in TS 38.300) in protocol architecture (16.12.2 in TS 38.300) and Control plane procedures for L2 U2U Relay (16.12.x in TS 38.300).

Proposal 1: For the rel-18 U2U relay description, the common parts with the rel-17 U2N relay can be described in the current existing sub-clause in TS 38.300: General(16.12.1 in TS 38.300), Relay Discovery(16.12.3 in TS 38.300 in TS 38.300) and Relay Selection/Reselection(16.12.4 in TS 38.300 in TS 38.300).

Proposal 2: For the rel-18 U2U relay description, the different parts from the rel-17 U2N relay can be described in the newly made sub-clause in TX 38.300: L2 UE-to-UE Relay(16.12.2.x in TS 38.300 in TS 38.300) and Control plane procedures for L2 U2U Relay(16.12.x in TS 38.300 in TS 38.300) can be made for rel-18 U2U.

Q1) Do you agree that the following contents for U2U Relay are applied in 38.300 CR?

## 16.12 Sidelink Relay ### 16.12.1 General ### 16.12.2 Protocol Architecture #### 16.12.2.1 L2 UE-to-Network Relay #### 16.12.2.x L2 UE-to-UE Relay ### 16.12.3 Relay Discovery 16.12.4 Relay Selection/Reselection ### 16.12.5 Control plane procedures for L2 U2N Relay ### 16.12.x Control plane procedures for L2 U2U Relay

| Company | Yes/No | If no, please write down your opinion. |
| --- | --- | --- |
| LG | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| China Telecom | Yes |  |
| Futurewei | Yes |  |
| ZTE | Yes |  |

Contents for Service continuity

In rel-18 service continuity enhancement for U2N relay, inter-gNB D2I, I2D and I2I path switching, and intra-I2I path switching is introduced [2]. In the case of inter-gNB path switching, RAN2 can follow the RAN3 decision about which gNB will select path type and target relay UE. The procedure for path switching of inter-gNB can be added by the RAN3 side. However, the service continuity procedure for intra-i2i path switching can be added by the RAN2 side. We propose that inter-gNB D2I and I2D procedures are added to the intra-gNB D2I and I2D procedures without making a new sub-clause in the current sub-clause: switching from direct to indirect path (16.12.6.1 in TS 38.300) and switching from direct to indirect path (16.12.6.2 in TS 38.300). For the description of the intra/inter-gNB I2I procedures, a new sub-clause is needed to be made: switching from indirect to indirect path (16.12.6.x in TS 38.300).

Proposal 3: RAN3 can put the description of inter-gNB d2i, i2d, and i2i path-switching procedures in the 38.300 draft spec.

Proposal 4: For the description of rel-18 service continuity enhancement, a new sub-clause can be made for the description of inter/intra i2i path switching procedure: Switching from indirect to indirect path (16.12.6.x in TS 38.300).

Proposal 5: For the description of rel-18 service continuity enhancement, the description of inter-gNB d2i, i2d and i2i path switching procedures can be added in the service continuity part for intra-gNB d2i, i2d and i2i path switching procedures: Switching from indirect to direct path (16.12.6.1 in TS 38.300), Switching from direct to indirect path (16.12.6.2 in TS 38.300) and Switching from indirect to indirect path (16.12.6.x in TS 38.300).

Q2) Do you agree that the inter-gNB i2d/d2i/i2i path-switching procedures for service continuity can be added by RAN3 for 38.300 CR?

| Company | Yes/No | If no, please write down your opinion. |
| --- | --- | --- |
| LG | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| China Telecom | Yes |  |
| Futurewei | Yes |  |

Q3) Do you agree that the following contents for service continuity are applied in 38.300 CR?

### 16.12.6 Service Continuity for L2 U2N relay #### 16.12.6.0 General #### 16.12.6.1 Switching from indirect to direct path #### 16.12.6.2 Switching from direct to indirect path #### 16.12.6.x Switching from indirect to indirect path

| Company | Yes/No | If no, please write down your opinion. |
| --- | --- | --- |
| LG | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| China Telecom | Yes |  |
| Futurewei | Yes |  |
| ZTE | Yes |  |

## 2.3 Contents for Multi-path relay

Multi-path relay in rel-18 can be described separately from the rel-17 sidelink relay. In the point of view that the remote UE has two path, direct and indirect path, it can be differentiated from the rel-17 U2N sidelink relay. So, we propose making a new clause for the multi-path relay different from the sidelink relay clause.

Proposal 6: New clause different from a sidelink relay (16.12 in TS 38.300) can be made for the multi-path relay: Multi-path relay (16.X in TS 38.300)

Q4) Do you agree that the following contents for Multi-path Relay are applied in 38.300 CR?

## 16.X Multi-path Relay ### 16.X.1 General ### 16.X.2 Protocol Architecture ### 16.X.3 Control plane procedure for multi-path relaying

| Company | Yes/No | If no, please write down your opinion. |
| --- | --- | --- |
| LG | Yes |  |
| Xiaomi | Yes |  |
| CATT | See comments | For Rel-18 SL relay enhancement, there are three parts(U2U,service continuity enhancement and MP). The first two bullets are added under 16.12 in TS 38.300, while the last part(MP) is added under a new clause 16.x in TS 38.300.It is a little bit confused to us and we would like to hear more companies view on this point. |
| OPPO | No | Multi-path relay can also be a sub-clause of sidelink relay (no need for a different clause) since a lot of procedures for single path relay are reused for multi-path, e.g., the relay selection/discovery … |
| Lenovo | No | Other minor comments: - Control plane procedures for multi-path relaying. - Both ‘Multi-path Relay’ and ‘multi-path relaying’ are used. |
| China Telecom | No | We prefer to have a sub-clause for multi-path relay (16.12.x ) under the current sidelink relay (clause 16.12 in TS 38.300), which seems clearer. |
| Futurewei | No | Multi-path relay should be a sub-clause of sidelink relay, like the following: ## 16.12 Sidelink Relay ### 16.12.1 General ### 16.12.2 Protocol Architecture #### 16.12.2.1 L2 UE-to-Network Relay #### 16.12.2.x L2 UE-to-UE Relay #### 16.12.2.y Multi-path Relay ### 16.12.3 Relay Discovery 16.12.4 Relay Selection/Reselection ### 16.12.5 Control plane procedures for L2 U2N Relay ### 16.12.x Control plane procedures for L2 U2U Relay ### 16.12.y Control plane procedures for Multi-path Relay |
| ZTE | Yes | ### Considering that scenario 2 of multi-path relay has nothing to do with sidelink, it is better to have a separate clause for multi-path relay. |

- Conclusion

We have the following proposals:

Reference

- TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2,” v17.4.0

- [2] RP-221262, “Revised WID on NR sidelink relay enhancements,” LG Electronics, June, 2022.