3GPP TSG-RAN WG2 Meeting #122	R2-2304837

# Incheon, Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0668 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on stage-2 descriptions for measurement in NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2023-5-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current stage-2 specification specifies “For a UE in Connected mode, it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19”. According to TS 38.331, however, a connected mode UE directly uses the SMTC configuration (e.g., smtc4list) in the measurement configuration to perform neighbour cell measurement in an NTN cell. The NW configures the SMTC configuration based on service link PDD reported by UE, and the neighbour cell ephemeris for UE to calculate the service link PDD is configured by the NW through dedicated signalling (i.e., OtherConfig). This means that the measurement of a UE in connected mode is irrelevant to the neighbour cell assistance information included in the SIB19 (i.e. no need for autonomous SMTC adjustment); thus, there is no reason to associate the measurement of a UE in connected mode to the neighbour cell assistance information in SIB19 as current specified in TS 38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 16.14.3.3, remove the problematic description “For a UE in Connected mode, it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19.”. Impact Analysis Impacted 5G architecture options: NR standalone Impacted functionality: Neighbour cell measurement Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong IE name is used in TS 38.304 for NR SL pre-configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 16.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities;

-	measurement gaps based on multiple SMTCs;

-	assistance information (e.g., ephemeris, Common TA parameters) provided in SIB19 for UE to perform measurement on neighbour cells in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED.

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

UE assistance information consists of the service link propagation delay difference(s) between serving the cell and neighbour cell(s).

For a UE in Idle/Inactive mode it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB4 but not included in SIB19.

In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE:

-	The timing and location information associated to the serving cell is provided in SIB19;

-	Timing information refers to the UTC time when the serving cell stops serving the current geographical area;

-	Location information refers to the reference location of the serving cell and a distance threshold to the reference location.

Measurement rules for cell re-selection based on timing information and location information are specified in clause 5.2.4.2 in TS 38.304 [10].

End of changes