3GPP TSG-RAN WG2 Meeting #122		 R2-2304923

Incheon, Korea, 22nd– 26th May 2023

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	7.22.2

Document for:	Discussion and Decision

- Introduction

In RAN#97e meeting, the revised SID for R18 low-power Wake-up Signal and Receiver for NR was agreed [1], and in RAN2#121bis-e meeting, we have the following relevant agreements on LP-WUS and LP-WUR[2]:

Confirm that we follow R1 and include RRC idle/inactive/connected. Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off. In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

In this contribution, we will further discuss the detailed RRC_IDLE/RRC_INACTIVE procedures with LP-WUR, including the LP-WUS coverage, network awareness of ultra-deep sleep, entry and exit conditions for ultra-deep sleep state, paging enhancements, SI change notification and RRM and mobility aspects with LP-WUR considering RAN1’s progress.

- Discussion

  - General

According to the SID, UE could be equipped with an additional Rx module apart from the current Rx module. The corresponding terminologies were agreed by RAN1 as follows:

Use the following terminology for future discussion, - Main radio (MR): the Tx/Rx module operating for legacy NR signals/channels apart from signals/channel related to low-power wake-up. - LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

In RAN2#121bis meeting, we have agreed that ultra-deep-sleep is a power saving state in which MR may sleep/turn off, we will still use the terminology in this contribution for convenience.

For a UE equipped with LR, the MR can be in ultra-deep sleep state when there is no data exchange between UE and network, while the LR remains active to monitor Low-Power Wake-Up Signal (LP-WUS) in DRX or continuous manner. When there is downlink data/signaling to be sent to the UE, the MR should be waked up for normal data transmission. As LR would adopt a minimalistic design, the power consumption of LR is expected to be significantly lower than legacy PDCCH monitoring.

Observation 1: The power consumption for MR will be significantly reduced when MR is in ultra-deep sleep.

  - LP-WUS Coverage

In the last RAN2 meeting, one issue was raised which will have extensive impacts on RAN2 procedures is if LP-WUS has the full coverage, i.e. same coverage as existing reference signal, or partial coverage, i.e. smaller coverage than existing reference signal. With different LP-WUS coverage assumptions, different mechanisms need to be studied.

For full coverage of LP-WUS, there could be continuous coverage of LP-WUS when UE moves across the cells with LP-WUS deployment. In this case, UE’s MR could stay in ultra-deep sleep during mobility. RAN2 needs to study, e.g.

- Whether/how LP-WUS/SS based measurement can be used to perform cell selection/reselection to ensure the UE’s MR to stay in ultra-deep sleep as long as possible.

For partial coverage of LP-WUS, UE would be out of LP-WUS coverage when UE moves across the cells. In this case, UE’s MR needs to wake up during mobility. RAN2 needs to study, e.g.

- Whether/how the network needs to know exact information for one UE’s MR power state (in ultra-deep sleep or not)

In addition, there are also some issues related to both full and partial coverage cases, e.g.

- The conditions for UE’s MR to enter into/exit the ultra-deep sleep

- Whether the cell information, tracking area information and RAN area information needs to be included in LP-WUS

The coverage of LP-WUS/LP-SS is being discussed in RAN1, which depends on the receiver design/signal design. Considering the current RAN1 progress, we think RAN2 study should cover the scenarios with both full and partial coverage.

Proposal 1: RAN2 needs to study mechanisms to support both full and partial coverage of LP-WUS/SS.

  - Network awareness

As mentioned above, the UE needs to wake to when it is out of the coverage of LP-WUS, and the UE’s MR may enter ultra-deep sleep again when it moves into the coverage of LP-WUS. Thus, whether the network is aware of UE’s MR power state, i.e. in ultra-deep sleep or not, should be discussed. Besides, in order to page the UE with LP-WUR, whether the gNB should send LP-WUS or just send legacy [PEI+] Paging also depends on whether the network is aware of UE’s MR power state, i.e. in ultra-deep sleep or not.

If the network is aware of UE’s MR power state, i.e. in ultra-deep sleep or not, it is beneficial for the network on resource efficiency, but it will lead additional procedure with extra sigaling overhead, e.g. reporting when entering into/exit ultra-deep sleep, and power consumption.

But we think RAN2 study should figure out some solutions with pros/cons on whether network is aware of UE’s MR power state, i.e. in ultra-deep sleep or not.

Proposal 2: For UE in RRC_IDLE/RRC_INACTIVE state, RAN2 to discuss whether the network is aware of the UE’s MR power state, i.e. in ultra-deep sleep or not.

  - Entry/Exit conditions for ultra-deep sleep

As discussed above, with LR, UE’s MR could enter into the ultra-deep sleep power state for further power saving. Here we further discuss the corresponding conditions for MR entry/exit for ultra-deep sleep.

2.4.1 Entry conditions

As we have discussed in section 2.3, whether network is aware of the UE’s MR power state should be further studied. However, in our view, it should be up to the network to control whether UE’s MR could enter into ultra-deep sleep power state or not. Otherwise, it will be hard for network to determine whether to send LP-WUS or not for a UE. In this way, the following two network-controlled alternatives could be considered:

- Alt 1: Network indication, i.e., UE’s MR enters ultra-deep sleep state upon receiving the network indication.

- Alt 2: Pre-conditions configured by network, i.e. UE’s MR automatically enters into ultra-deep sleep state when UE fulfills the conditions configured by network.

There are a lot of issues related to the above alternatives that need to be further studied, for example, whether dedicated signaling or common signaling should be used in alternative 1 and what conditions should be defined in alternative 2. In our view, for alternative 2, similar conditions as existing conditions defined for power saving purposes could be re-used, e.g. low mobility, stationary and not at cell edge criterion introduced in Rel-16/Rel-17 RRM relaxation. Anyway, the details could be discussed in later meeting after we make some progress.

Proposal 3: whether UE’s MR could enter into ultra-deep sleep state should be controlled by network, following examples could be considered, details are FFS.

- Alt 1: via Network indication, i.e., UE’s MR enters into ultra-deep sleep power state upon receiving the network indication.

- Alt 2: UE’s MR automatically enters into ultra-deep sleep power state when pre-configured condition(s) are fulfilled.

2.4.2 Exit conditions

UE’s MR won’t perform the paging monitoring when it enters into ultra-deep sleep state. Hence, it should be ensured that UE could detect LP-WUS via LR in this case since UE uses LP-WUS for paging reception according to the following RAN2 agreements. Otherwise, UE will miss the paging reception and couldn’t work normally.

In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

Observation 2: It should be ensured that UE could detect LP-WUS when UE’s MR enters into ultra-deep sleep state.

It is agreed that when LP-WUS coverage is insufficient, UE’s MR needs to fall back to the legacy operation according to the following RAN1 agreements in RAN1#112bis meeting.

- Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

Observation 3: RAN1 has agreed Main Radio switches to legacy operation in case UE is out of the LP-WUS coverage.

Based on the RAN1’s agreements, when UE moves out of the coverage of LP-WUS, UE’s MR should return back to normal state to monitor paging (or even PEI) as legacy, that is to say, UE’s MR exits the ultra-deep sleep state. On the other hand, when UE is in the coverage of LP-WUS, it could decode LP-WUS via LR, in this case, the paging notification won’t be missed, hence UE’s MR could continue to stay in ultra-deep sleep.

It could be observed that the coverage of LP-WUS is a region in which the quality of received LP-WUS, e.g. the RSRP, RSRQ, SINR, or other metrics (detail is up to RAN1) of LP-WUS is strong enough for UE to process/decode LP-WUS correctly. According to RAN1 progress, UE performs the RRM measurement by LR based on the signals detected by LR and may also perform the RRM measurement by MR when UE’s LR turns on in RRC_Idle/inactive mode, it is proposed that UE determines the coverage of LP-WUS via measurement results performed by LR and/or MR.

Observation 4: RAN1 is studying the RRM measurement performed by LR and it is FFS on whether/how to perform the RRM measurement by MR.

Proposal 4: UE’s MR could stay in ultra-deep sleep power state only when UE is in the coverage of LP-WUS. UE’s MR should exit ultra-deep sleep power state when UE moves out of coverage of LP-WUS. The determination of whether UE is in coverage or out of coverage of LP-WUS could be based on the measurements performed by LR and/or MR, details are FFS.

As discussed above, during ultra-deep sleep state, the LR will monitor LP-WUS. If the UE receives LP-WUS via LR which indicates UE to wake up, UE’s MR should also exit the ultra-deep sleep state to perform legacy RRC_IDLE/RRC_INACTIVE behaviour, e.g. paging monitoring, SI reception, etc.

Proposal 5: UE’s MR should exit ultra-deep sleep power state when UE receives LP-WUS which indicates the UE to wake up.

Besides, RAN2 could also study other scenarios to wake up UE’s MR from ultra-deep sleep state based on companies’ contributions.

  - Paging monitoring

Paging monitoring for power consumption and latency reduction

For legacy UEs in RRC_Idle/RRC_Inactive state, UE should periodically wake up to perform paging monitoring with DRX. In addition, UE may wake up every paging cycle(s) to perform measurements/sync according to the defined requirements. These two behaviors contribute most of the power consumption for RRC_Idle/RRC_Inactive UEs.

Observation 5: For legacy UE in RRC_Idle/RRC_Inactive state, paging monitoring and measurement contribute most of the power consumption.

To reduce the power consumption of paging monitoring, PEI (Paging Early Indication) was introduced in Rel-17. One PEI is associated with one PO, which is used to indicate UEs whether need to monitor the following PO. A UE supporting PEI can monitor PEI associated with its PO. If no PEI is detected, the UE can skip the following PO monitoring. In this way, the power consumption of paging monitoring can be reduced by PEI.

However, the PEI solution requires the MR to monitor PEI, which implies the MR cannot sleep for a duration longer than one DRX cycle. It could be micro/light/deep sleep according to the DRX cycle length. If LP-WUS is introduced, the LR can be applied to monitor the LP-WUS. Using LR to perform LP-WUS monitoring allows the MR to ultra-deep sleep for a duration longer than one DRX cycle. Besides, the power consumption of LR is expected to be significantly low. Thus, LP-WUS-based solution can consume much less power than legacy.

Observation 6: For paging monitoring, LP-WUS-based solution can consume much less power than legacy.

As we known, eDRX is also another solution to reduce the power consumption of RRC_Idle/RRC_Inactive UE. With eDRX, the UE is required to monitor paging only in a PTW window for one eDRX cycle, i.e. the MR is allowed to sleep out of the PTW window. The drawback of the eDRX solution is long paging latency according to the eDRX cycle, which limits its applicability and is usually applied to devices with delay tolerant services, such as IoT, but is not suitable for the eMBB and IIoT use cases. In comparison, using LP-WUS could extend the sleep duration of the MR without increasing the paging latency.

Observation 7: Compared with eDRX, using LP-WUS could extend the sleep duration of the MR without increasing the paging latency.

To compare the power saving effects of various power saving solutions, RAN1 is evaluating the power evaluation for all legacy solutions. Here, we also provide some evaluation results on power saving gain in RRC_IDLE and RRC_INACTIVE state with LP-WUS in Annex, more detailed simulation assumptions and results could be found in [3]. It could be observed that the LP-WUS could achieve substantial power saving gain for RRC_IDLE and RRC_INACTIVE state in various scenarios.

Taking the above observations into account, we suggest:

Proposal 6: RAN2 to study LP-WUS applicable for RRC_Idle/RRC_Inactive state to reduce power consumption and latency of paging monitoring.

Subgrouping for LP-WUS

As justified in [1], if UE’s MR wakes up only when it needs to be triggered, e.g., for paging addressed to this UE, power consumption could be dramatically reduced. That is to say, LP-WUS FAR (False Alarm Rate) is also critical to UE power consumption. The LP-WUS FAR could be caused by channel condition or insufficient paging grouping. The former is to be addressed by RAN1, which is being discussed. The latter means the detection of an LP-WUS which is intended to wake up/alarm the LR of another UE within the same UE group. It is obvious that high FAR caused by insufficient grouping paging deteriorates UE power consumption as well.

Observation 8: High FAR caused by insufficient paging grouping deteriorates UE power consumption.

To address the high FAR issue caused by insufficient paging grouping, subgrouping for LP-WUS could be considered, similar as the paging subgrouping introduced in Rel-17. In Rel-17, paging subgrouping via PEI was introduced, where UEs monitoring paging on the same PO are divided into maximum 8 subgroups. In order to obtain more power saving gain, it is better to introduce subgrouping for LP-WUS. Especially in case there is no association between LP-WUS and PO, e.g. LP-WUS is common for all UE (i.e. no UE grouping based on the transmission time of LP-WUS), paging grouping is more beneficial for LP-WUS. Hence, we propose that:

Proposal 7: RAN2 to study introducing subgrouping for LP-WUS to reduce false alarms rate.

How to include subgrouping information in LP-WUS is an open issue needs further study. LP-WUS may include sync signal /sequence, payload and CRC as discussed in RAN1. The structure for LP-WUS is being discussed in RAN1, which highly depends on how many information bits can be conveyed. For example, sequence-based LP-WUS can be considered if limited information bits are conveyed, while control information payload may be required if more bits would be conveyed.

Subgrouping information could be conveyed via LP-WUS sequence or payload. More bits for subgrouping information, less paging FAR. In PEI solution in Rel-17, up to 8 subgroups can be supported within one PO, i.e. 3 bits are used for subgrouping. Considering 10-bit UE ID is already used to determine the time location of PO, 13 bits in total are used for legacy paging subgrouping in PEI. If a LP-WUS can be monitored by all UEs in one cell, at least 13-bit paging subgrouping information is required in order to achieve similar subgrouping granularity as legacy. We may further investigate how to convey this paging subgrouping information by LP-WUS sequence and/or payload. However, this relies on the LP-WUS signaling design by RAN1. It would be better to send an LS to RAN1 to raise this paging subgrouping requirement, and check whether it is feasible to include paging subgrouping information in LP-WUS, if yes, how many bits could be used for this purpose.

Proposal 8: Send an LS to RAN1 to check whether it is possible to include paging subgrouping information in LP-WUS, if yes, how many bits could be used for this purpose.

  - SI change and ETWS/CMAS notification

In legacy, the UE in RRC_IDLE/RRC_INACTIVE shall ensure having a valid (latest) version of system information of the current serving cell. The UE shall monitor SI change indication in its own paging occasion every DRX cycle in RRC_IDLE/RRC_INACTIVE state and the UE needs to acquire the updated SI upon receiving SI change indication. Moreover, UE needs to acquire the SI of the serving cell when UE performs cell selection/reselection.

In case UE’s MR enters into ultra-deep sleep state, it should be studied whether the UE needs to maintain valid SI during ultra-deep sleep, including the scenarios of SI change or cell selection/reselection. The following options could be considered:

- Option 1: UE maintains valid SI during ultra-deep sleep.

- Option 2: UE doesn’t maintain valid SI during ultra-deep sleep.

Option 1 could achieve less paging latency with some power consumption since the MR may need to wake up to receive the updated SI, while option 2 has more power saving gain while the paging latency may be longer since the UE needs to receive SI after waking up before paging reception or initiating the RRC connection setup.

Proposal 9: RAN2 to discuss whether to maintain valid SI in case UE’s MR is in ultra-deep sleep state.

If maintaining valid SI in case UE’s MR is in ultra-deep sleep state, one issue is how to notify the SI change for the UE in ultra-deep sleep state. The similar issue may also exist for ETWS/CMAS. ETWS/CMAS is regulatory requirement for UEs supporting ETWS/CMAS. Therefore, even for UE in ultra-deep sleep, the ETWS/CMAS massages should be ensured to be reachable if the UE is capable of ETWS/CMAS.

Proposal 10: ETWS/CMAS should be supported by UE with LP-WUR, if capable, i.e. UEs in ultra-deep sleep are reachable for ETWS/CMAS notification.

In order to achieve the reachability, there could be two methods to notify the SI change or ETWS/CMAS for the UEs in ultra-deep sleep:

- Method 1: based on legacy indication in short message/paging, i.e. waking UE up by LP-WUS, and receiving the notification of SI change or ETWS/CMAS as in legacy.

- Method 2: introduce direct notification indicator(s) in LP-WUS signal, i.e. waking the UE up to receive updated SI, or ETWS/CMAS directly.

With Method 1, after receiving LP-WUS signal, the UE’s MR wakes up to monitor PDCCH in its PO. After receiving the ETWS/CMAS notification/SI change in short/paging message, the UE will receive ETWS/CMAS information and updated SI.

With Method 2, the UE can immediately know via the notify indicator(s) that ETWS/CMAS or SI change occurs.

Method 1 could save the overhead for the payload of WUS signal. Method 2 could reduce the latency for reception, especially for ETWS/CMAS.

Proposal 11: RAN2 to discuss how to indicate the notification of ETWS/CMAS or SI change (if valid SI needs to be maintained) for UE in ultra-deep sleep, including: by legacy indication or direct notification indicator(s) in LP-WUS.

  - RRM and Mobility for UE with LR

In RAN1#111 meeting, the following agreements are reached:

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

RAN1 is studying the feasibility of RRM measurements performed by LR, while whether/how the measurement performed by MR is FFS. Since UE supporting LR may be a mobile UE, the UE needs to continuously perform measurement at least to decide whether it is in the coverage of LP-WUS. Otherwise, the LP-WUS signal couldn’t be detected if the UE moves out of the coverage of LP-WUS. In this case, the UE becomes unreachable, which is undesirable from mobility perspective. Thus, RAN2 should study the measurement based on LR at least for serving cell.

If the measurement is mainly performed by MR, the power saving gain will be very limited because the MR will anyway need to wake up periodically to perform measurement. Therefore, the measurement based on LR for serving cell should be considered for power saving purpose. In RAN1#112 meeting, RAN1 discussed synchronisation signal, and reached the following agreements:

Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

In our understanding, the measurement performed by LR can be based on the above synchronisation signal, at least for serving cell measurement.

Another issue is whether LR needs to measure neighbour cells, e.g., for the purpose of cell selection/reselection. It may depend on RAN1’s progress, e.g., synchronisation signal design, whether the accuracy for the measurement performed by LR is enough, and whether LR can simultaneously measure synchronisation signals of multiple cells/frequencies (separately corresponding to serving cell and neighbour cells) or not. If it is feasible that cell selection/reselection could be based on the measurement by LR, UE’s MR doesn’t need to wake up periodically for measurement. Otherwise, the MR may need to wake up and perform the measurement as legacy in some cases, e.g., the measurement result of serving cell from LR is not enough.

Proposal 12: Measurement based on LR at least for serving cell is supported. FFS for neighbour cell and corresponding cell selection/reselection.

For UEs in ultra-deep sleep power state, the reachability should be guaranteed. It implies that idle/inactive UEs in ultra-deep sleep needs to identify whether moving out of registration area (TA list) and inactive UE in ultra-deep sleep needs to identify whether to move out of RNA, otherwise the UE will be unreachable. Therefore, the UE needs to obtain the cell information, tracking area information and RAN area information during ultra deep sleep.

In partial coverage as mentioned above, if the coverage of LP-WUS is always smaller than the coverage of the cell’s SSB, UE always needs to wake up whenever moving cross the cell’s edge. In this case, we assume the UE’s MR will be used to obtain the cell information, tracking area information and RAN area information of the new serving cell via system information acquisition after cell reselection/selection. Hence, the cell information, tracking area information, and RAN area information are not necessary to be included in LP-WUS.

If this is not the case, i.e. in full coverage of LP-WUS, the cell information, tracking area information, and RAN area information may be indicated in LP-WUS if measurement based on LR is used for cell reselection is agreeable, which needs further discussion.

Proposal 13: Cell information, tracking area information and RAN area information indicated in LP-SS/LP-WUS are not needed for the scenario with partial coverage of LP-WUS.  FFS for full coverage of LP-WUS.

- Conclusion

In this contribution, we discuss the LP-WUS coverage, network awareness of ultra-deep sleep, entry and exit conditions for ultra-deep sleep state, paging enhancements, SI change notification and RRM and mobility aspects with LP-WUR considering RAN1’s progress.Based on the discussion, we have the following observations and proposals.

General

Observation 1: The power consumption for MR will be significantly reduced when MR is in ultra-deep sleep.

LP-WUS coverage

Proposal 1: RAN2 needs to study mechanisms to support both full and partial coverage of LP-WUS/SS.

network awareness of ultra-deep sleep

Proposal 2: For UE in RRC_IDLE/RRC_INACTIVE state, RAN2 to discuss whether the network is aware of the UE’s MR power state, i.e. in ultra-deep sleep or not.

entry and exit conditions for ultra-deep sleep state

Observation 3: RAN1 has agreed Main Radio switches to legacy operation in case UE is out of the LP-WUS coverage.

Observation 4: RAN1 is studying the RRM measurement performed by LR and it is FFS on whether/how to perform the RRM measurement by MR.

Proposal 3: whether UE’s MR could enter into ultra-deep sleep state should be controlled by network, following examples could be considered, details are FFS.

- Alt 1: via Network indication, i.e., UE’s MR enters into ultra-deep sleep power state upon receiving the network indication.

- Alt 2: UE’s MR automatically enters into ultra-deep sleep power state when pre-configured condition(s) are fulfilled.

Proposal 4: UE’s MR could stay in ultra-deep sleep power state only when UE is in the coverage of LP-WUS. UE’s MR should exit ultra-deep sleep power state when UE moves out of coverage of LP-WUS. The determination of whether UE is in coverage or out of coverage of LP-WUS could be based on the measurements performed by LR and/or MR, details are FFS.

Proposal 5: UE’s MR should exit ultra-deep sleep power state when UE receives LP-WUS which indicates the UE to wake up.

paging enhancements

Observation 5: For legacy UE in RRC_Idle/RRC_Inactive state, paging monitoring and measurement contribute most of the power consumption.

Observation 6: For paging monitoring, LP-WUS-based solution can consume much less power than legacy.

Observation 7: Compared with eDRX, using LP-WUS could extend the sleep duration of the MR without increasing the paging latency.

Observation 8: High FAR caused by insufficient paging grouping deteriorates UE power consumption.

Proposal 6: RAN2 to study LP-WUS applicable for RRC_Idle/RRC_Inactive state to reduce power consumption and latency of paging monitoring.

Proposal 7: RAN2 to study introducing subgrouping for LP-WUS to reduce false alarms rate.

Proposal 8: Send an LS to RAN1 to check whether it is possible to include paging subgrouping information in LP-WUS, if yes, how many bits could be used for this purpose.

SI change notification

Proposal 9: RAN2 to discuss whether to maintain valid SI in case UE’s MR is in ultra-deep sleep state.

Proposal 10: ETWS/CMAS should be supported by UE with LP-WUR, if capable, i.e. UEs in ultra-deep sleep are reachable for ETWS/CMAS notification.

Proposal 11: RAN2 to discuss how to indicate the notification of ETWS/CMAS or SI change (if valid SI needs to be maintained) for UE in ultra-deep sleep, including: by legacy indication or direct notification indicator(s) in LP-WUS.

RRM and mobility aspects with LP-WUR

Proposal 12: Measurement based on LR at least for serving cell is supported. FFS for neighbour cell and corresponding cell selection/reselection.

Proposal 13: Cell information, tracking area information and RAN area information indicated in LP-SS/LP-WUS are not needed for the scenario with partial coverage of LP-WUS.  FFS for full coverage of LP-WUS.

References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

- Report of 3GPP TSG RAN WG2 meeting #121bis-e

- R1-2302506, Evaluation methodologies for R18 LP-WUS/WUR, vivo.

Annex – Simulation results in RRC_idle/inactive

We provide the power evaluation results considering the following aspects which will do impact on power consumption and latency performance.

- Baselines: PO monitoring with i-DRX, e-DRX, with or without PEI

- Assumed MR sleep state during LP-WUS monitoring: deep sleep or ultra-deep sleep

- The relative power of LP-WUR “ON” state: [0.01/0.05/0.1/0.5/1/2/4/20/40] units

- MR Ramp-up time: [400ms] and Ramp-up and down transition energy: [15000]

- Sync/re-sync time X: X= [60, 100ms] and sync/re-sync energy: [2180, 3340]

- RRM:

- For baselines: option 1: RRM measurement is only performed by MR.

- For LP-WUS scheme:

Option 2: MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And LP-WUR performs assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

Option 3: RRM measurement is totally performed by LP-WUR.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Note: Adopt Alt-2 of FAR target which is determined across a reference time duration (e.g., 1.28s) of one or multiple WUS attempts/trials.

- Per UE paging rate i.e., RE=RE, REF= [1%, 0.1%, 0.01% or 0.001%], Y=YREF = 1.28s

- Per group paging rate i.e., RG=1 – (1 – RE)N , N is the number of UEs in the group: [10, 20]

- LP-WUS monitoring:

  - Option 1: continuously monitoring;

  - Option 2: discontinuously monitoring

- [200ms, 600ms, 1000ms] as the period for an on-and-off cycle, and for each cycle the duty cycle ratio is [0.1%, 2%, 10%, 40%].

- LP-WUR on-off transition time: 10ms.

Comparing i-DRX, e-DRX (with or without PEI) with LP-WUS scheme

Figure 2 shows the simulation results with different baseline schemes under the configurations in Table 5.

Table 5. Configurations for evaluation with different baselines.

| Parameters | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous | 0.001% | Ultra-deep sleep | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 2.  Initial simulation results of power consumption and latency for different schemes

According to the results, the following observations are given:

Observation 1: Compared with I-DRX paging, LP-WUR/WUS scheme with continuously monitoring configuration can achieve around 50%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, with marginal latency increase.

Observation 2: Compared with eDRX, LP-WUR/WUS scheme with continuously monitoring configuration can largely reduce the paging latency (23x), with comparable UE power consumption.

Observation 3:  LP-WUR/WUS scheme provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with I-DRX paging and eDRX scheme.