3GPP TSG-RAN WG2 Meeting #122		R2-2304924

Incheon, Korea, 22nd – 26th May 2023

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_Connected

Agenda Item:	7.22.3

Document for:	Discussion and Decision

- Introduction

In RAN #97e meeting, the SID for R18 low-power Wake-up Signal and Receiver for NR was agreed upon [1], and in RAN2#121 bis meeting, we have the following relevant agreements on LP-WUS and LP-WUR [2]:

- Confirm that we follow R1 and include RRC idle/inactive/connected.

Besides, RAN1 has the following agreements on RRC_Connected for LP-WUS in RAN1#112bis meeting [3]:

- For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different In RRC connected, study the relationship between LP-WUS and legacy UE power saving

In this contribution, we will discuss the detailed RRC_CONNECTED procedures with LP-WUS/WUR considering RAN1’s progress.

- Discussion

  - General

According to the SID, UE supporting LP-WUS/WUR will be equipped with an additional Rx module apart from the current Tx/Rx module. The corresponding terminologies were agreed by RAN1 as follows:

Use the following terminology for future discussion, - Main radio (MR): the Tx/Rx module operating for legacy NR signals/channels apart from signals/channel related to low-power wake-up. - LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

For legacy UE in RRC_Connected state, it should keep MR monitoring PDCCH in C-DRX active time. The PDCCH monitoring of MR is unnecessary if there is no UE schedule. However, the legacy UE can confirm that a PDCCH monitoring is unnecessary only after already monitored the PDCCH with MR and found that no PDCCH address to itself. As a result, the power consumption caused by unnecessary PDCCH monitoring was wasted.

This issue has been also discussed in Rel-16/Rel-17. Finally, DCP in Rel-16 and PDCCH monitoring adaptation in Rel-17 were introduced to reduce unnecessary PDCCH monitoring. They are both PDCCH based signaling. For a UE equipped with LR, the MR can be in sleep power state, while the LR remains active to monitor Low-Power Wake-Up Signal (LP-WUS) in DRX or continuous manner. When LP-WUS is received by LR, it will trigger the MR to wake up to monitor PDCCH. As LR would adopt a minimalistic design, the power consumption of LR is expected to be significantly lower than legacy PDCCH based signaling in some cases.  Anyway, this is related to the LR receiver architecture, which is being discussed in RAN1.

According to the evaluation results in Annex, it could be observed that more power saving gain is obtained via LP-WUS than the existing power saving schemes. Compared with UE in RRC_Idle/Inactive state, UE in RRC_Connected state is more sensitive to the latency. Therefore, we suggest:

Proposal 1: RAN2 to study LP-WUS procedures for RRC_Connected state to reduce power consumption while guarantee the latency performance, e.g. with unnecessary PDCCH monitoring reduction.

According to the following agreement in RAN1#112bis meeting, the ultra-deep sleep state is not allowed for MR for RRC connected mode.

- For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR.

Observation 1: Ultra-deep sleep power state is not allowed for MR for RRC_Connected state.

  - PDCCH monitoring reduction

In RAN1#110bis meeting, it agreed with the following models for R18 LP-WUS/WUR power evaluation in RRC_Connected state.

Agreement For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered, - XR traffic model with evaluation methodologies and assumptions captured in TR 38.838. - eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840 - Heartbeat traffic models in 3GPP TR 38.875. - Other models are not precluded.

In our view, LP-WUS is well suited for XR service which has stringent requirements on both power consumption and latency. Since the XR data packets have a wide range of unpredictable jitter, UE has to perform intensive PDCCH monitoring in the entire jitter range to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB even it is configured with the existing R15/16/17 power saving schemes (e.g the DCP or PDCCH skipping).

Observation 2: LP-WUS is well suited for service which has stringent requirements on both power consumption and latency, e.g. XR.

Besides, RAN1 has agreed to study the relationship between LP-WUS and legacy UE power saving in RRC connected. Regarding the LP-WUS combined with C-DRX, the following alternatives could be considered:

- Alt 1: UE performs LP-WUS monitoring from the beginning of DRX on duration

- Alt 2: UE performs LP-WUS monitoring when UE is in DRX off

The Alt 1 is useful for XR, for example, as shown in Figure 1, C-DRX is adopted. To further reduce the unnecessary PDCCH monitoring during the jitter range, UE could firstly monitor LP-WUS by LR when DRX onduration timer is started, and after LR detects LP-WUS which indicated the PDCCH monitoring, MR is waken up to monitor PDCCH. In this way, the unnecessary PDCCH monitoring during the jitter range could be saved.

Moreover, to guarantee the latency requirement of XR traffic, main radio needs to enter micro or light sleep state rather than the deep sleep state during LP-WUS monitoring.

Figure 1.  Working procedure for LP-WUS/WUR based jitter handling

Observation 3: LP-WUS used within DRX on duration could help reduce power consumption for XR service without additional latency.

For Alt2, UE performs LP-WUS monitoring when UE is in DRX off, and MAC entity enters into Active Time for PDCCH monitor, it is useful for eMBB use case e.g., FTP3, since the long latency caused by DRX off applied can be reduced by LP-WUS monitoring.

Observation 4: LP-WUS used within DRX off could help reduce the latency for eMBB use case.

Regarding the LP-WUS combined with PDCCH skipping, for example, the UE starts monitoring LP-WUS when it applies the PDCCH skipping. During the PDCCH skipping period, when LP-WUS is detected, the UE resumes PDCCH monitoring until the next PDCCH skipping command is received. In this way, UE could start and stop PDCCH monitoring quickly based on NW indication, which could reduce the additional latency in C-DRX.

Observation 5: LP-WUS combined with PDCCH skipping could reduce power consumption and latency than current C-DRX.

According to our simulation (details can be found in Annex), compared to existing R15/16/17 power saving schemes, LP-WUS monitoring combined with MR micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases. And LP-WUS monitoring combined with MR light sleep can bring {10%~22%} additional power saving gain, with acceptable capacity loss at least in low load case. Moreover, for eMBB services with latency requirements less stringent than XR (e.g. FTP), MR can be in deep sleep power state to obtain more power saving gain.

Observation 6: LP-WUS could bring significant power saving gain for RRC_CONNECTED mode according to the simulation.

Therefore, we propose:

Proposal 2: UE monitors LP-WUS by LR to determine whether to trigger UE’s main radio to wake up from micro/light/deep sleep to perform PDCCH monitoring.

Proposal 3: RAN2 to study the procedures of LP-WUS monitoring interacting with the existing UE power saving mechanisms for RRC connected mode, e.g. C-DRX, R16 DCP, R17 PDCCH skipping, and R17 SSSG switching.

  - RRM measurement

In RAN1#121 bis meeting, it was agreed that RRM/RLM/BFD/CSI are performed by MR in RRC connected mode, and RAN1 will study whether additional support of RRM measurement by LP-WUR for RRC connected mode.

- For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode

In current specification, the RRM measurement mainly intends to the mobility, i.e. ensure UE works in a serving cell with good quality via handover. For UEs with LP-WUR, UE’s MR is in micro or light sleep power state and could perform RRM measurement normally, which is same as current DRX off state, hence the RRM measurement by MR is enough for mobility purpose. Regarding whether additional support of RRM measurement by LP-WUR is needed, it could be further studied if suitable use cases are discovered based on companies’ contributions.

Proposal 4: Current RRM measurement by Main Radio is enough for mobility in RRC connected mode, FFS additional support of RRM measurement by LP-WUR.

- Conclusion

In this contribution, we discuss the RRC_CONNECTED procedures with LP-WUS/WUR considering RAN1’s progress. Based on the discussion, we have the following observation and  proposals:

Observation 1: Ultra-deep sleep power state is not allowed for MR for RRC_Connected state.

Observation 2: LP-WUS is well suited for service which has stringent requirements on both power consumption and latency, e.g. XR.

Observation 3: LP-WUS used within DRX on duration could help reduce power consumption for XR service without additional latency.

Observation 4: LP-WUS used within DRX off could help reduce the latency for eMBB use case.

Observation 5: LP-WUS combined with PDCCH skipping could reduce power consumption and latency than current C-DRX.

Observation 6: LP-WUS could bring significant power saving gain for RRC_CONNECTED mode according to the simulation.

Proposal 1: RAN2 to study LP-WUS procedures for RRC_Connected state to reduce power consumption while guarantee the latency performance, e.g. with unnecessary PDCCH monitoring reduction.

Proposal 2: UE monitors LP-WUS by LR to determine whether to trigger UE’s main radio to wake up from micro/light/deep sleep to perform PDCCH monitoring.

Proposal 3: RAN2 to study the procedures of LP-WUS monitoring interacting with the existing UE power saving mechanisms for RRC connected mode, e.g. C-DRX, R16 DCP, R17 PDCCH skipping, and R17 SSSG switching.

Proposal 4: Current RRM measurement by Main Radio is enough for mobility in RRC connected mode, FFS additional support of RRM measurement by LP-WUR.

- References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR.

- Report of 3GPP TSG RAN WG2 meeting #121bis-e

- Chairman’s Notes of RAN1 #112bis.

- Annex

  - Power evaluation results for RRC_Connected state

In the following evaluation for XR traffic, performance metrics including power consumption, latency, capacity are considered. The detail evaluation methodologies and assumptions can be found in TR 38.838.

- Evaluation for XR traffic model (refer to TR38.838 and the agreed jitter model in R18 XR SI)

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. And the detailed configurations can be founded in section5.2. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for low load is given, corresponding to 5 UEs per cell.

Figure 2.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 2, for low load case, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {6%~13%} additional UE power saving gain with no capacity loss. In addition, when main radio enters light sleep during LP-WUS monitoring, the LP-WUS scheme can bring {10%~22%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss, comparing to R17 PDCCH monitoring adaptation scheme.

Observation 1: Compared to the existing R17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~13%} additional UE power saving gain with no capacity loss in low load case.

Observation 2: Compared to the existing R17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss in low load case.

  - Simulation assumptions for RRC_Connected state

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 1. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 2. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |