3GPP TSG-RAN WG2 Meeting #122                                                          R2-2305284

Incheon, KR, 22nd – 26th May, 2023

Source:              CATT, GOHIGH

Title:	             Discussion on MCSt

Agenda Item:	7.15.3

Document for:	Discussion and Decision

Introduction

In the last RAN1 meeting, RAN1 has decided to send LS on MCSt to RAN2 [1]. In the LS, three approaches to implement MCSt for multiple TBs were listed. In this contribution, we will discuss RAN1’s questions in the LS from RAN2 perspective.

Discussion

RAN2 background on resource (re-)selection

According to the description in subclause 5.22.1.1 of TS38.321 [1], the sidelink grant can be created for a single MAC PDU or multiple MAC PDUs:

Case 1: Transmission for Multiple MAC PDUs

If the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel, the following steps will be performed:

- Select a resource pool;

- Select the SL resource reservation period and set it as the value of Prsvp_TX;

- Select the HARQ retransmission number and the amount of frequency resources;

- Performs resource selection for one transmission opportunity according to the following descriptions in [2]. Use the selected resource and the resource reservation period to determine the the initial transmission opportunities for multiple MAC PDUs.

4> if transmission based on random selection is configured by upper layers: 5> randomly select the time and frequency resources for one transmission opportunity from the resource pool which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> else: 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

- Selected retransmission resource for each MAC PDUs as below:

5> if transmission based on full sensing or partial sensing is configured by upper layers and there are available resources left in the resources indicated by the physical layer according to clause 8.1.4 of TS 38.214 [7] for more transmission opportunities; or 5> if transmission based on random selection is configured by upper layers and there are available resources left in the resource pool for more transmission opportunities: 6> randomly select the time and frequency resources for one or more transmission opportunities from the available resources which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier by ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources and that a retransmission resource can be indicated by the time resource assignment of a prior SCI according to clause 8.3.1.1 of TS 38.212 [9].

Case 2: Transmission for a single MAC PDU

If the MAC entity has selected to create a selected sidelink grant corresponding to transmission(s) of a single MAC PDU, and if SL data is available in a logical channel, or an SL-CSI reporting is triggered, or a Sidelink DRX Command indication is triggered or a Sidelink Inter-UE Coordination Information reporting is triggered, or a Sidelink Inter-UE Coordination Request is triggered, the following steps will be performed:

- Select a resource pool;

- Select the HARQ retransmission number and the amount of frequency resources;

- Performs resource selection according to the following descriptions in [2]:

4> if transmission based on random selection is configured by upper layers: 5> randomly select the time and frequency resources for one transmission opportunity from the resources pool which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and the latency requirement of the triggered SL CSI reporting. 4> else: 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX Active if configured time as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting.

Based on the above descriptions, it is obvious that:

- For a single MAC PDU transmission, MAC only created SL grant for initial transmission and retransmission(s) of the same TB. And the resource allocated for the initial transmission and retransmission may be not consecutive.

- For multiple MAC PDU transmissions, MAC can create SL grant for initial transmission and retransmission(s) of multiple TBs, but the time interval of the initial transmission of each TB equals to the SL configured grant periodicity.

Observation 1: The current MAC resource allocation procedure cannot ensure the MCSt of both a single TB and multiple TB(s).

Observation 2: The current MAC resource allocation framework is aiming to allocate resource for each sidelink process, e.g., one TB (the single MAC PDU transmission case) or for multiple TBs arriving periodically (the multiple MAC PDUs transmission case).

Discussion on the MCSt approaches suggested by RAN1

Regarding to MCSt, RAN1 provided three approaches in [2], listed below:

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

In the following, we will analyze the above approaches one by one:

Approach 1

For step 1~3 of Approach 1, if random resource selection is performed, according to legacy MAC procedure, it cannot ensure the MCSt of a single MAC PDU; and if sensing based resource selection is performed, whether MCSt can be guaranteed depends on whether consecutive candidate resource(s) is provided or not in Step 2.

For step 4, considering the resource selection procedure for each TB is independent in MAC, it cannot guarantee the MCSt of multiple TBs.

Approach 2

For step 1~3 of Approach 2, compared with Approach 1, considering the candidate resource provided by PHY is consecutive, hence, if MAC performs resource allocation based on the candidate resource, at least the MCSt of a single TB can be guaranteed.

For step 4, considering the resource selection procedure for each TB is independent in MAC, it also cannot guarantee the MCSt of multiple TBs.

Approach 3

The most challenge is step 3. According to the observation 2, the current MAC resource allocation framework is aiming to allocate resource for one TB (the single MAC PDU transmission case) or for multiple TBs arriving periodically (the multiple MAC PDUs transmission case). It does not support resource allocation for multiple TBs simultaneously unless the MAC resource allocation framework is changed.

Proposal 1: Send LS reply to RAN1 to clarify that from RAN2 perspective, Approach 2 is more attractive compare with Approach 1 and Approach 3 and only MCSt of a single MAC PDU is suggested to be considered.

In addition, in RAN1 LS in [2], there are three questions:

Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs)

Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time

Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

For Question 1, if the current MAC resource allocation framework is not changed, the concatenating across separate resource selection triggers is not feasible.

For Question 2, similarly, if the current MAC resource allocation framework is not changed, the feasibility of triggering the resource selection procedures for multiple SL processes at the same time is not allowed.

For Question 3, regarding to Approach 2, from RAN2 perspective, if only the MCSt of a single TB is considered, MAC can provided the number of slots based on the minimum of {number of HARQ transmissions (including initial and possible retransmission(s)), COT}. But if MCSt of multiple TBs needs to be considered, it is hard for RAN2 to decide the number of slots.  Regarding to Approach 3, since MCSt of multiple TBs needs to be considered, similarly, it is hard for RAN2 to decide the number of slots.

Proposal 2: For Question 1, send LS reply to RAN1 to clarify that it is infeasible to concatenate separate resource selection triggers for Approach 1 and Approach 2 from RAN2 perspective.

Proposal 3: For Question 2, send LS reply to RAN1 to clarify that it is infeasible to trigger the resource selection procedures for multiple SL processes as indicated in Approach 3 from RAN2 perspective.

Proposal 4: For Question 3, send LS reply to RAN1 to clarify if MCSt of a single TB is considered, it is feasible for MAC to provide the new parameter “number of slots for MCSt”, but MCSt of multiple TBs is considered, it is hard for MAC to provide the new parameter “number of slots for MCSt”.

Conclusion

According to the analysis in section 2, it is observed:

Observation 1: The current MAC resource allocation procedure cannot ensure the MCSt of both a single TB and multiple TB(s).

Observation 2: The current MAC resource allocation framework is aiming to allocate resource for each sidelink process, e.g., one TB (the single MAC PDU transmission case) or for multiple TBs arriving periodically (the multiple MAC PDUs transmission case).

And it is also proposed:

Proposal 1: Send LS reply to RAN1 to clarify that from RAN2 perspective, Approach 2 is more attractive compare with Approach 1 and Approach 3 and only MCSt of a single MAC PDU is suggested to be considered.

Proposal 2: For Question 1, send LS reply to RAN1 to clarify that it is infeasible to concatenate separate resource selection triggers for Approach 1 and Approach 2 from RAN2 perspective.

Proposal 3: For Question 2, send LS reply to RAN1 to clarify that it is infeasible to trigger the resource selection procedures for multiple SL processes as indicated in Approach 3 from RAN2 perspective.

Proposal 4: For Question 3, send LS reply to RAN1 to clarify if MCSt of a single TB is considered, it is feasible for MAC to provide the new parameter “number of slots for MCSt”, but MCSt of multiple TBs is considered, it is hard for MAC to provide the new parameter “number of slots for MCSt”.

Reference

- TS38.321  Medium Access Control (MAC) protocol specification

- R1-2304257   LS on MCSt resource (re-)selection   RAN1