3GPP TSG-RAN WG2 #122	R2-2305548

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:	7.9.2

Source:	Ericsson

Title:	Control Plane Procedures for Layer 2 UE-to-UE Relays

Document for:	Discussion, Decision

# 1	Introduction

The following agreements were made in the RAN#121bis-e meeting on the CP aspects for U2U relays:

Agreements:

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

The following proposals were not discussed due to time constraints:

Proposals:

Proposal 5a: Option 1 (Target remote UE ID (layer-2 ID) in first hop and source remote UE ID (layer-2 ID) in second hop) is excluded.

Proposal 5b: In Rel-18 with a single relay, ID(s) in option 4/5 should be same in each hop to avoid replacing ID in the SRAP header when relay UE transfers the received packet.

Proposal 5c: RAN2 to discuss which ID (24-bit layer-2 ID or short ID) can be used in SRAP header.

-If 24-bit layer-2 ID is used in the SRAP header, Option 3 (both source remote UE 24-bit layer-2 ID and target remote UE 24-bit layer-2 ID included in each hop) can be agreed.

Proposal 5d: If short ID is agreed, RAN2 to discuss which option can be agreed.

-Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop. (8)

-Option 4: Both source remote UE ID (local ID) and target remote UE ID (local ID) included in each hop. (11)

-Option 5: A local pair ID for a pair between source UD and target remote UE included in each hop. (9)

[Easy] [15:1] Proposal 5e: If short ID (one of Option 2, Option4 and Option 5) is agreed, relay UE is responsible for ID assignment.

In this paper we’ll discuss the Layer-2 specific aspects and RAN2 details for the control plane (CP) procedures, adaptation layer design and QoS handling based on the left-over proposals and SA2’s progress.

# 2	Discussion

The descriptions below are written in the context of a source remote (SRC) UE communicating with one or more destination (DST) remote UEs via a relay UE (U2U relay).

## 2.1 SRAP Layer Design

In the rapp’s summary from the previous meeting, the following options were being considered for the ID to be included in the SRAP header:

- Option 1: Target remote UE ID (layer-2 ID) in first hop and source remote UE ID (layer-2 ID) in second hop.

- Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop.

- Option 3: Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID) included in each hop.

- Option 4: Both source remote UE ID (local ID) and target remote UE ID (local ID) included in each hop.

Option 5: A common ID for a pair between source UE and target remote UE included in each hop.

From the above options, we think there is no need to deviate from Rel-17 U2N relaying in terms of including the local ID in the SRAP header. In addition, using a 48-bit information (source remote UE ID + target remote UE ID) at the SRAP layer leads to unnecessary packet overheads.

Using a 48-bit ID (source remote UE ID + target remote UE ID) at the SRAP layer leads to unnecessary packet overheads.

It should also be noted that the local ID need not be unique on a global level, it would just need to be unique to the relay connection at a particular U2U relay. Hence, size would not be a limiting factor.

Local ID is to be included in the SRAP header for source remote UE identification

Based on the above, we believe that Option 1 and Option 3 should be excluded. In addition, the principle of L2 relaying is that the U2U relay performs transparent forwarding of the data from the source remote UE to the destination remote UE i.e., the U2U relay is not required to perform any changes. As a result, we think that Option-2 should also be excluded.

The following options should be excluded from consideration:

- Option 1: Target remote UE ID (layer-2 ID) in first hop and source remote UE ID (layer-2 ID) in second hop

- Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop.

- Option 3: Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID) included in each hop

- Option 4 is reasonable and makes sense to include both the source remote UE ID and target remote UE ID for easy processing at the U2U relay. Whilst in Option 5, there is some benefit in terms of forward compatibility in the case of a multi-hop scenario. But it is still to be studied if this common ID pair is different value from the local ID.

- RAN2 can further down select between Options 4 and 5.

Down select between Option 4, where both source remote UE (local ID) and target remote UE (local ID) and Option 5, where a common ID for a pair between source and target remote UE is included in each hop. FFS if this common ID is different from the local ID.

- For Option 4, the local ID should be unique for a SRC, and DST UE and different values should be assigned for the SRC and DST. By assigning different values, if required, it is possible to reconfigure only one link without affecting the other link. For example, if there is a change in the local ID and if the local ID was common, then both the first and second hop links would have to be reconfigured to use the new local ID. As opposed to having different values where the change in the local ID over one link will not affect the other link.

Different local IDs are assigned to the SRC and DST UEs.

- Unlike in the case of U2N relaying, in U2U relaying, the SRC/DST/U2U relays would need to operate in any coverage scenario i.e., in-coverage, partial coverage and out-of-coverage. In which case, it is not possible to rely on the gNB to provide the appropriate configurations and local ID assignments. As a result, we believe the U2U relay should be responsible for the appropriate bearer mapping and should also be the one to assign the local IDs to the SRC/DST UEs.

The U2U relay assigns the local ID for the SRC and DST UEs.

## 2.3 Multiplexing Over First Hop

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

- From the source remote UEs perspective, the multiplexing of the data over the first hop can happen in two layers:

- RLC-layer Multiplexing (RLC-Multiplex): This is related to the above agreement i.e., the data from the SRAP-layer can be multiplexed into the same RLC-channel.

- MAC-layer Multiplexing (MAC-Multiplex): In addition, in the RAN2#121 meeting, there was also a discussion about multiplexing the data for different final destinations at the MAC-layer i.e., data from the different LCHs (different final destinations) can be multiplexed into the same grant.

- In general, for sidelink without relaying, it would make sense to multiplex the data from the different logical channels as they are intended for the same destination. However, for U2U relaying, this is not the case. The different RLC channels/LCHs are associated with different (final) destinations in the second hop.

- As a result, we believe there are issues when the source remote UE is always allowed to multiplex the data intended for different destinations over the first hop:

- For both RLC-Multiplex and MAC-Multiplex, as the different logical channels are associated with different (final) destination remote UEs, it is possible that low priority transmissions get a treatment above its indicated priority. Thereby degrading the performance of the other UEs in the system. In addition, as the source remote UE can communicate with multiple final destinations further exacerbating the problem.

- For both RLC-Multiplex and MAC-Multiplex, in mode-2, the selection window (sl-SelectionWindow-r16) is configured independently for each priority value (sl-Priority-r16). In which case, it is possible that the sl-SelectionWindow-r16 are different for different (final) destination remote UEs i.e., T1 for destination remote UE1, T2 for destination remote UE2 and T1 < T2. The source remote UE can select resources for transmission using T2 in which case multiplexing would of data could result in the PDB not being satisfied for one of the (final) destination remote UEs. The same is also applicable for the case when a high priority data arrives for a different (final) destination UEs.

- Based on our concerns above, we think RAN2 should discuss the issue of multiplexing of data associated with different (final) destination remote UEs. Although it is up to the source UE’s implementation whether to perform multiplexing, there should be some restrictions for when the UE is allowed to do so.

RAN2 to discuss restrictions for multiplexing data from different final destinations in the first hop.

## 2.3 QoS Handling

In an LS from SA2 [1], SA2 has asked RAN2 to define an AS solution/signalling to address the QoS splitting. This is because the negotiation for QoS handling is performed on an end-to-end connection i.e., between the source remote UE and target remote UE. But such negotiations are not visible to the U2U relay hence, additional AS-layer handling is required.

For QoS split, in L2 U2N relaying, it is up to gNB implementation to perform PDB split between Uu hop and PC5 hop. For L2 U2U relaying, however, gNB should not be involved in the QoS split as the UEs might be in different coverage scenarios and a common mechanism should be defined. Further, the traffic does not go through the network. In our view, the QoS split shall be performed by the U2U relay as it knows the channel situation of both the links i.e., first hop and the second hop.

The U2U relay configures the QoS split for the end-to-end unicast link.

Moreover, to avoid that the SRC/DST UE is unable to satisfy the QoS for the first/second hop respectively, the SRC UE may provide some assistance information to the U2U relay which takes this into account when configuring the QoS split.

SRC UE can provide the U2U relay with assistance info to assist in splitting the QoS.

## 2.4 End-to-End Connection Establishment

As mentioned above, SA2 has concluded the following procedures for connection establishment in L2 and L3 U2U relays [2]:

- Standalone Discovery Procedure based PC5 end-to-end unicast link establishment

- Integrated PC5 end-to-end unicast link establishment

Although SA2 has specified the solutions, the details for the L2 U2U relaying have not yet been captured and is up to RAN2 to finalize the details. In addition, RAN2 also needs to discuss the details of the sidelink relay adaptation protocol (SRAP) layer and handling of end-to-end QoS. Figure 1 below illustrates the RAN2 related CP procedures in L2 U2U relaying for link establishment based on the details as captured in [2].

Although SA2 has specified the solutions for standalone and integrated end-to-end unicast link establishment in general, the details for L2 U2U relaying have not yet been captured. RAN2 needs to finalize the details and in addition, study the aspects of SRAP and QoS handling.

- Standalone Discovery Procedure based PC5 end-to-end unicast link establishment

- Integrated PC5 end-to-end unicast link establishment

Figure 1. Connection Establishment Procedure for L2 U2U relays

The following are the steps involved in the end-to-end connection establishment procedure for both cases (a) and (b). The order of these steps can vary but the intention is to use the following as a starting point for discussions.

- With a standalone procedure, the SRC UE performs a discovery procedure (using model A/B) to find the DST UE. In the integrated procedure, the SRC UE initiates a direct communication request message (contents of which are decided by SA2) to find the DST UE.

- In the integrated procedure, the DST UE then performs the relay selection. Then, in both procedures, a PC5 link establishment/modification can be performed.

- In both procedures, the U2U relay can then configure the SRC UE and DST UE with the SRAP configuration along with the local ID(s).

- Subsequent messages for establishing an end-to-end unicast link are transmitted using the SRAP layer.

- Once the end-to-end unicast link is established, the per-hop bearer mapping with the option to perform QoS split can be performed.

RAN2 to consider Figure 1. as the baseline for L2 U2U relaying in PC5 end-to-end link establishment for cases (a) and (b).

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Using a 48-bit ID (source remote UE ID + target remote UE ID) at the SRAP layer leads to unnecessary packet overheads.

Observation 2	Although SA2 has specified the solutions for standalone and integrated end-to-end unicast link establishment in general, the details for L2 U2U relaying have not yet been captured. RAN2 needs to finalize the details and in addition, study the aspects of SRAP and QoS handling.

Based on the discussion in the previous sections we propose the following:O

Proposal 1	Local ID is to be included in the SRAP header for source remote UE identification

Proposal 2	The following options should be excluded from consideration:

Option 1: Target remote UE ID (layer-2 ID) in first hop and source remote UE ID (layer-2 ID) in second hop

Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop.

Option 3: Both source remote UE ID (layer-2 ID) and target remote UE ID (layer-2 ID) included in each hop

Proposal 3	Down select between Option 4, where both source remote UE (local ID) and target remote UE (local ID) and Option 5, where a common ID for a pair between source and target remote UE is included in each hop. FFS if this common ID is different from the local ID.

Proposal 4	Different local IDs are assigned to the SRC and DST UEs.

Proposal 5	The U2U relay assigns the local ID for the SRC and DST UEs.

Proposal 6	RAN2 to discuss restrictions for multiplexing data from different final destinations in the first hop.

Proposal 7	The U2U relay configures the QoS split for the end-to-end unicast link.

Proposal 8	SRC UE can provide the U2U relay with assistance info to assist in splitting the QoS.

Proposal 9	RAN2 to consider Figure 1. as the baseline for L2 U2U relaying in PC5 end-to-end link establishment for cases (a) and (b).

# 4 References

[1] S2-2305915, Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement, SA2#156e, April 17-21, 2023

[2] TS 23.304, V18.0.0, “Proximity based Service (ProSe) in the 5G System (5GS) (Release 18)”