# 3GPP TSG-WG2 Meeting #122                                 R2-2305621

# Incheon, Korea, May 22- 26 , 2023

Source: 			CMCC

Title:  			Considerations on multi-path scenario 2

Agenda item:	7.9.4

Document for: 	Discussion

Introduction

In last RAN2 meeting, we have the following conclusions and some open issues for MP scenario 2. In this contribution, we will discuss those open issues of scenario 2.

- Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

- Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.

- If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path.  Details of the reporting mechanism can be further discussed.

- In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

Discussion

Terminology

During study item, the terms specific to scenario 2 has been discussed, however, the terminology is still FFS.

As we clarifies with the discussion on scenario 2, the typical scenario of UE aggregation is to address the restricted UL bitrates and fulfills the 5G UL wide bandwidth service requirement. The linking topology in scenario 2 we expect is the anchor UE connects to more than one associated UE. So, the terms “anchor UE” and “associated UE” are suggested to be used for scenario 2.

Proposal 1:  Terms “anchor UE” and “associated UE” are suggested to be used for scenario 2.

UE-UE association

UE association between anchor UE and associated UE is assumed as pre-configured or static. gNB pre-configures the associated UE information to an authorized UE, before the indirect path is added to anchor UE. Considering the flexibility of associated UE selection in scenario 2, anchor UE can inform gNB with one or more candidate associated associated UE and gNB make decision on the selection of the associated UE.

Proposal 2: Anchor UE can inform gNB with one or more candidate associated UEs and gNB decides the associated UE.

How to identify the associated UEs in scenario 2 had been discussed. If both associated UE and anchor UE are in RRC-CONNECTED, the anchor UE reports associated UE’s ID to gNB. For associated UE in RRC_CONNECTED state, C-RNTI can be used as associated UE ID. For associated UEs in RRC_IDLE/RRC_INACTIVE, temporary or local ID can be pre-configured as UE identification.

For scenario 2, it was agreed that it is left to UE implementation to trigger the RRC_IDLE/RRC_INACTIVE target associated UE to initiate RRC connection establishment procedure. However, if the associated UE is in RRC_CONNECTED state before the gNB configure it, it will avoid latency caused by RRC establishment. The RRC state can be taken into account when anchor UE reports the associated UEs or when gNB select the associated UE. We have some concerns on the idea that anchor UE triggers the RRC_IDLE/RRC_INACTIVE associated UE into RRC_CONNECTED state while reporting the associated UE information. For the UE selected as associated UE, it may accelerate the multi-path establishment procedure. But, for UE who are not chosen, it brings extra expanses.

Proposal 3 : For associated UEs in RRC_CONNECTED, C-RNTI can be used as associated UE identification.

Proposal 4 : For associated UEs in RRC_IDLE/RRC_INACTIVE, temporary or local ID can be used as UE identification.

Proposal 5: The RRC state can be taken into account when anchor UE reports the candidate associated UE or when gNB decides the associated UE.

UE-UE link failure

RAN2 have discussed how to handle the failure over UE-UE link in scenario 2 and agreed that UE-UE link failure detection is supported. Anchor UE can report UE-UE link failure to network via direct path, once the link failure is detected. From network’s perspective, it can reconfigure another aggregated UE to anchor UE for indirect link recovery. The aggregated UE to be reconfigured as indirect path can be in the list of candidate associated UE as informed by anchor UE before. The anchor UE can also report one or more associated UEs to gNB along with the UE-UE link failure report.

Proposal 5: Anchor UE can report UE to UE link failure to network via direct path, once the UE-UE link failure is detected.

Proposal 6: Network can reconfigure another aggregated UE for indirect link recovery, where that aggregated UE can be the candidate associated UE as the informed by anchor UE.

Besides the scenario of UE-UE link failure and recovery, gNB can re-configure another associated UE to anchor UE to change the indirect path for better system performance with some trigger events. Comparing with the recovery for UE-UE link failure, indirect path change can lower the latency and avoid the possible data loss.

Proposal 7: Indirect path change can be supported for scenario 2.

Bearer mapping configuration

In RAN2 #121 meeting, it was concluded that gNB provides bearer mapping information to associated UE through dedicated signalling. We further discuss the bearer mapping information here. 1:1 bearer mapping between anchor UE’s RB and Uu RLC channel of associated UE, only LCID is needed to over uu link in scenario 2. When gNB reconfigures the anchor UE to add the indirect path, the mapping between the radio bearer of anchor UE and the RLC logical channel information of associated UE is contained in this reconfiguration message. Such mapping rule is also included in RRC reconfiguration message to associated UE. With such configuration for both anchor UE and associated UE, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link. Legacy IE, RadioBearerConfig for radio bearer of anchor UE and RLC-BearerConfig for uu RLC entity of associated UE can be used to configure anchor UE and associated UE.

- Proposal 8：gNB configures anchor UE and associated UE with the mapping between the radio bearer of anchor UE and the LCID (uu RLC entity) of associated UE.

- Proposal 9: RadioBearerConfig for radio bearer of anchor UE and RLC-BearerConfig for uu RLC entity of associated UE can be used to configure anchor UE and associated UE.

Indirect path adding/release

For scenario 2, the link between UE to UE are non-standard interface, the link establishment procedure is out of 3GPP scope. However, the conditions to trigger the UE to UE link establishment is needed for scenario 2. In our understanding, for scenario 2, the multi-path establishment can be triggered by UE assistant information report or by gNB implementation, such as, measurement report similar as SL associated, UE buffer size status report for UE triggered multi-path establishment, and service/traffic type or some other pre-configuration for gNB implementation triggered multi-path establishment. Here, we suggest RAN2 to have further discussion on that.

Proposal 10: Ask RAN2 to discuss the trigger event for multi-path establishment in scenario 2.

As in Fig.1, anchor UE with direct path adds the indirect path for multi-path for scenario 2,

Step 1: The anchor UE reports trigger event relative information to gNB (FFS), or gNB triggered indirect path adding with gNB implementation. (FFS)

Step 2: gNB decides to add indirect path for the anchor UE.

Step 3: gNB sends the RRCReconfiguration message to anchor UE, which indicates the associated UE for indirect path, the association between E2E PDCP and uu RLC channel of associated UE, data duplication or split related configuration.

Step 4: gNB sends the RRCReconfiguration message to associated UE, which contains the bearer mapping from E2E RB to uu RLC channel of associated UE.

Step 5: The anchor UE and associated UE establish non-standard link. (without 3GPP impact)

Step 6: The anchor UE sends the RRCReconfigurationComplete message to gNB via direct path.

Fig.1 Indirect path adding procedure for scenario 2

Proposal 11: It is proposed to agree Fig.1 as baseline for indirect path adding for scenario 2.

As in Fig.2, anchor UE configured multi-path release the indirect path for multi-path for scenario 2,

Step 1: The gNB decides to release the indirect path.

Step 2: The gNB sends the RRCReconfiguration message to anchor UE to release the indirect path.

Setp 3: The gNB sends the RRCReconfiguration message to associated UE to release the anchor UE related configuration.

Step 4: The anchor UE or the associated UE release non-standard link.(without 3GPP impact)

Fig.2 Indirect path release procedure for scenario 2

Proposal 12: It is proposed to agree Fig.2 as baseline for indirect path release for scenario 2.

Protocol stack

With the progress on topology and the presence of adaption layer for scenario 2, it is very clear that the protocol stack for scenario 1 and 2 are different. The typical scenario 1 is multiple anchor UEs associated to one associated UE, the SRAP is necessary to distinguish the packets from different anchor UEs encapsulated in one logical channel. Conversely, as in scenario 2, one associated UE connects to more than one anchor UE, where SRAP is not needed and the data routing for split bearers can be taken over by PDCP, as illustrated in Figure 3-1 for UP and figure 3-2 for CP.

Figure.3-1 DC-like UP protocol stack for scenario 2

Figure.3-2 DC-like CP protocol stack for scenario 2

Alternatively, for UE aggregation in scenario 2, we propose another possible kinds of protocol stack, DAPS like, which is shown as Figure.3-3. For DAPS-like UE aggregation, the aggregated RB will be configured with two or multiple PDCP entities and RLC entities for different UEs respectively, which are involved in the delivery of the aggregated service. Besides, PDCP in anchor UE will response for the unified SN allocation across the anchor UE and aggregated UE(s). PDCP SDU with allocated SN will be further handled by PDCP in aggregated UE for security and ROHC.

Figure.3-3 UP protocol stack for scenario 2 (DAPS-like)

Figure.3-4 CP protocol stack for scenario 2 (DAPS-like)

Proposal 13: Agree TP to TR 38.836 as in Annex.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Terms “anchor UE” and “associated UE” are suggested to be used for scenario 2.

Proposal 2: Anchor UE can inform gNB with one or more candidate associated UEs and gNB decides the associated UE.

Proposal 3 : For associated UEs in RRC_CONNECTED, C-RNTI can be used as associated UE identification.

Proposal 4 : For associated UEs in RRC_IDLE/RRC_INACTIVE, temporary or local ID can be used as UE identification.

Proposal 5: The RRC state can be taken into account when anchor UE reports the candidate associated UE or when gNB decides the associated UE.

Proposal 6: Anchor UE can report UE to UE link failure to network via direct path, once the UE-UE link failure is detected.

Proposal 7: Network can reconfigure another aggregated UE for indirect link recovery, where that aggregated UE can be the candidate associated UE as the informed by anchor UE.

Proposal 8: Indirect path change can be supported for scenario 2.

- Proposal 9：gNB configures anchor UE and associated UE with the mapping between the radio bearer of anchor UE and the LCID (uu RLC entity) of associated UE.

- Proposal 10: RadioBearerConfig for radio bearer of anchor UE and RLC-BearerConfig for uu RLC entity of associated UE can be used to configure anchor UE and associated UE.

Proposal 11: Ask RAN2 to discuss the trigger event for multi-path establishment in scenario 2.

Proposal 12: It is proposed to agree Fig.1 as baseline for indirect path adding for scenario 2.

Proposal 13: It is proposed to agree Fig.2 as baseline for indirect path release for scenario 2.

Proposal 14: Agree TP to TR 38.836 as in Annex.

Annex

- (TP to TR 38.836 for SL relay enhancement)

  - 4.x	Multi-path for L2 U2N relay

  - 4.x.1	Architecture and Protocol Stack

    - 4.x.1.1	Protocol Stack

The protocol stacks for user plane and control plane of multi-path scenario 2 are described in Figure 4.x.1.1-1 and Figure 4.x.1.1-2 for DC-like architecture, Figure 4.x.1.1-3 and Figure 4.x.1.1-4 for DAPS-like architecture.

Figure 4.x.1.1-1:User plane protocol stack for scenario 2 (DC-like)

Figure 4.x.1.1-2:Control plane protocol stack for scenario 2(DC-like)

Figure.4.x.1.1-3 User plane protocol stack for scenario 2 (DAPS-like)

Figure.4.x.1.1-4 Control plane protocol stack for scenario 2 (DAPS-like)