3GPP TSG RAN WG2 #122	R2-2306109

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.14.3

Source:	ZTE Corporation, Sanechips

Title:	Considerations on Rel-17 leftover issues for QoE

Document for:	Discussion and Decision

Introduction

The new WID of enhancement on NR QoE management and optimizations for diverse services was approved in RAN#96[1]. In which, the following objective is included:

... - Left-over features from Rel-17, as well as the enhancements of existing features which are not included in Rel-17 normative phase, should be supported in Rel-18 if consensus on benefits are reached [RAN3, RAN2]. - Specify per-slice QoE measurement configuration enhancement. - Specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1. - Specify QoE reporting handling enhancement for overload scenario.

In RAN2#119bis e-meeting, based on the contributions, the following agreements have been achieved:

- From RAN2’s perspective, there is no further work for per-slice-based QoE measurement. - RAN2 can wait for RAN3 progress on enhancement to per-slice RAN visible QoE measurement. - RAN2 needs to wait for the progress of RAN3 on RVQoE value. - The enhancement on UAI message to express the UE’s preference on QoE reporting configurations is not pursued. - QoE reporting via unlicensed band is out of the WID scope. - RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3). - FFS on whether to send the priority information 1) UE and gNB or 2) only to gNB - To wait for RAN3 decision on granularity of priority. - RAN2 can discuss event-based RVQoE, including possible options, benefits, spec impacts, and complexities based on company contributions. - FFS whether to add the QoS flow ID in the RVQoE report. If RAN3 already agreed to this, RAN2 can progress this in the next meeting where we discuss Rel-17 leftovers.

Based on the discussion in RAN2#121 meeting, following agreements on QoE measurements have been achieved:

- Introduce the QoS flows ID information in the RVQoE reporting from the UE.

In this contribution, we will discuss the remaining issues of overload scenario enhancement and RAN visible QoE enhancement. Then we’ll give our proposals.

Discussion

Overload scenario enhancement

An LS [2] is sent from RAN3 to RAN2 as following indicating that RAN3 has agreed that there is no need to send assistance information to UE.

# 1 Overall description RAN3 discussed about the assistance information for handling of QoE reporting during RAN overload. RAN3 noticed that the same issue is also under discussion in RAN2, and RAN3 would like to inform RAN2 about the following agreement: In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE. # 2 Actions To RAN2: ACTION: RAN3 kindly asks RAN2 to take the above into account and provide feedback if necessary.

According to the above LS,  RAN3 thinks there is no need to send assistance information to UE during RAN overload. In our understanding, in Rel-17 the reporting of encapsulated QoE measurements can be temporarily paused in RAN overload situations. It is left to gNB implementation which encapsulated QoE configurations of a UE to pause for reporting. The RRC specification allows a selective pause, i.e. the gNB can pause reporting of one or multiple or all QoE configurations of a UE.

In addition, RAN2 discussed whether priority information needs to be sent to UE. We don't think there is a need for both NW-based and UE-based solutions. And normally if something can be done by NW without significant burden then we don’t introduce UE based solution since the resource in air interface is more limited. Therefore, we think RAN2 can support the RAN3’s conclusion on this topic.

Proposal 1: RAN2 doesn’t discuss further enhancements (i.e. UE-based solution other than pause/resume) to deal with RAN overload.

RVQoE enhancements

An LS [3] is sent from SA4 to RAN2 as following indicating that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer.

- 1 Overall description SA4 thanks RAN2 on their LS on buffer level threshold-based RVQoE reporting. Given that - There already exists mechanism before Rel-18 for application layer to be configured for QoE reporting, and that this mechanism can be reused by the application layer to do RVQoE reporting based on the trigger of the buffer level threshold, and - The application layer can make a buffer-threshold based decision in a more timely fashion compared to the AS layer, since the corresponding application layer reporting, based strictly on reporting periodicity, may be unable to submit QoE reports at the exact time that buffer level threshold is reached. As result, and depending on the reporting periodicity, the delay between a threshold occurrence and the next scheduled QoE report may precluding a more timely remedial response by the gNB. Hence SA4 can confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. - 2 Actions To RAN2 ACTION: SA4 kindly requests RAN2 to take the above reply into account and to keep SA4 updated on further details of work on buffer level threshold-based RVQoE reporting.

Based on the above LS, the application layer can make a buffer-threshold based decision in a more timely fashion compared to the AS layer. we also think it is better to choose app layer. Because the measurement for buffer level is performed in application layer and the application layer can evaluate the buffer level directly. Therefore, we think the application layer can make a buffer-threshold based decision to align with SA4’s conclusion.

Proposal 2: Confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by  application layer.

In addition, another issue that needs to be discussed is the details of buffer level thresholds events. For example whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two threshols. Also RAN3 has already started the discussion taking into account the LS last meeting, and has identified some candidate solutions on the details of measurement procedure, e.g., how to check the entering/leaving condition of events configured (e.g., periodicall，one-shot or others) and how to perform the measurements (periodically, event-periodically).  RAN2 can leave the discussion to RAN3 to avoid duplicated discussion on this topic.

Proposal 3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

For the configuration of the buffer level threshold, the threshold should be configured within RVQoE configuration. Because the AS layer needs to know whether send the RVQoE report to gNB periodically or based on event trigger according to this parameter. After the AS layer receiving the RVQoE configuration, the  application layer can receive the buffer level threshold from the AS layer and determine whether send the RVQoE report to AS layer based on this threshold.

Proposal 4: Introduce a buffer level threshold information in the RVQoE configuration from the gNB.

Conclusion

In previous sections, the following observations and proposals were made:

Proposal 1: RAN2 doesn’t discuss further enhancements (i.e. UE-based solution other than pause/resume) to deal with RAN overload

Proposal 2: Confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by  application layer.

Proposal 3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

Proposal 4: Introduce a buffer level threshold information in the RVQoE configuration from the gNB.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022

- R2-2302425, LS on assistance information for handling of QoE reporting upon RAN overload, April, 2023

- R2-2304492, Reply LS on buffer level threshold-based RVQoE reporting, April, 2023