3GPP TSG-RAN WG2 Meeting #122                   		         R2-2306285

Incheon, Korea, 22 - 26 May 2023                                  Revision of R2-2304173

Agenda item:	7.16.2.1

Source:	LG Electronics Inc.

Title:	AIML method Architecture General

Document for:	Discussion and Decision

# 1. Introduction

RAN2 reached agreements regarding Model ID as following:

RAN2 #119b R2 assumes that a model is identified by a model ID. Its usage is FFS. RAN2 #119b R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN2 #121 RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified. RAN2 #121bis Model ID can be used to identify model or models for the following LCM purposes: - model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). - (e.g. for so called “model ID based LCM”) If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose. How to achieve globality of the Model ID is FFS. - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point: - Direction1: Pre-defined/hard-coded global unique model ID - Direction3: Assigned global unique model ID via specific ID management node. - Note: Other global unique model ID definition is not precluded. - Model ID structure, if any, is FFS

In this contribution, we discuss architecture general related to the model ID and mapping of functionality to entities focusing on the CSI/beam prediction use case.

# 2. Discussion

## 2.1 Model ID

### 2.1.1 Model ID Type

As per the RAN1 agreement in RAN1 #112bis, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

Observation 1. As per the RAN1 agreement, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

RAN2 assumed that model ID is unique “globally”, and the Model ID structure is FFS. However, a formal(global) model ID may be too long to uniquely identify models globally. Using such a long ID over radio networks is not efficient. It would be beneficial if a formal model ID is temporarily mapped to a shorter model ID and the logical model ID is used over 3GPP radio interface. To identify a model by a logical model ID, some dictionary or encoding/decoding mechanism may be needed to enable translation between logical IDs and formal IDs.

Observation 2. A formal(global) model ID may be too long to uniquely identify models globally. Using such a long ID over radio networks is not efficient.

RAN2 can study the logical model ID as a starting point. It is FFS how to provision logical ID in relation to formal model IDs. How to provision logical IDs may also be dependent on model transfer/delivery methods.

Proposal 1. To study logical model ID with the following assumptions:

- logical model ID is temporarily mapped to a global model ID. FFS where/how the mapping is done (considering model transfer/delivery methods)

- logical model ID is used to control model over radio interface

### 2.1.2 Model ID Use Cases

From the LCM perspective, if multiple models are configured for a single functionality, different model should be assigned a different model ID.

Observation 3. From the LCM perspective, if multiple models are configured for a single functionality, different model should be assigned a different model ID.

There are scenarios where the model ID is necessary for model management as follows:

Scenario 1) When the location of model training and model inference is different:

- Model ID is included along with the model-related configuration (model transfer/delivery) from model training site.

- Model ID is included in model inference results from model inference site.

- Model ID and model inference results can be referred to model training in model training site.

- There are two use cases:

  - NW-sided training / UE-sided inference

  - UE-sided training / NW-sided inference : it can be deprioritized

Scenario 2) When the location of model inference and model monitoring is different:

- Model ID is included in the model inference results from model inference site.

- Model ID and the model inference results can be referred to the model monitoring for activation/deactivation/switching of the corresponding model in model monitoring site.

- There are two use cases:

  - UE-sided inference / NW-sided monitoring

  - NW-sided inference / UE-sided monitoring

Scenario 3) When the location of model monitoring and model training is different:

- Model ID is included in the model monitoring-related information, e.g., accuracy, performance metric, etc., from model monitoring site

- Model ID and model monitoring-related information is used for model training in model training site

- There are two use cases:

  - UE-sided monitoring / NW-sided training

  - NW-sided monitoring / UE-sided training

| NW UE | Model Training | Model Inference | Model Monitoring |
| --- | --- | --- | --- |
| Model Training | NA | Deprioritize | Network to UE: - Model monitoring results with model ID, e.g., performance metric - Model management with model ID, e.g., model deactivation |
| Model Inference | Network to UE: - Model transfer/delivery with model ID UE to Network: - Model inference data (output) with model ID | NA | Network to UE: - Model monitoring results with model ID, e.g., performance metric - Model management with model ID, e.g., model deactivation UE to Network - Model inference data(output) with model ID |
| Model Monitoring | Network to UE: - Model update with model ID UE to Network: - Model monitoring results with model ID, e.g., performance metric - Model management result with model ID, e.g., model switching | Network to UE: - Model inference data(output) with model ID UE to Network: - Model monitoring results with model ID, e.g., performance metric - Model management result with model ID, e.g., model switching | NA |

<Table 1. Model ID usage cases per each scenario for CSI/Beam prediction use cases>

For example, in the case of a combination of sceanrio1 and scenario 2, the network and the UE can operate as follow:

<Figure 1. signaling procedures with model LCM>

Step 0. The Network sends beam resource configuration and report configuration for model training.

Step 1. The UE performs measurement and sends the measurement results.

Step 2. The network performs model training for several models. [Model Training]

Step 3. The network transfer models with model IDs and sets model related configurations for multiple models. Each model configuration can have beam resource configuration and report configuration for model inference. Each model configuration can have report configuration for the calculation of performance metric for model monitoring.

Step 4. The network sends model activation command with a model ID.

Step 5. The UE sends model inference results with model ID. [Model Inference]

Step 6. The network can facilitate the inference result for training the model indicated by model ID [Model Training]

Step 7. The UE sends a report related to model monitoring configuration with model ID.

Step 8. The network recognizes the model based on the model ID and evaluates the model based on the report related to model monitoring. The network can also use model inference results for model monitoring. [Model Monitoring]

Step 9a. The network sends model deactivation/switching command with the model ID.

Step 9b. The network sends model update configuration with the model ID.

Therefore, it is proposed to study each scenario or combination of scenarios for requiring Model ID configuration to define why a model ID is needed and how it is used.

Proposal 2. To study each scenario and/or combination of scenarios whether model ID is necessary and how it is used.

- Scenario 1) The location of model training and model inference is different

- Scenario 2) The location of model inference and model monitoring is different

- Scenario 3) The location of model monitoring and model training is different

In addition, RAN2 can consider the model-switching scenario. In some cases, model ID is necessary for the model-related report for each LCM purpose. For example, the model-related report in step 9a may include the inference result of the old model (model A) and the inference result of the current model (model B). If the network fails to identify which model was used for the inference, it may make incorrect decisions based on the outdated report from the old model. Similarly, if the model-related report of the old model is utilized for model monitoring, the network may mistakenly reconfigure the UE with the wrong model or incorrect model parameters.

Observation 4. For the model switching scenario,

- If the model-related report of the old model is used for model inference, it may make incorrect decisions based on the outdated report from the old model.

- If the model-related report of the old model is utilized for model monitoring, the network may mistakenly reconfigure the UE with the wrong model or incorrect model parameters.

| Model management | Model Training | Model Inference | Model Monitoring |
| --- | --- | --- | --- |
| Model Switching | Up to method of data collection | -Model Inference Results for the previous model with model ID -Model Inference Results for the current model with model ID | -Model monitoring data, e.g., performance metric, for the previous model with model ID - Model monitoring data, e.g., performance metric, for the current model with model ID |
| … | … | … | … |

<Table 2. Model ID usage cases for model switching for CSI/Beam prediction use cases>

For example, the network and the UE can operate as follow:

<Figure 2. signaling procedures with model switching >

Step 0. The network transfer models with model IDs and sets model related configurations for multiple models. Each model configuration can have beam resource configuration and report configuration for model inference. Each model configuration can have report configuration for the calculation of performance metric for model monitoring.

Step 1. The network sends model activation command with model A.

Step 2-3. The UE performs model inference and sends model inference results. [Model Inference for model A]

Step 4. The UE sends a report related to model monitoring configuration for model A.

Step 5. The network recognizes the model based on the model ID and evaluates the model based on the report related to model monitoring. The network can also use model inference results for model monitoring. [Model Monitoring for model A]

Step 6. The UE still performs model inference for model A

Step 7. The network sends a model switching command for model B.

Step 8a-9a. The UE performs model inference for model B. The UE sends model inference results for model A/B with model ID. [Model Inference results for model A/B]

Step 8b-9b. The UE removes model inference results for model A and perform model inference for model B. The UE sends model inference results for model B with model ID. [Model Inference results for model B]

Therefore, we propose to study model switching scenario whether model ID is necessary to identify model in the model-related report. It is FFS whether model-related results for old model are removed when model is switched.

Proposal 3. To study model switching scenario whether model ID is necessary to identify model and how it is used.

- FFS whether model related results for old model are removed when model is switching.

As a starting point, RAN2 can study above scenario that requires Model ID. Also, we can capture the above procedures in TR for further discussion with the consensus in RAN2.

Proposal 4: Capture the procedure/table of P2/P3 that requires Model ID into the TR

### 2.1.3 Meta Information

Through the latest email discussion, RAN2 needs to further study the content of meta data.

Open issue 6: RAN2 to further study the content of meta data, the following bullets can be considered as the starting point for future study: - a, model input info; - b, model output info; - c, model version info; - d, model format info; - e, required AI capability; - f, vendor info; - g, applicable scenario, configuration, site information; - h, computational complexity: FLOPs, level of pre-/post-processing; - i, model complexity: number of real-value model parameter, number of real-value operations; - j, model size; - k, model performance: Model accuracy, model bias, model variance; - l, model functionality; - m, use case. - Note: The coordination with RAN1 is needed.

However, regarding the contents of meta info, they are currently under discussion in RAN1. In RAN1, g is being discussed as the most important information, and additional discussion is needed for the remaining contents. To avoid redundant discussions, we propose to postpone discussion of the detailed contents within the meta information.

Proposal 5. To postpone discussion of the details within the meta information

It is challenging to specify all meta information. If essential information is required, the model ID and associated meta information can be provided. That is meta information can be conveyed along with the model ID. The essential meta information is up to RAN1 input, and how to associate the meta information with model ID is FFS.

Proposal 6. Meta information can be conveyed along with the model ID. FFS how to associate the meta information with model ID

## 2.2 Mapping of Functionality to Entities

RAN2 is currently discussing the model transfer/delivery for various entities such as gNB, CN, and servers. The location of model transfer/delivery is related to the location of model training. As per the RAN1 agreements, the model training can be performed by UEs, NW, and neutral sites. To align with this, RAN2 can also open to studying for multiple entities for model training rather than assuming a single entity.

RAN1 #112 Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side.

Observation 5. RAN1 has made an agreement related to training location that model training can be performed by UEs, NW, and neutral sites. To align with this, RAN2 can also open to studying for multiple entities for model training rather than assuming a single entity.

Regarding Model inference, it should be performed at the entity responsible for the actual legacy operation for each use case. Otherwise, there would be duplicated unnecessary operations in the other entity. Accordingly, for CSI and Beam use cases, the gNB can be the target entity for the NW-sided model.

Observation 6. Model inference should be performed at the entity responsible for the actual legacy operation for each use case. Accordingly, for CSI and Beam use cases, the gNB can be the target entity for the NW-sided model.

As for the Model monitoring, RAN1's agreement was reached as follows. For the CSI and Beam use cases, regardless of whether it is UE-sided monitoring or NW-sided monitoring, the NW (gNB) is involved in the configuration/signaling for monitoring.

RAN1 #112 Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and re]=porting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indictation/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring Other aspect(s) is not precluded

For NW-sided monitoring, the gNB can understand the results of CSI or Beam-related model monitoring. Therefore, the gNB should be responsible for the model monitoring to comprehend the inference results and ensure timely model updates.

Observation 7. For NW-sided monitoring, the gNB can understand the results of CSI or Beam-related model monitoring. Therefore, the gNB should be responsible for the model monitoring to comprehend the inference results and ensure timely model updates.

For UE-sided monitoring, the UE is responsible the model monitoring. However, the involvement of the gNB may vary depending on whether the UE forwards the monitoring results to the NW or not.

Observation 8. For UE-sided monitoring, the UE is responsible the model monitoring. However, the involvement of the gNB may vary depending on whether the UE forwards the monitoring results to the NW or not.

Therefore, for CSI/Beam use cases, it is proposed to take a base line for mapping functionality of entities as follows:

- Model training: CN/Server/gNB/UE

- Model inference: gNB/UE

- Model Monitoring: gNB/UE

| Use case | Model Training | Model Inference | Model Monitoring |
| --- | --- | --- | --- |
| CSI compression | CN/Server/gNB/UE | -CSI generation: UE -CSI reconstruction: gNB | -NW sided monitoring: gNB -UE sided monitoring: UE |
| CSI prediction | CN/Server/gNB/UE | -NW sided model: gNB -UE sided model: UE | -NW sided monitoring: gNB -UE sided monitoring: UE |
| Beam management | CN/Server/gNB/UE | -NW sided model: gNB -UE sided model: UE | -NW sided monitoring: gNB -UE sided monitoring: UE |

<Table 3. Functionality Mapping table of entities for CSI/Beam use cases >

Proposal 7. For CSI/Beam use cases, to take a baseline for mapping functionality of entities as follows:

- Model training: CN/Server/gNB/UE

- Model inference: gNB/UE

- Model Monitoring: gNB/UE

# 3. Conclusion

Observation 1. As per the RAN1 agreement, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

Observation 2. A formal(global) model ID may be too long to uniquely identify models globally. Using such a long ID over radio networks is not efficient.

Proposal 1. To study logical model ID with the following assumptions:

- logical model ID is temporarily mapped to a global model ID. FFS where/how the mapping is done (considering model transfer/delivery methods)

- logical model ID is used to control model over radio interface

Observation 3. From the LCM perspective, if multiple models are configured for a single functionality, different model should be assigned a different model ID.

Proposal 2. To study each scenario and/or combination of scenarios whether model ID is necessary and how it is used.

- Scenario 1) The location of model training and model inference is different

- Scenario 2) The location of model inference and model monitoring is different

- Scenario 3) The location of model monitoring and model training is different

Observation 4. For the model switching scenario,

- If the model-related report of the old model is used for model inference, it may make incorrect decisions based on the outdated report from the old model.

- If the model-related report of the old model is utilized for model monitoring, the network may mistakenly reconfigure the UE with the wrong model or incorrect model parameters.

Proposal 3. To study model switching scenario whether model ID is necessary to identify model and how it is used.

- FFS whether model related results for old model are removed when model is switching.

Proposal 4: Capture the procedure/table of P2/P3 that requires Model ID into the TR

Proposal 5. To postpone discussion of the details within the meta information

Proposal 6. Meta information can be conveyed along with the model ID. FFS how to associate the meta information with model ID

Observation 5. RAN1 has made an agreement related to training location that model training can be performed by UEs, NW, and neutral sites. To align with this, RAN2 can also open to studying for multiple entities for model training rather than assuming a single entity.

Observation 6. Model inference should be performed at the entity responsible for the actual legacy operation for each use case. Accordingly, for CSI and Beam use cases, the gNB can be the target entity for the NW-sided model.

Observation 7. For NW-sided monitoring, the gNB can understand the results of CSI or Beam-related model monitoring. Therefore, the gNB should be responsible for the model monitoring to comprehend the inference results and ensure timely model updates.

Observation 8. For UE-sided monitoring, the UE is responsible the model monitoring. However, the involvement of the gNB may vary depending on whether the UE forwards the monitoring results to the NW or not.

Proposal 7. For CSI/Beam use cases, to take a baseline for mapping functionality of entities as follows:

- Model training: CN/Server/gNB/UE

- Model inference: gNB/UE

- Model Monitoring: gNB/UE

# 4. References

[1] R2-2304195 Report of [AT121bis-e][014][AIML18] Model ID (incl meta data) progress (OPPO)_v34_Summary