3GPP TSG-RAN WG2 Meeting #122                                                                        R2-2306529

Incheon, Korea, May 22nd- 26th 2023

Agenda item:	7.8.2

Source:	China Telecom

Title:	Measurement reporting for mobility and interference control

Document for:	Discussion and Decision

- Introduction

The WID on NR Support for UAV [1] was approved in Rel-18. One of the objectives is to specify the enhancements on measurement reports of UAV. In the previous meetings, the following agreements were achieved [2]:

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

The intention of this paper is to further discuss the remaining issues and related details of measurement reporting enhancement.

- Discussion

2.1	Event combination

Unlike the legacy UE in ground, the mobility mechanism for UAV should consider vertical direction information. With more height-depending information collected from UAV, NW can make an appropriate strategic decision mobility for the UAV to ensure the handover to target gNB timely and correctly. In RAN2#121bis meeting, the MR can be configured with height-dependent more-than-one configurations/values. In existing measurement configuration mechanism for LTE and NR UAV, the height-depending measurement event H1/2 was introduced. One solution is to combine events, i.e., to combine A4 with H1/H2 [3][4]. The benefits or advantage of this solution is that it can reduce measurement reporting and increase the flexibility of measurement configuration. Therefore, we propose to introduce a new measurement event combination mechanism to combine A4 and H1/2 for UAV UE.

Proposal 1: agree to introduce the combination of event A4 and height-dependent event H1/2.

2.2	Height-dependent Configuration

which parameters that can be configured with height-dependent multiple configurations/values was discussed in post-email discussion. At least the following parameters can be used for height-dependent multiple configuration in NR Rel-18 UAV:

- SSB-ToMeasure

- allowed/excluded cells

- absThreshSS-BlocksConsolidation ThresholdNR, nrofSS-BlocksToAverage, nrofCSI-RS-ResourcesToAverage

In the last meeting [2], it was agreed that SSB-ToMeasure can be configured with height-dependent more-than-one configurations/values, each for a specific height region. As the flight height of UAV UE increases, UAV UEs will find more cells and even measure the side lobes of far cells’ beams. Introducing height-based SSB-ToMeasure would avoid unnecessary RRM measurement on the down-tilted beams that are not intended for UAV UE. Therefore, we proposed to introduce height-dependent SSB-ToMeasure for reducing the number of beam measurements and measurement reporting.

Observation 1: NR Rel-18 UAV is expected to support height-dependent SSB-ToMeasure.

Besides, the no-transmit/no-fly zones might be imposed to limit the transmissions from drones in some scenarios. The purpose of that is usually to avoid causing too much interference to nearby base stations. Linking allowed/excluded cell lists with UAV UEs’ height allows for not performing measurements on the unneeded cells in a certain height range. Reducing measurements at specific altitudes would limit the size of the measurement report and reduce UE power consumption .

Observation 2: Link allowed/excluded cell lists with UAV UEs’ height would limit the size of the measurement report and reduce UE power consumption.

As shown below [5], absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage is part of height dependent cell quality derivation parameters in MeasObjectNR.

------------------------------------------------------------- Excerpted from TS 38.331 ---------------------------------------------------------

MeasObjectNR field descriptions

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

----------------------------------------------------------------------End of Exception ------------------------------------------------------------

The cell quality in NR is derived based on the consolidation of beam-level measurements. Since the height-dependent SSB-to-measure is expected to be supported in NR UAV, the relevant parameter configuration should also be enhanced. To improve interference control, it’s better to use a smaller number of nrofSS-BlocksToAverage and a higher measurement consolidation threshold.

Proposal 2: To introduce the following MO configuration related parameters:

- SSB-ToMeasure

- allowed/excluded cells

- consolidation threshold (e.g. absThreshSS-BlocksConsolidation)

- max number of the beam to average (e.g. nrofSS-BlocksToAverage)

In our view, when such a configuration switch is triggered, the UE behavior is to receive an RRCReconfiguration message with MeasConfig from the NW. This would be similar to the case of a normal measurement reconfiguration triggered by the NW and therefore no new UE behavior needs to be specified. For SSB-ToMeasure, only the beam configuration is affected by the flying height, so the NW is not interested in the RRM configuration for the specific application of the UE. Only available measurements need to be sent by the UE to the NW.

Proposal 3: There is no need to specify new UE behavior for height-dependent SSB-ToMeasure.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation 1: NR Rel-18 UAV is expected to support height-dependent SSB-ToMeasure.

Observation 2: Link allowed/excluded cell lists with UAV UEs’ height would limit the size of the measurement report and reduce UE power consumption.

Proposal 1: agree to introduce the combination of event A4 and height-dependent event H1/2.

Proposal 2: To introduce the following MO configuration related parameters:

- SSB-ToMeasure

- allowed/excluded cells

- consolidation threshold (e.g. absThreshSS-BlocksConsolidation)

- max number of the beam to average (e.g. nrofSS-BlocksToAverage)

Proposal 3: There is no need to specify new UE behavior for height-dependent SSB-ToMeasure.

- References

- RP-213600 WI on NR Support for UAV (NR_UAV), RAN#94e, December 2021.

- R2-230xxxx Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online

- R2- 2212638,Further discussion on UAV measurement enhancements, Huawei, HiSilicon

- R2-2212657, Discussion on measurement reporting for NR UAV, Xiaomi

- TS 38.331 V17.2.0, Radio Resource Control (RRC) protocol specification