3GPP TSG RAN WG2 Meeting #122          	      	                	                  R2-2306871

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	7.12.2.2

Source:	Intel Corporation

Title:	Summary of [AT122][033][mIAB] Usage of the mIAB cell indication (Intel)

Document for:   Discussion and Decision

# Introduction

This is the summary of post email discussion:

[AT122][033][mIAB] Usage of the mIAB cell indication (Intel)

Scope: clarify further, if possible narrow the scope even more. Identify the points for decision (for next meeting).

CB at available CB occasion

During online session, following discussion and agreement are captured in Chari’s Note:

DISCUSSION

-	QC asks what is the scenario.

-	Intel think that a UE has high mobility state, meaning that he is not on the mobility IAB cell already.

-	Intel think no signalling is needed for a UE to understand that he is on-board ..

-	Xiaomi are wondering about the scenario.

-	CATT think it is important to help the UE stay on the mIAB cell. Intel assumes this is done by current cell res parameters. ZTE agrees, and think that a UE should not look for other freq. LG agrees and think that the mobility state is not needed. Sony also agrees and think for inbund mobility we can discuss more ..

R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state.

Following two phases are considered for this email discussion:

- Phase 1: 25th May, 1600 KST

- F2F in Brk3: 25th May, 1630 - 1700 KST

- Phase 2: 26th May, 0830 KST

Companies providing input to this email discussion are invited to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Intel | Ziyi Li | Ziyi.li@intel.com |
| Qualcomm | Georg Hampel | ghampel@qualcomm.com |
| Nokia | Andrew Lappalainen | andrew.lappalainen@nokia.com |
| CATT | Yang Lu | luyang@catt.cn |
| Canon | Pascal Lagrange | pascal.lagrange@crf.canon.fr |
| Sharp | Art Ishii | ishiia@sharplabs.com |
| Kyocera | Masato Fujishiro | masato.fujishiro.fj@kyocera.jp |
| Samsung | June Hwang | June77.hwang@samsung.com |
| Xiaomi | Shukun Wang | Wangshukun3@xiaomi.com |

# Discussion

During the online discussion, companies are discussing solutions towards different scenarios. For IDLE/INACTIVE UEs, when the mobile IAB-cell is moving, two significant problems need to be solved jointly:

Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

Note that, during cell reselection without solving Problem 1, the UE should always follow existing cell (re)selection mechanism (i.e. inter-frequency prioritization and intra-frequency measurement). The UE may continue camping on the stationary cells and end up with frequent switching among multiple stationary gNB cells if mobile IAB-cell’s frequency priority is not the highest one.

Even setting mobile IAB-cell’s frequency priority as the highest one has limitations, since there’s no dedicated/reserved frequency for mobile IAB-cell. It causes overloading of the stationary cell(s) that share the same frequency as mobile IAB-cell, as all UEs will camp on it.

Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

Once the UE is camped on a mobile IAB-cell, it’s better for the UE to treat this mobile IAB-cell as highest priority and avoid switching to others.

###### Q1: Do you agree UE behavior of problem 1 and problem 2 to be jointly addressed? If no, please explain.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Nice explanation! |
| Nokia | No | We do not think problems 1 and 2 are major issues that need to be addressed. Problem 1 (UE physically on board the mIAB vehicle but not yet camped on the mIAB cell): We think this is an edge case. If the priority of the mIAB cell is the same or higher than surrounding cells, UEs would already be expected to naturally reselect onto the mIAB cell (if not when it is stationary, then at least once it starts moving based on measurement/filter criteria). Then the only issue is when the mIAB is lower priority than surrounding cells. In this case, when the vehicle is stationary UEs might stay camped on surrounding cells (but this is not really an issue). Once the mIAB starts moving, cell reselection criteria would eventually get triggered. If the vehicle is moving at a fast enough speed, it is expected that UEs would select the mIAB cell based on measurement filtering. If the vehicle is moving slowly UEs might keep reselecting surrounding, higher priority cells, but this is not really an issue (and is not solved by the proposed solutions anyway). Problem 2 (UE camped on the mIAB cell should avoid reselecting surrounding cells): This can be avoided altogether by configuration at the mobile IAB cell, e.g. configuring the mIAB cell with highest reselection priority and/or configuring cell reselection thresholds/offsets, so physically on board UEs do not reselect to neighbouring cells. Even if the UE briefly measures another cell of equal priority, we think it would not likely reselect that cell based on the relative Rx level and/or other filtering criteria. Surrounding UEs may temporarily camp on the mIAB cell (e.g. when the vehicle is stationary), but they would simply reselect back to the surrounding cells once the vehicle starts moving. In summary we think these are low-priority issues which can already be addressed sufficiently through network implementation/configuration. |
| CATT | Yes |  |
| Canon | Yes |  |
| Sharp | Yes |  |
| Kyocera | No | For Problem 1, the WID clearly stated that “No optimizations for the targeting of surrounding UEs”, so we think this is out of scope. For Problem 2, as the picture above shows, the best cell should be the mobile IAB cell from the on-boarding UE’s perspective. So, we think the existing mechanism (i.e., based on the radio condition) works well. We agree with Nokia that the mobile IAB cell can broadcast the legacy frequency priority that configures the current frequency as higher priority than other frequencies. |
| Samsung | No | We think the problem 1 is not severe because it is not highly probable for the UE to stick to only stationary cell during movement. The measured signal strength from one cell is going up and down, and once UE in mIAB node (not camped on mIAB cell) is approaching to the edge of the stationary cell which the UE is now camped on, it can find the mIAB cell strength is better, and reselect to that. But problem 2 should be resolved, because UE camped on mIAB cell can face the better signal strength from stationary neighbor cells, and camp on that cell, and this will happen frequently. Once reselection to mIAB cell happened, prioritizing this mIAB cell seems enough. |
| Xiaomi | No | Fine with problem 2. We think current ranking and frequency priority mechanism is enough. E.g. the mIAB cell will set the frequency of the mIAB with high frequency priority. For problem 1, we share same view with Kyocera. |

Based on the above problem discussion, following solutions are proposed to be down-selected:

Solution 1: HSDN prioritization mechanism for mIAB (i.e. neighbouring cells broadcasts intra-/inter-frequency mobile IAB-cell frequency and PCI range) for Problem 1 + mIAB-cell indication in SIB for Problem 2

Solution 2: HSDN prioritization mechanism for mIAB (i.e. neighbouring cells broadcasts intra-/inter-frequency mobile IAB-cell frequency and PCI range) for Problem 1 + by network implementation (e.g. mobile IAB-cell itself sets cellReselectionPriority in its SIB2 with higher absolute priority compared to neighbouring cell frequencies) for Problem 2

Solution 3: no optimization

###### Q2: Which solution do you prefer?

| Company | Solution 1/2/3 | Comments |
| --- | --- | --- |
| Qualcomm |  | I am getting lost. Let me rephrase: Solution 1: IAB-cell broadcasts mIAB-cell-type indicator in SIB1 For problem 1: NON-IAB cell broadcasts intra-/inter-frequency Mobile-IAB-specific frequency and PCI range in SIB3/4. For problem 2: IAB cell broadcasts intra-/inter-frequency Mobile-IAB-specific frequency and PCI range in SIB3/4 + mIAB-cell-type indicator in SIB 1. Solution 2: IAB-cell does not broadcast mIAB-cell-type indicator in SIB1 For problem 1: NON-IAB cell broadcasts intra-/inter-frequency Mobile-IAB-specific frequency and PCI range in SIB3/4. For problem 2: IAB cell broadcasts cellReselectionPriority in SIB2 with higher absolute priority compared to neighbouring cell frequenices. On problem 1: Same for solution 1 and 2. Allows UEs to prioritize mIAB-cells for cell reselection over stationary cells. It should only be broadcast by stationary cells close to areas where mIAB-nodes pass (bus terminal, etc) and the UE is likely to enter the vehicle. This implies that the stationary network may have to update SIB 3/4 with the arrival and departure of mobile IAB nodes. On problem 2: Solution 2 should be able to achieve the same benefits as solution 1. In summary: There is benefit to introduce mIAB-specific cell/frequency prioritization in SIB3/4. However, it does not seem obvious how the mIAB-cell-type indicator has any benefit for cell reselection. |
| Nokia | Solution 3 | In general, we think mechanisms for configuring the network (cell reselection priorities/thresholds/offsets) are sufficient to address the above problems. Beyond that, the effectiveness of the proposed optimizations/solutions are very much dependent on the scenario (including the configuration of the mIAB cell and the surrounding cells, speed of the vehicle, whether or not the mIAB is moving, etc.), and can in certain cases cause undesired effects, especially if a UEs incorrectly determines themselves to be on board. Already, UE measurements and filtering will likely direct reselection to mIAB cells when UEs are in the vehicle. When the vehicle is stationary, UEs should basically treat all cells (including fixed cells) equally as there may not be knowledge whether the UE is leaving/entering or inside/outside the vehicle. In such case, “optimizations” could cause ambiguous UE behaviour. |
| CATT | Solution 1 | Agree with QC that Solution 1 and Solution 2 are same for Problem 1. The assistance information should only be broadcast by cells close to the mIAB-node cell, on that aspect, due to the movement of mIAB-node, the stationary cells will update the broadcast information according to whether mIAB-node cells appear as neigbour cell. However, Solution 2 for Problem 2 is not feasible as will prevent all UEs camping on the mIAB-cell to reselect other cells. That means the surrounding UEs should reselect to the stationary cell but they will keep camping on mIAB cell for a while by reading SIB2 and prioritizing the mIAB cell, which is not desirable. Although RAN2 has decided not introducing any enhancement to surrounding UEs, but the surrounding UEs should not be impacted significantly anyway. |
| Canon | Solution 4 (based on Solution1) | A UE should prioritize a cell for which the following conditions are met: - Cond1: mIAB-cell indication in SIB - Cond2: The signal strength with the considered mobile IAB cell has been above a minimum threshold for a minimum period of time; Cond3: The UE is moving at a minimum speed, i.e., UE is in a mobility state. |
| Sharp |  | Problem 1: We do not understand why the UE needs to prioritize the neigbouring mobile-IAB cell. The UE may or may not be on-board in the vehicle that serves the mobile-IAB cell. In addition, we are not sure if HSDN reliably gives the on-board status. Problem 2: Agree on Qualcomm’s opinion. |
| Kyocera | Solution 3 | We think the current mechanism works for the mobile IAB scenarios, as considered in our paper. If some kind of speed-dependent cell reselection is needed, we wonder why the existing HSDN solution is not applicable (instead of introducing a new solution). |
| Samsung | Not in options listed | First we think Problem 1 is not a big one. And Problem 2 is to be solved. So, we think indication in mIAB is enough. Moreover, wonder if it is already agreed or not to assign the mIAB specific frequency in deployment, for broadcasting the mIAB cell/frequency info in neighbor and mIAB cells. |
| Xiaomi | 3 | As comments in Q1. |

During online meeting, there’s an FFS to discuss whether a further detailed condition to the IDLE/INACTIVE UEs needs to be considered during mobile IAB-cell prioritization:

FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state

Based on rapporteur’s observation, cell dwelling timer will only be useful for Problem 2 for the purpose of holding UE camped on the mobile IAB-cell, while considering mobility state would be helpful to reduce impact to surrounding users in Problem 1.

###### Q3: In Problem 1, do you agree considering mobility state of IDLE/INACTIVE UEs during mobile IAB cell prioritization (if supported)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | The mobility state is derived from the frequency of cell-reselections. Problem 1: - If the onboard UE reselects outside cells, it believes to be in high mobility state if the vehicle moves fast. Otherwise, it believes to be in normal mobility state. - However, if an outside UE only reselects outside cells, it believes to be in high mobility state if itself moves fast. Otherwise, it believes to be in normal mobility state. Since the UE does not know its own speed, and it does not know if it is inside or outside, the mobility state does not provide any valuable information. Problem 2: - If the onboard UE reselects the onboard cell, it believes to be in normal mobility state. - However, if an outside UE reselects an outside cell, it also believes to be normal mobility state if it itself doesn’t move. As for problem 1, since the UE does not know its own speed, and it does not know if it is inside or outside, the mobility state does not provide any valuable information. Note that things well become worse if you also consider scenarios where the UE reselects between a mix of mobile and stationary cells. May be because it is onboard, or may be because it resides on the side of the road with mIAB-nodes passing by. |
| Nokia | No | We think mobility state is not needed. Once the mIAB starts moving it is expected that the UE will eventually reselect to the mIAB cell (even if the priority is less than surrounding cells) once cell reselection thresholds are met and based on measurement filtering (if moving at a high enough speed, the surrounding cells will likely not meet evaluation criteria for a long enough time anyway). Besides, even with this solution, UEs can still incorrectly determine themselves to be on board (e.g. if they are in another vehicle travelling side-by-side with the mIAB node). |
| CATT | Use relative mobility status | We agree with analysis of QC that the absolute mobility state cannot be used for on-boarding judgement. The feasible on-board criteria should be that UE is moving together with the mIAB cell, thus the mobility state should refer to the relative mobility status to the mIAB cell. If UE can determine its relative speed to the mIAB cell is low enough, the UE can consider itself on-boarding of the mIAB cell. Relative mobility status can be used as on-boarding judgement for both scenarios of Problem 1 and Problem 2. Present 38.304 spec has specified the method how UE determines it’s in low mobility status by relative Rx level (to a specific cell) for relaxed measurement, thus, fulfillment of the relaxed measurement criterion for UE with low mobility can be reused: ##### 5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell. |
| Canon | Yes |  |
| Sharp | No | Agree with Qualcomm |
| Kyocera | No |  |
| Samsung | No | As QC said above, cell mobility state is based on the former visited cell count, and this cannot fit well for defining on-board UE. |
| Xiaomi | No |  |

###### Q4: In Problem 2, do you agree considering cell dwelling timer during mobile IAB cell prioritization (if supported)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | If the UE camps on the mobile IAB cell for a prolonged time, it can infer with rather high probability that it is onboard. This may not have benefits for cell re-selection but it may have benefits for other purposes. |
| Nokia | No | We do not see how a dwelling timer addresses problem 2 any better then simply setting the cell reselection configuration at the mIAB cell. The effectiveness of the dwelling timer is completely dependent on the scenario (e.g. mIAB moving/not moving, whether there are other vehicles that could be moving within close proximity of the mIAB vehicle) and requires optimization for the specific case anyway. |
| CATT | No | The “stay-on period” can be used for UE camping on the mIAB cell, but it cannot be used in case UE is not camping on the mIAB cell, i.e., the scenario for above Problem 1. |
| Canon | Yes |  |
| Sharp |  | No strong opinion now but UE behavior before and after the timer expiration needs to be clarified. |
| Kyocera | No |  |
| Samsung | No | Have the sympathy on QC’s comment, i.e., staying in long time in mIAB cell can be used for identifying on-board UE itself. But not sure if this can impact to the cell reselection procedure, where UE camped on mIAB cell, it prioritize that cell. And that’s it. |
| Xiaomi | No |  |

# For Offline Discussion

Before offline F2F discussion, five companies provide their replies. Based on the comments, rapporteur would like to propose the followings:

Proposal 1: RAN2 acknowledges following two problems need to be addressed for idle/inactive UEs:

- Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

DISCUSSION:

Nokia: thinks both problems are unnecessary as it can be achieved by current implementation. But fine with majority.

Proposal 2: For problem 1, the UE considers mobile IAB cell frequency as highest priority based on inter-frequency neighbouring cell list in SIB4 (if broadcasted). FFS cell prioritization is based on PCI range/list in SIB4 or mIAB indication in SIB1.

DISCUSSION:

CANON: should also consider mobility state also.

HW: the SIB1 is always needs to be checked after UE reads it’s a mobile IAB-cell in Whether UE can select any cell based on SIB3/4 information. SIB1 is more important in case the frequency has other non-mIAB-cells. The UE needs to check the cell is mIAB-cell then to be camped.

SS: Problem 1 is not a problem, and thinking problem 2 is more important.

CATT: if PCI range is broadcasted in SIB3/4, the UE doesn’t need to read SIB1.

Proposal 3: For problem 2, the UE considers the camped mobile IAB cell frequency as highest priority by 1) network configuring itself with highest absolute priority (e.g. mobile IAB-cell itself sets cellReselectionPriority in its SIB2 with higher absolute priority compared to neighbouring cell frequencies) or 2) by reading mobile IAB-cell indication in SIB1 (if broadcasted).

DISCUSSION:

CATT: 1st solution may have significant impact to surrounding UEs, they may prioritize mIAB-cell.

Apple: no cell prioritization

SS: clarification for 2) whether it’s frequency prioritization. Rapp: Yes.

Kyocera: what is surrounding UEs scenario

CATT: the UE that originally camped on mIAB, then get off, the mIAB moves away. The surrounding UEs that never moves into the vehicle.

Proposal 4: For intra-frequency cell (re)selection, 1) the UE reselects a mobile IAB-cell based on existing intra-frequency mechanism based on cell ranking, 2) based on SIB3 intra-frequency cell list of mobile IAB, 3) based on SIB1 mIAB-cell indication.

DISCUSSION:

SHARP: if cell list in SIB3, are they still prioritized if intra-frequency cell reselection is not optimized as P4?

Rapp: depend on the progress of P2.

CATT: whether P4 is mapped to Problem 1/2.

Summary:

During offline discussion, in the end, companies all acknowledge that both problem 1 and problem 2 need to be addressed. Companies converge in following two directions of the UE behavior:

Solution 1: UE observes SIB4 broadcast of mIAB prioritization indicator and mIAB-cell frequency, and it may prioritize mIAB-cell at this frequency if this cell broadcasts mIAB-cell type indication in SIB1.

Solution 2: UE may prioritize mIAB cell for cell reselection based on SIB4 assistance information, which includes mIAB-cell frequencies and the corresponding PCI range for each frequency. This strictly follows HSDN approach to prioritize a cell over others, no SIB1 mIAB-cell indication is needed.

To moving forward, companies during offline are ok with using the following UE behavior to address Problem 1 and 2:

To address problem 1 and problem 2, UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies.

# Conclusion

Proposal 1: RAN2 acknowledges following two problem needs to be addressed for idle/inactive UEs:

- Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

Proposal 2: A UE, that is facing problem 1 and/or 2, may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.