3GPP TSG-RAN WG2 Meeting #123	R2-2307144

Toulouse, France, August 21- 25, 2023

Agenda Item:	7.19.2

Source:	NEC

Title:	Remaining issues for Fallback behaviour for eRedcap UE

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, RAN2 agreed the basic handling of the fallback behaviour for eRedcap UE:

Agreement @RAN2#122:

- UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX.

- A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX.

- We confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

- A UE configured with Rel-18 INACTIVE eDRX will fallback to use INACTIVE RAN DRX if it is either not configured with Rel-17 INACTIVE eDRX or the cell does not allow Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX.

However, it is not clear how to capture these agreement into the specification.

# Discussion

For the following agreements reached during the last RAN2 meeting, it is important to clarify how to capture them within the specification:

Agreement-1: UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX.

Agreement-2: A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX.

Agreement-3: We confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX

Agreement-4: A UE configured with Rel-18 INACTIVE eDRX will fallback to use INACTIVE RAN DRX if it is either not configured with Rel-17 INACTIVE eDRX or the cell does not allow Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX.

For the decoupled UE support of Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX (i.e., the first agreement), it is the UE operation within its RRC INACTIVE state, which may be captured within TS 38.304.

Proposal-1: Capture the decoupled UE support of Rel-17 RRC_INACTIVE eDRX and Rel-18 enhanced eDRX in RRC_INACTIVE within TS 38.304.

For the decoupled cell support of Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX, the best place should be in TS 38.300, even though the different indications may be used within system information (to be specified within TS38.331). Otherwise, we may consider a Note within TS 38.304 to clarify no such restrictions for the cell.

Proposal-2: Capture the decoupled cell support of Rel-17 RRC_INACTIVE eDRX and Rel-18 enhanced eDRX in RRC_INACTIVE within TS 38.300.

As expressed within Agreement-3 and Agreement-4, for the UE fallback operation (i.e., UEs configured with Rel-18 enhanced INACTIVE eDRX falls back to use Rel-17 INACTIVE eDRX, or fallbacks to use INACTIVE RAN DRX), it should be captured within TS 38.304. A Note may be used in TS 38.304 to clarify that the gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

Proposal-3: Capture the UE fallback behaviour support (i.e., UEs configured with Rel-18 enhanced INACTIVE eDRX falls back to use Rel-17 INACTIVE eDRX, or fallbacks to use INACTIVE RAN DRX) within TS 38.304. A Note is used in TS 38.304 to clarify that the gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Capture the decoupled UE support of Rel-17 RRC_INACTIVE eDRX and Rel-18 enhanced eDRX in RRC_INACTIVE within TS 38.304.

Proposal-2: Capture the decoupled cell support of Rel-17 RRC_INACTIVE eDRX and Rel-18 enhanced eDRX in RRC_INACTIVE within TS 38.300.

Proposal-3: Capture the UE fallback behaviour support (i.e., UEs configured with Rel-18 enhanced INACTIVE eDRX falls back to use Rel-17 INACTIVE eDRX, or fallbacks to use INACTIVE RAN DRX) within TS 38.304. A Note is used in TS 38.304 to clarify that the gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

# Reference

- R2-2304682	Fallback behaviour for eRedcap UE

- R2-2305312	Discussion on enhanced eDRX in RRC_INACTIVE