3GPP TSG-RAN WG2 Meeting #123	R2-2307184

Toulouse, France, August 21st – August 26th, 2023

Agenda Item:	7.12.2

Source: 	Canon Research Centre France

Title:	Discussion on mobility enhancements for mobile IAB

Document for:	Discussion

# Introduction

In RAN2 #119bis, it was agreed that [2]:

- RAN2 observes that a UE could potentially consider itself onboard of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e. the UE then need to know that this is such a cell). FFS the time. FFS if this is needed. - RAN2 assumption: For the mobile IAB cell broadcasting info: 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known) FFS how this is used (might be implementation specific). - RAN2 has from the Mobile IAB WI perspective not identified any modifications to prevent the surrounding UE from accessing the mobile IAB-node, but believes that SA2 may be working on Rel-18 solutions that may be applicable (wait for SA2)

In RAN2 #120, it was agreed that [3]:

- Regarding the assumed mobile-IAB cell type indication, RAN2 assumes is may be specified if some related UE behaviour is specified.

In RAN2 #121, it was agreed that [4]:

- Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection.

In RAN2 #122, it was agreed that [5]:

- R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state. - R2 direction (solution agreements at later stage, no other directions will be considered): RAN2 acknowledges following two problems to be addressed for idle/inactive UEs: - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low. - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells. - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

In this paper, we discuss some potential enhancements for the mobility of an IAB-node, with a focus on how a UE may be considered as an onboard UE for a given mobile IAB cell.

# Discussion

## Onboard UE detection

In RAN2 #119bis, RAN2 observed that a UE could potentially consider itself onboard of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e., the UE then needs to know that this is such a cell).

In RAN2 #122, RAN2 considered that UEs can use the mIAB-cell indication to prioritize (cell and/or frequency) when the UE is camped on the mIAB cell and to further study the prioritization when the UE is not yet camped on the mIAB cell.

Still, performing the onboard UE detection based on some cell measurement may not be sufficient, as some static UE may be considered as an onboard UE for a static mobile IAB node in case the onboard monitoring time period is not long enough. For instance, the mobile phone of a person waiting for train #A may be considered as an onboard UE for train #B in case train #B remains static at the train station for a too long period.

In this respect, as an input for some further study, it was agreed in RAN2 #122 that RAN2 may also consider condition based on cell dwelling timer or Mobility state to decide on when to apply such prioritization (addressing the case 1) where the UE is camped on the mIAB cell and 2) where the UE is not camped on the mIAB cell).

Observation 1: In order to prevent wrong onboard UE detection, the mobility state of a UE should be considered when determining its onboard status.

Based on the above considerations, a UE may be detected, or may detect itself, as an on-board UE for a given mobile IAB cell once the following conditions are met:

- The signal strength with the considered mobile IAB cell remains above a minimum threshold for a minimum period of time, which may be evaluated using a cell dwelling timer;

- The UE is in a minimum mobility state.

Proposal 1a: In order to prevent wrong onboard UE detection, a UE may be considered as an on-board UE for a given mobile IAB cell once the following conditions are met:

- The signal strength (e.g., RSRP) with the considered mobile IAB node / mobile IAB cell remains above a minimum threshold for a minimum period of time;

- The UE is in a minimum mobility state during the considered period of time.

Similarly, an onboard UE having the signal strength (e.g., RSRP) with its serving mobile IAB cell being less than a minimum threshold for a minimum time period may be considered as a surrounding UE (i.e., the UE is no longer considered as an onboard UE).

Proposal 1b: A UE is no longer considered as an onboard UE if the signal strength (e.g., RSRP) with the mobile IAB cell associated to the vehicle it used to be on-board remains below a minimum threshold for a minimum period of time.

TS 38.304 defines several mobility states (i.e., Normal, Medium, High) based on a number of cell reselections considered during a given time period. This could be reused as a basis for determining the mobility state of an on-board UE. However, the number of cell reselections may not be a relevant parameter when considering an on-board UE already connected to the mobile IAB cell associated to the vehicle it is physically on-board. In such case, it is likely that the on-board UE would remain in most cases connected to its mobile IAB cell. Thus, it would be preferable to define the UE mobility state considering the number of detected fixed cells (instead of the number of cell reselections) during a given time period.

Proposal 1c: The mobility state of an on-board UE may be determined based on the number of detected fixed cells during a given time period.

## Cell reselection / handover for onboard UEs

In RAN2 #122 meeting, RAN2 identified the need to help a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e., the UE is camped on a stationary cell) to identify a neighbour mobile IAB-cell, prioritize this mobile IAB-cell (frequency and cell) and “pull” the UE into this mobile IAB-cell. This is also referred to as “Problem 1”. Indeed, a UE moving along with a mobile IAB cell while not being connected to this mobile cell may experience some frequent handover / cell reselection which may impact the service continuity at UE level.

Observation 2: A UE that is physically on a moving vehicle but not camped / connected on its mobile IAB cell may experience unnecessary handover (for a UE in CONNECTED state) or cell reselection (in case the UE enters the IDLE/INACTIVE state).

Therefore, a UE in IDLE/INACTIVE state that determines it is onboard for a mobile IAB cell while not being camped on this mobile IAB-cell yet should perform cell resection from its current camping cell and prioritize the mobile IAB-cell.

Proposal 2a: Considering Problem 1, in case a UE in IDLE/INACTIVE state determines it is onboard for a mobile IAB cell but not camped on this mobile IAB-cell yet (i.e., the UE is camped on a stationary cell), the UE should perform cell resection from its current camping cell and prioritize the mobile IAB-cell associated to the vehicle it is onboard.

In order to prevent service discontinuity that would result from the onboard UE device camped on a mobile IAB cell disconnecting from this mobile IAB cell to transiently camp to a stationary cell (managed by a gNB or a fixed IAB node) or to some other mobile IAB cell passing by in the vicinity, an onboard UE in IDLE/INACTIVE state may no longer perform any cell reselection and associated neighbouring cell measurements as long as it remains onboard for the considered mobile IAB node. This is related to “Problem 2” identified in RAN2 #122.

Proposal 2b: Considering Problem 2, an onboard UE in IDLE/INACTIVE state that is camped to the mobile IAB-cell associated to the vehicle it is onboard, may no longer perform cell reselection and associated neighbouring cell measurements to any fixed cell as long as it determines it is an onboard UE.

In relation with both Problem 1 and Problem 2, RAN2 acknowledged in RAN2 #122 that a UE in IDLE/INACTIVE state may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. The UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

Proposal 2c: Considering problem 1 and Problem 2, an onboard UE in IDLE/INACTIVE state may limit cell reselection and associated neighbouring cell measurements to mobile IAB cells which frequency is specified in SIB4 assistance information.

Similarly, in a case where a UE determines that it is no longer an onboard UE, it may resume cell measurements and enable cell reselection.

Proposal 2d: When a UE determines that it is no longer an onboard UE, it may resume neighbouring cell measurements and enable cell reselection to any cell, either mobile of fixed, in the vicinity.

Considering now an onboard UE in CONNECTED state with a mobile IAB-cell, an IAB-donor CU should refrain from performing the handover of this UE. In case the onboard UE determination is performed at the UE level, the UE should therefore notify its onboard status to its serving IAB donor CU.

Proposal 3a: The onboard status of a UE may be determined either by the UE itself or by its serving IAB donor CU.

Proposal 3b: In case the UE determines a change in its onboard status (i.e., the UE determines it is an onboard UE or the UE determines it is no longer an onboard UE), it should notify its IAB donor CU about its new onboard status.

Upon reception of such notification from the UE or in case the IAB donor CU determines the onboard status of the UE for a given mobile IAB-cell, the IAB donor CU should perform one of the following:

- In case the onboard UE is connected to a fixed cell, the IAB donor CU should perform handover for the considered onboard UE to a mobile IAB-cell associated to the vehicle for which the UE is onboard.

- In case the onboard UE is already connected to the mobile IAB-cell, the IAB donor CU should no longer perform handover for the considered onboard UE to a fixed cell (managed by a gNB or a fixed IAB node) as long as the UE’s onboard status remains unchanged.

- In case the onboard UE is already connected to the mobile IAB-cell, the IAB donor CU should limit the handover for the considered onboard UE to the other mobile cells which mobile IAB node is mounted on the same vehicle the UE is actually onboard, as the long as the UE’s onboard status remains unchanged.

Proposal 4a: In case an IAB donor CU determines that a UE in CONNECTED state is onboard for a mobile IAB cell but not connected on this mobile IAB-cell yet (i.e., the UE is connected to a stationary cell), the IAB donor CU should perform the handover of the UE from its current serving cell to the mobile IAB-cell associated to the vehicle it is onboard.

Proposal 4b: In case an onboard UE is already connected a mobile IAB-cell associated to the vehicle it is onboard, an IAB donor CU should not perform the handover of this UE to a fixed cell (managed by a gNB or fixed IAB node) until the UE determines it is no longer onboard of the vehicle (except in the case of DU migration of the mobile IAB node).

Proposal 4c: In case an onboard UE is already connected a mobile IAB-cell associated to the vehicle it is onboard, an IAB donor CU should limit the handover of this UE to another mobile IAB-cell associated to the same vehicle.

# Conclusion

In this contribution, we discussed some potential enhancements for the mobility of an IAB-node, with a focus on how a UE may be considered as an onboard UE for a given mobile IAB cell.

The following proposals have been made:

Observation 1: In order to prevent wrong onboard UE detection, the mobility state of a UE should be considered when determining its onboard status.

Proposal 1a: In order to prevent wrong onboard UE detection, a UE may be considered as an on-board UE for a given mobile IAB cell once the following conditions are met:

- The signal strength (e.g., RSRP) with the considered mobile IAB node / mobile IAB cell remains above a minimum threshold for a minimum period of time;

- The UE is in a minimum mobility state during the considered period of time.

Proposal 1b: A UE is no longer considered as an onboard UE if the signal strength (e.g., RSRP) with the mobile IAB cell associated to the vehicle it used to be on-board remains below a minimum threshold for a minimum period of time.

Proposal 1c: The mobility state of an on-board UE may be determined based on the number of detected fixed cells during a given time period.

Observation 2: A UE that is physically on a moving vehicle but not camped / connected on its mobile IAB cell may experience unnecessary handover (for a UE in CONNECTED state) or cell reselection (in case the UE enters the IDLE/INACTIVE state).

Proposal 2a: Considering Problem 1, in case a UE in IDLE/INACTIVE state determines it is onboard for a mobile IAB cell but not camped on this mobile IAB-cell yet (i.e., the UE is camped on a stationary cell), the UE should perform cell resection from its current camping cell and prioritize the mobile IAB-cell associated to the vehicle it is onboard.

Proposal 2b: Considering Problem 2, an onboard UE in IDLE/INACTIVE state that is camped to the mobile IAB-cell associated to the vehicle it is onboard, may no longer perform cell reselection and associated neighbouring cell measurements to any fixed cell as long as it determines it is an onboard UE.

Proposal 2c: Considering problem 1 and Problem 2, an onboard UE in IDLE/INACTIVE state may limit cell reselection and associated neighbouring cell measurements to mobile IAB cells which frequency is specified in SIB4 assistance information.

Proposal 2d: When a UE determines that it is no longer an onboard UE, it may resume neighbouring cell measurements and enable cell reselection to any cell, either mobile of fixed, in the vicinity.

Proposal 3a: The onboard status of a UE may be determined either by the UE itself or by its serving IAB donor CU.

Proposal 3b: In case the UE determines a change in its onboard status (i.e., the UE determines it is an onboard UE or the UE determines it is no longer an onboard UE), it should notify its IAB donor CU about its new onboard status.

Proposal 4a: In case an IAB donor CU determines that a UE in CONNECTED state is onboard for a mobile IAB cell but not connected on this mobile IAB-cell yet (i.e., the UE is connected to a stationary cell), the IAB donor CU should perform the handover of the UE from its current serving cell to the mobile IAB-cell associated to the vehicle it is onboard.

Proposal 4b: In case an onboard UE is already connected a mobile IAB-cell associated to the vehicle it is onboard, an IAB donor CU should not perform the handover of this UE to a fixed cell (managed by a gNB or fixed IAB node) until the UE determines it is no longer onboard of the vehicle (except in the case of DU migration of the mobile IAB node).

Proposal 4c: In case an onboard UE is already connected a mobile IAB-cell associated to the vehicle it is onboard, an IAB donor CU should limit the handover of this UE to another mobile IAB-cell associated to the same vehicle.

# References

- RP-221815 - Mobile IAB (Integrated Access and Backhaul) for NR - Work Item Description

- Chairman notes of RAN2#119bis-e meeting

- Chairman notes of RAN2#120 meeting

- Chairman notes of RAN2#121 meeting

- Chairman notes of RAN2#122 meeting