3GPP TSG-RAN WG2 Meeting #123	R2-2307285

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.13.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RACH enhancement for SON

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

After RAN2#122 meeting, the following issues on RACH enhancement for SON/MDT identified in RAN2#121-bis e-meeting are still open:

- FFS: Include NSAG priority in RA report.

3b. FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1.

3c. FFS: Include S-NSSAI(s) in RA report.

This contribution discussed the identified open issues and other issues of RACH enhancement for SON.

# 2	Discussion

2.1	Discussion on network slicing related RACH enhancement for SON

In RAN2#119-bis-e meeting, RAN2 agreed that UE reports the following parameters in RA report for RACH partitioning [R2-2211101]:

- feature or feature combination that triggered the RA attempt

- the used feature combination applied during RA attempt

In RAN2#120 meeting, RAN2 agreed to include NSAG ID in RA report for RACH partitioning when the applicable feature is network slicing [R2-2300002].

According to clause 5.3.3.2 of TS 38.331 the UE only considers the S-NSSAI(s) triggering the RA attempt when at least one NSAG associated to them is included in SIB1:

1>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]): 
 
2>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

A consequence of the above procedure specification is that the NSAG-IDs that are not present in SIB1 are not relevant from RA perspective, and thus reporting about them is not meaningful from RA perspective. Note also that the UE does not know if an NSAG is used for RA purposes or only for cell reselection. Therefore, if the UE reports about NSAG IDs that are not advertised in SIB1, then it may report triggering NSAGs that are never intended to be used for RA, and thus the information provided to network is not beneficial and may be misleading.

Proposal 1: The UE reports the NSAG ID that triggered the RA attempt and belongs to the NSAG ID of the feature combination used to select the RA configuration. If no NSAG is used to select the RA configuration (e.g., no NSAG associated to the S-NSSAI(s) triggering the RA attempt is included in SIB1), then no NSAG is reported.

Regarding report of the slice information that triggered the RA attempt, it may be beneficial for the NW to identify the RA resource partitioning configuration issue in a finer granularity than NSAG ID. If the S-NSSAI(s) are reported, the network can identify which network slice or slices cause the high use of some RACH resources, and then the network can optimize the RA resource partitioning not only based on the resources configured for a specific NSAG, but also to optimize the slice grouping, e.g., by moving some network slices from one slice group to another slice group. However, if RA attempt is triggered by multiple network slices belonging to different NSAGs, the UE initiates the RA attempt only using the RA resource that corresponds to the NSAG of highest priority that is present in SIB1. In this case, the UE should not report the S-NSSAI(s) corresponding to other NSAGs as they are not used to select the RA parameters for RA attempt.

Proposal 2: UE may report the S-NSSAI(s) that triggered the RA attempt and belong to the NSAG ID of the feature combination used to select the RA configuration.

Regarding report of NSAG priority, the AMF determines the NSAG priority information per UE, i.e., different UEs may get different priority orders for the same NSAGs. According to TS23.501 it is not specified how the AMF assigns the priorities, and the AMF can take into account information like e.g., UE MM capabilities, subscribed S-NSSAIs and HPLMN. Therefore, the NSAG priority information for some specific UEs does not reflect any overall NSAG priority information in cell or network level. As RA configuration is not for a specific UE, but common for all the UEs in the cell, NSAG priority information of the reported UE is not very useful for RACH optimization from SON perspective. In addition, if UE has multiple network slices belonging to multiple NSAGs, the UE uses the highest priority NSAG to select the RA resources for performing RA and reports that NSAG ID in RA report. Therefore, the NSAG with the highest priority is implicitly reported to the NW even without explicit NSAG priority report.

Proposal 3: RAN2 to agree not include NSAG priority information in the RA report.

2.2	RA report and configuration for RA partitioning optimization

RA Report as the part of SON feature for RACH optimization is usually used by the NW to identify e.g., the need of RA reconfiguration. Thus, the NW should be able to associate the RA report with the corresponding RA configuration. As UE can log maximum 8 RA entries in the RA report, the RA entries may not always be related to the latest RA configuration. Especially when RACH partitioning feature is supported, the configuration of RACH partitioning may depend on real-time UE distribution that supports different features in the feature combination of RACH partitioning. Thus, the RA reconfiguration for different RA resource partitioning among the features and feature combination may happen more often than in the legacy scenario when RA partitioning is not supported. In case of RA reconfiguration triggered in the NW for e.g., RA partitioning, NW may not be interested in the RA reports that are related to the previous or out-dated RA configuration. To save the storage and processing overhead in the UE side as well as the signalling and processing overhead in the NW side, it’s better for the NW to control and indicate for which RA configuration (e.g., related to the last RA configuration and/or all RA reports regardless of the RA configuration)  the RA reports should be stored in the UE side and sent to the NW.

Proposal 4: RAN2 to discuss for allowing the NW to control and indicate to the UE what type of RA report (e.g., related to the last RA configuration and/or all RA reports regardless of the RA configuration) should be sent to the NW.

# 3	Conclusion

This paper contains the following proposals:

Proposal 1: The UE reports the NSAG ID that triggered the RA attempt and belongs to the NSAG ID of the feature combination used to select the RA configuration. If no NSAG is used to select the RA configuration (e.g., no NSAG associated to the S-NSSAI(s) triggering the RA attempt is included in SIB1), then no NSAG is reported.

Proposal 2: UE may report the S-NSSAI(s) that triggered the RA attempt and belong to the NSAG ID of the feature combination used to select the RA configuration.

Proposal 3: RAN2 to agree not include NSAG priority information in the RA report.

Proposal 4: RAN2 to discuss for allowing the NW to control and indicate to the UE what type of RA report (e.g., related to the last RA configuration and/or all RA reports regardless of the RA configuration) should be sent to the NW.