3GPP TSG-RAN WG2 Meeting #123		R2-2307290

Toulouse, France, 21st Aug. – 25th Aug. 2023

Source:	vivo

Title:	Discussion on Early TA Acquisition and LTM procedure

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

- Introduction

In RAN2#122 meeting, RACH-less LTM and early TA acquisition are discussed and reached a lot of agreements [1].

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. - Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization - For early TA acquisition for candidate Cells - For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). - RAN2 doesn’t see a need for a solution with RAR in for Rel-18. - Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 - The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). - We send LS to R1

In this contribution, we will further discuss the remaining issues on early TA acquisition and RACH-less LTM.

- Discussion

  - Access to target cell during LTM

When UE receives the LTM cell switch command, UE will switch to the indicated cell. According to our agreement in RAN2#119bis meeting, CFRA, CBRA, and RACH-less procedures for LTM are all supported [2].

- RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this.

Besides, in the RAN2#122 post email discussion[3], we have discussed how to differentiate the following cases:

- Case 1: RACH-based (actual TA value not needed in the MAC CE)

- Case 2: RACH-less with UE based TA measurement (actual TA value not needed in the MAC CE) , if supported based on RAN1 discussion

- Case 3: RACH-less with actual TA value provided (including the zero or same value)

Hence, the options for UE to access the target cell during LTM are as follows:

- Option 1: UE skips RACH procedure if UE has a valid TA value of the target cell at the time when LTM execution is performed or if the LTM cell switch command includes TA value information, e.g. for Case 2 and Case 3.

- Option 2: UE performs RACH to access the target cell following the CFRA procedure if the CFRA resource for LTM execution is included in the pre-configured candidate cell configuration.

- Option 3: UE performs RACH to access to target cell following the CBRA procedure if the above conditions aren’t met.

Proposal 1: UE performs LTM cell switch with the following principles:

- If  TA value is provided in the LTM cell switch commandor UE maintains valid TA value of target cell(if RAN1 decides to support UE-based TA measurement), UE skips RACH procedure while accessing the target cell;

- Else if CFRA resource is included in pre-configured RRC configuration, UE performs RACH via CFRA procedure while accessing the target cell;

- Else UE performs RACH via CBRA procedure while accessing the target cell.

  - Remaining Issue for Early TA Acquisition

### 2.1.1 Configuration for Early TA Acquisition

PDCCH order

It was agreed that PDCCH order RACH is applied for early TA acquisition. The contents in the existing PDCCH order are as follows:

Table 1: Contents in PDCCH order

| Field (Item) | Bits | Reference |
| --- | --- | --- |
| Identifier for DCI formats | 1 | Always set to 1, meaning this is for DL |
| Frequency domain resource assignment | Variable | All ones |
| Random access preamble index | 6 | 6 bits according to ra-PreambleIndex |
| UL/SUL indicator | 1 | For CFRA, to indicate which UL carrier in the cell to transmit the PRACH |
| SS/PBCH index | 6 | For CFRA, the field indicates the SSB that shall be used to determine the RACH occasion for the PRACH transmission |
| PRACH Mask index | 4 | For CFRA, the field indicates the RACH PDCCH order occasion associated with the SSB indicated by SS/PBCH index for the PRACH transmission |

In our understanding, the contents of the legacy PDCCH order are also needed for early TA acquisition. In addition, RAN1 also has the agreement that the cell indication should be introduced in the PDCCH order.

Regarding what the candidate cell identity should be included, we have the following agreement[2]:

- The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID.

We think the configuration ID of LTM candidate cell configuration could be used as the cell indicator in PDCCH order, to avoid exposing the cell ID of potential target cell.

Proposal 2: In addition to the contents in the legacy PDCCH order used to trigger RACH, the configuration ID of LTM candidate cell configuration should be included in the PDCCH order to indicate the candidate cell ID for early TA acquisition.

RRC PRACH occasion configuration

In RAN2#121bis-e meeting, we have the following agreement on RRC RACH configuration[4]:

- RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate Ies) from the candidate cell configuration (the part that need to be applied at cell switch).

Here, we further discuss the RRC RACH configuration for early TA acquisition. The contents in PDCCH order only provide the SSB index, preamble index, and the PRACH Mask index.  However, the PRACH occasion, the preamble format, and power control parameters are not provided. Hence the RRC RACH configuration must provide this information for UE for preamble transmission.

In our understanding, the following IE should be considered for RRC RACH configuration for early TA acquisition:

Table 2: RRC RACH configuration for early TA acquisition

| field in the RACH-ConfigGeneric | function |
| --- | --- |
| PRACH-ConfigurationIndex | PRACH configuration index, used to determine the PRACH time occasion |
| msg1-FDM | The number of PRACH transmission occasions FDMed in one time instance |
| msg1-FrequencyStart | Offset to lowest PRACH transmission occasion in frequency domain with respective to PRB 0 |
| zeroCorrelationZoneConfig | N-CS configuration, used to determine the preamble |
| preambleReceivedTargetPower | The target power level at the network receiver side |
| powerRampingStep | Power ramping steps for PRACH |
| SSB-perRACH-Occasion | Number of SSBs per RACH occasion |
| prach-RootSequenceIndex | PRACH root sequence index, used to determine the preamble format |
| Msg1-SubcarrierSpacing | PRACH root sequence index |

Hence, we have the following proposal:

Proposal 3: For each candidate cell, the RACH configuration for early TA acquisition pre-configured via RRC includes at least the following:

- PRACH-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- powerRampingStep

- ssb-perRACH-Occasion

- prach-RootSequenceIndex

- Msg1-SubcarrierSpacing

### 2.1.2 RACH collision for early TA acquisition

In the 38321 mobility CR [5], there is a following Editor’s note:

Editor’s note: FFS any impact pending on RAN1 further conclusion on the RACH collision/parallel transmission for PDCCH-order based PRACH for candidate cell, and pending on RAN2 further conclusion on handling of  multiple RA procedures, if any.

In the current specification, there is only one Random Access procedure ongoing at any point in time in a MAC entity. After the early TA acquisition procedure is introduced, we think the legacy principle should be re-used and no parallel RACH procedures in a MAC entity are allowed.

Proposal 4: RAN2 to confirm there is only one RACH procedure ongoing at any point in time in a MAC entity, no matter whether the RACH procedure is legacy RACH or RACH for early TA acquisition.

However, even not support parallel RACH procedures, the RACH collision is unavoidable, i.e. a new RACH procedure is triggered while another RACH procedure is ongoing. In legacy, the solution of RACH collision is up to UE implementation as the following note in specification:

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

Before we discuss how to handle RACH collision after introducing early TA acquisition, we should first discuss how UE determines RACH collision occurs if early TA acquisition is involved, i.e. how to determine the completion of early TA acquisition.

For legacy RACH procedure, we have defined explicit methods to determine whether RACH is completed, e.g. UE considers the RACH is completed if UE receives a PDCCH transmission addressed to the C-RNTI and containing a UL grant for a new transmission after Msg3 is transmitted. However, there is no RAR for early TA acquisition, UE couldn’t know whether the ongoing early TA acquisition is completed after it sends the PRACH. According to the following RAN1 agreement, a 1-bit field is introduced in PDCCH order indicating initial transmission or retransmission of PRACH.

On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter

Hence, UE could determine that the last early TA acquisition is completed when UE receives a PDCCH order indicating the initial transmission of PRACH.

However, in some cases, the PDCCH order indicating the initial transmission may missing, then a PDCCH order indicating retransmission of PRACH will be sent. In these cases, UE could determine that the last early TA acquisition for cell A is completed if UE receives a PDCCH order indicating early TA acquisition for cell B even if the PDCCH order indicates retransmission of PRACH.

In other cases, if there is no next early TA acquisition for other candidate cells triggered, UE couldn’t know whether the last early TA acquisition is completed based on the above principle. To resolve this scenario, we should introduce a timer for early TA acquisition which starts when UE sends the preamble, UE considers RACH for early TA acquisition to be completed when the timer expires.

Proposal 5: Specify the way to determine early TA acquisition procedure completion, an ongoing early TA acquisition procedure is considered completed if any of the following occurs:

- UE receives a PDCCH order for early TA acquisition procedure which indicates it is the initial transmission.

- UE receives a PDCCH order for early TA acquisition procedure which indicates a candidate cell different from the ongoing early TA acquisition procedure.

- the newly introduced timer expires, the timer is introduced for early TA acquisition which starts when UE sends the preamble.

Based on the above analysis, the network will ensure the RACH procedure for different candidate cells does not collide, i.e. the source cell won’t trigger early TA acquisition for another candidate cell while one early TA acquisition for one candidate cell is ongoing.

Observation 1: It is up to network implementation to ensure there is no RACH collision between early TA acquisition procedures.

However, since UE couldn’t predict when the UL data transmission occurs, UE-initiated RACH for UL data transmission may be triggered while one early TA acquisition procedure is ongoing, thus the RACH collision occurs in this case. In our view, the legacy principle could be re-used, i.e. it is up to UE implementation to determine which RACH procedure should be performed first.

Proposal 6: It is up to UE implementation on whether UE performs UE initiated RACH in serving cell or PDCCH order triggered RACH for early TA acquisition when RACH collision occurs.

### 2.1.3 RACH resource and determination of UE identity for early TA acquisition

RACH resource for early TA acquisition preparation

In RAN2#121bis meeting, RAN2 has agreed that the preamble resources for early TA acquisition could be shared among multiple UEs as the following agreements:

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

Observation 2: The CFRA resource for early TA acquisition(e.g., the preamble resource) could be shared among multiple UEs.

In our view, it is reasonable the shared resources are per DU allocation and could be acquired by source DU before the early TA acquisition procedure. In particular, the RACH resources for early TA acquisition of a target cell are allocated to a DU, i.e., source DU or candidate DU in case the resource will be used when the candidate DU becomes to source DU, and the shared RACH resource could be used by the UEs within the DU during the early TA acquisition to the target cell if the source DU includes the resource information in PDCCH order.

It is straightforward for intra-DU case since DU has the RACH resource of its controlled cells, additional procedures for the acquisition of the RACH resources is not needed. However, for inter-DU case, one DU doesn’t have the RACH resource of the cells belonging to another DU, hence new interactions between DUs are needed, which is up to RAN3.

Observation 3: Interactions between source DU and candidate DUs are needed for early TA acquisition.

Determination of UE identity for shared preambles resource

As we have discussed above, since the RACH resource for early TA acquisition of a candidate cell could be shared by UEs within the same source DU, the candidate cell couldn’t determine the UE identity when it receives the shared preambles from UEs within S-DU, and the source DU couldn’t determine which UE the TA value provided by candidate DU belongs to if it triggers several UEs to the same candidate cells using the shared RACH resource. Hence more information has to be interacted between source DU and candidate DU, e.g. the source DU informs the UE_ID The source DU informs the candidate DU of the UE_ID that used the shared RACH resource for early TA acquisition. and candidate DU returns the UE_ID and its TA value to source DU, or candidate DU return the used RACH resource and computed TA value to source DU, source DU determine the UE_ID based on the used RACH resource, anyway, details is up to RAN3.

Observation 4: Candidate DU couldn’t determine which UE the calculated TA value belongs to in case the UEs use the shared preambles resource to perform early TA acquisition.

Proposal 7: Send an LS to RAN3 for the shared RACH resource for early TA acquisition:

- Ask RAN3 to design the F1 procedure for shared RACH resources acquisition for inter-DU early TA acquisition case;

- Inform RAN3 of the issue on determination of UE identity for shared preambles resource, and provide the suggested solutions to RAN3 if possible.

  - LTM completion for RACH-less based LTM

In RAN2#121bis-e meeting[4], RAN2 has discussed the LTM completion issue, the agreements are as follows:

- For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

However, there is no conclusion on how UE determines the NW has successfully received its first UL data for RACH-less LTM execution. And there are three options on how UE to determine the successful reception of its first UL data by the network on the table:

- Option 1: RLC ACK of RRCReconfigurationCompete message

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution identify MAC CE

For option 1, since the delay of RLC ACK may be long and if it is longer than supervisor timer, UE may consider the LTM to have failed and the LTM recovery may be triggered. Someone may argue that the supervisor timer could consider this worse case, however, option 1 is still not efficient.

Observation 5: Using RLC ACK of RRCReconfigurationComplete message to determine whether LTM is successfully completed, UE may need to wait for a long time after sending the first UL data.

For option 2, in our view, it could work only in the following scenarios:

- Scenario 1: UE receives the C-RNTI addressed PDCCH which schedules a new UL transmission (i.e., the NDI provided in the associated HARQ information has been toggled) and the identified HARQ process is the same as the HARQ process used for the transmission of RRCReconfigurationComplete message. Only in this way， the UE can ensure the RRCReconfigurationComplete has been successfully received by the network

- Scenario 2: UE receives the C-RNTI addressed PDCCH which schedules a new DL transmission, considering the network will only start to schedule the DL transmission when it confirms that the UE has accessed the target cell,i.e. it receives the RRCReconfigurationComplete message.

In other scenarios e.g. UE receives C-RNTI addressed PDCCH which is used for schedule of first UL data, i.e. the dynamic scheduling scenario couldn’t work well, since in this case UE has not sent RRCReconfigurationComplete message, or the case that UE receives the C-RNTI addressed PDCCH which schedules a retransmission also couldn’t work well since in this case UE doesn’t successfully send the RRCReconfigurationComplete message. Hence if option 2 is applied, it has to be clarified clearly. Besides, if UE has no UL/DL data to transmit/schedule after UE sends (or NW receives) the RRCReconfigurationComplete message, the PDCCH won’t be tranmitted in time, and option 2 may be inefficient similar to option 1.

Observation 6: Using C-RNTI addressed PDCCH to determine whether LTM is successfully completed is not valid in all scenario and UE may also need to wait for a long time after sending the first UL data .

Option 3 reuses the LTE mechanism and in which the network always sends a MAC CE at every LTM cell switch procedure hence there is no delay issue. Hence propose:

Proposal 8：For RACH-less LTM of the MCG-switch case, the UE considers LTM cell switch procedure to be completed when UE receives the UE contention resolution identity MAC CE from the target cell.

  - LTM failure handling

In RAN2#121bis-e meeting, RAN2 agreed to use the RRC re-establishment as a baseline for MCG LTM failure and support fast recovery to a candidate cell at RLF or LTM execution failure [4]. We will further discuss the fast recovery mechanism in this section.

- Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

According to the current specification, when RLF/HOF occurs, the UE performs cell selection. If the selected cell is a CHO candidate, the UE may attempt CHO execution, otherwise, re-establishment is performed.

For LTM failure handling, the same mechanism as CHO could be re-used, i.e. at MCG RLF/HOF, the UE could perform RRC re-establishment. And if the selected cell is an LTM candidate cell, UE could stop the RRC re-establishment procedure and perform LTM to the selected cell.

Proposal 9：After MCG RLF/HOF, if RRC re-establishment is triggered and an LTM candidate cell is selected during the cell selection phase, UE stops the RRC re-establishment procedure and performs LTM to the selected cell.

  - Scenarios that TA=0 and TA is same as serving cell for candidate cell

In RAN2#121 bis-e meeting, we discussed whether UE could trigger RACH-less cell switch when the target cell is a current serving cell or when TA of the target cell is 0 as follows but without conclusion.

- With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - Value 0, - Value indicating that the UE shall apply the TA of one source cell.

And in RAN2#122 meeting, we agreed that when LTM cell switch MAC CE provides the TA value, e.g. UE performs early TA acquisition, UE could trigger RACH-less cell switch. In our view, this agreement doesn’t exclude using RACH-less LTM for TA value is 0 or same as source cell scenarios.

The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

Besides, in last RAN1 meeting, RAN1 also had the related agreements:

From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or keeping the same value as source cell in cell switch command.

Based on the above agreements, it is reasonable that TA value to be 0 is included in LTM cell switch command, i.e. even UE doesn’t perform early TA acquisition to a target cell but if the target cell is small, source cell could also include TA=0 in cell switch command.

Proposal 10: Targe cell with TA=0 should be supported via RACH-less cell switch without early TA acquisition, i.e. UE could skip RACH when TA=0 is included in LTM cell switch command, even early TA acquisition has not been performed towards to target cell.

Regarding the scenario that target cell has the same TA value of source cell, LTM should also support it as a RACH-less scenario without early TA acquisition since if UE has maintained the valid TA value of target cell, there is no need for UE to perform RACH naturally when cell switch.

As for how to include the value indicating that the UE shall apply the TA of serving cell in LTM cell switch command, in LTE, it uses the current TAG to indicate the TA value of the target cell, detail is as follows:

And for NR LTM, the following options could be considered:

- Option 1: exact TA value is included in LTM cell switch command

- Option 2: Similar to LTE, source TAG indication(i.e. MCG-pTAG/SCG-pTAG/MCG-sTAG/SCG-sTAG) is included in LTM cell switch command

In our view, option 1 couldn’t work, since the exact TA value is maintained by UE.  And UE may adjust the TA value to adapt to DL synchronization changes and the gNB couldn’t know the TA value accurately in this case. Hence, option 2 is preferred.

Since LTM only involves one cell group, it is not likely for UE to apply the TA of source SCG-pTAG or SCG-sTAG to target PCell. Similarly, it is not like for UE to apply the TA of source MCG-pTAG or MCG-sTAG to target PSCell. Hence, option 2 can be improved as follows: pTAG/sTAG may be included in LTM cell switch command to indicate UE to apply the same TA value of source pTAG/sTAG to target SpCell. Hence we propose:

Proposal 11: When target cell has the same TA value as source cell, UE could skip RACH for LTM cell switch. Source TAG indication (i.e. pTAG or sTAG), instead of exact TA value, may be indicated in LTM cell switch command. The source TAG indication informs UE to apply the TA of one of the source TAGs to the target SpCell. The source TAGs are related to the MAC entity which receives the LTM cell switch command.

  - LTM in NR-DC scenario

Due to limited time to complete the WI, some companies propose to deprioritize the LTM for SCG in Rel-18 mobility. What’s more, some companies further propose that Rel-18 LTM is only applied to non-DC UE, i.e. network needs to ask DC UEs to release SN before configure LTM to the UE.

In current specification, intra-MN PCell change without SN change is allowed. Considering the Rel-18 LTM is within the same CU, hence the PCell change via LTM is always within the same MN.  In our view, the SCG could be kept during the PCell change just as current L3 handover and current specification could be re-used as a baseline.

Proposal 12: Rel-18 LTM is applicable to both SA and DC UE for intra-MN PCell change even if RAN2 agrees to deprioritize the SCG LTM in Rel-18.

  - RAN3 issue on LTM trigger entity

RAN 3 sent an LS to RAN2[6] on the following two approaches to support inter-DU LTM cell switch during execution.

RAN3 has discussed the following two approaches to support inter-DU LTM cell switch during execution. Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE. RAN3 would like to get feedback from RAN2 about the above-mentioned approaches, and provide suggestion if there is any other possibility identified.

Apporach1 requires no information exchange between source DU and target DU before LTM triggering, which is faster than Approach 2 and leads to shorter handover latency than Approach 2.

In our view, the serving gNB-DU has all the information to trigger inter-DU LTM and no need to request the target DU before triggering the LTM cell switch. Hence, Apporach1 is feasible. Moreover, when the serving gNB-DU has determined to trigger cell switch, it means the signal quality of the current serving cell is not good, and UE should perform cell switch as soon as possible to avoid potential RLF. However, approch2 leads to a higher RLF rate compared with apporch1, hence Approach 2 is not preferred.

Proposal 13: For inter-DU LTM, RAN2 only supports that source DU decides by itself whether to trigger LTM, i.e. approach 1 in RAN3 LS.

- Conclusion

In this contribution, we discuss the early TA acquisition and LTM procedures. We also discuss the NR-DC scenario and other RACH-less scenario and impacts to/expectations of other groups. Based on the discussion, we have the following observation and proposals.

The observations are as follows:

Observation 1: It is up to network implementation to ensure there is no RACH collision between early TA acquisition procedures.

Observation 2: The CFRA resource for early TA acquisition(e.g., the preamble resource) could be shared among multiple UEs.

Observation 3: Interactions between source DU and candidate DUs are needed for early TA acquisition.

Observation 4: Candidate DU couldn’t determine which UE the calculated TA value belongs to in case the UEs use the shared preambles resource to perform early TA acquisition.

Observation 5: Using RLC ACK of RRCReconfigurationComplete message to determine whether LTM is successfully completed, UE may need to wait for a long time after sending the first UL data.

Observation 6: Using C-RNTI addressed PDCCH to determine whether LTM is successfully completed is not valid in all scenario and UE may also need to wait for a long time after sending the first UL data .

The proposal are as follows:

Access to target cell during LTM

Proposal 1: UE performs LTM cell switch with the following principles:

- If  TA value is provided in the LTM cell switch commandor UE maintains valid TA value of target cell(if RAN1 decides to support UE-based TA measurement), UE skips RACH procedure while accessing the target cell;

- Else if CFRA resource is included in pre-configured RRC configuration, UE performs RACH via CFRA procedure while accessing the target cell;

- Else UE performs RACH via CBRA procedure while accessing the target cell.

Early TA Acquisition

Proposal 2: In addition to the contents in the legacy PDCCH order used to trigger RACH, the configuration ID of LTM candidate cell configuration should be included in the PDCCH order to indicate the candidate cell ID for early TA acquisition.

Proposal 3: For each candidate cell, the RACH configuration for early TA acquisition pre-configured via RRC includes at least the following:

- PRACH-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- powerRampingStep

- ssb-perRACH-Occasion

- prach-RootSequenceIndex

- Msg1-SubcarrierSpacing

Proposal 4: RAN2 to confirm there is only one RACH procedure ongoing at any point in time in a MAC entity, no matter whether the RACH procedure is legacy RACH or RACH for early TA acquisition.

Proposal 5: Specify the way to determine early TA acquisition procedure completion, an ongoing early TA acquisition procedure is considered completed if any of the following occurs:

- UE receives a PDCCH order for early TA acquisition procedure which indicates it is the initial transmission.

- UE receives a PDCCH order for early TA acquisition procedure which indicates a candidate cell different from the ongoing early TA acquisition procedure.

- the newly introduced timer expires, the timer is introduced for early TA acquisition which starts when UE sends the preamble.

Proposal 6: It is up to UE implementation on whether UE performs UE initiated RACH in serving cell or PDCCH order triggered RACH for early TA acquisition when RACH collision occurs.

Proposal 7: Send an LS to RAN3 for the shared RACH resource for early TA acquisition:

- Ask RAN3 to design the F1 procedure for shared RACH resources acquisition for inter-DU early TA acquisition case;

- Inform RAN3 of the issue on determination of UE identity for shared preambles resource, and provide the suggested solutions to RAN3 if possible.

LTM completion

Proposal 8：For RACH-less LTM of the MCG-switch case, the UE considers LTM cell switch procedure to be completed when UE receives the UE contention resolution identity MAC CE from the target cell.

LTM failure handling

Proposal 9：After MCG RLF/HOF, if RRC re-establishment is triggered and an LTM candidate cell is selected during the cell selection phase, UE stops the RRC re-establishment procedure and performs LTM to the selected cell.

Scenarios that TA=0 and TA is same as serving cell for candidate cell

Proposal 10: Targe cell with TA=0 should be supported via RACH-less cell switch without early TA acquisition, i.e. UE could skip RACH when TA=0 is included in LTM cell switch command, even early TA acquisition has not been performed towards to target cell.

Proposal 11: When target cell has the same TA value as source cell, UE could skip RACH for LTM cell switch. Source TAG indication (i.e. pTAG or sTAG), instead of exact TA value, may be indicated in LTM cell switch command. The source TAG indication informs UE to apply the TA of one of the source TAGs to the target SpCell. The source TAGs are related to the MAC entity which receives the LTM cell switch command.

NR-DC scenario

Proposal 12: Rel-18 LTM is applicable to both SA and DC UE for intra-MN PCell change even if RAN2 agrees to deprioritize the SCG LTM in Rel-18.

RAN3 issue on LTM trigger entity

Proposal 13: For inter-DU LTM, RAN2 only supports that source DU decides by itself whether to trigger LTM, i.e. approach 1 in RAN3 LS.

- References

- R2#122 Charir Notes(MediaTek)

- R2#119bis Chair Notes (MediaTek)

- R2-2308434, Summary of [Post122][058][Mob18] Contents of Cell Switch MAC CE, Huawei, HiSilicon

- R2#121bis Chair Notes (MediaTek)

- R2-2308435, 38.321 running CR for introduction of NR further mobility enhancements, Huawei, HiSilicon

- R3-230889, LS on Approaches during execution for inter-DU LTM