3GPP TSG RAN WG2 #123											     	 	        R2-2307322

Toulouse, FR, August 21st – August 25th, 2023

Agenda Item:	7.25.4

Source:	Thales

Title:	Discussion on IMT-2020 Satellite self-evaluation for Latency and Mobility

Document for:	Discussion and decision

# Introduction

In [1, RP-230736], a new SID was approved on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite radio interface technology.

Detailed objectives of this study item include:

a)	Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc]

b)	Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Peak data rate

•	Peak spectral efficiency

•	User experienced data rate

•	5th percentile user spectral efficiency

•	Average spectral efficiency

•	Area traffic capacity

•	Latency, including user plane latency and control plane latency

•	Energy efficiency, including both network and device

•	Mobility

•	Mobility interruption time

c)	Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Connection density

d)	Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Reliability

e)	Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4]

IoT NTN will at least target self-evaluation against bullets c) and e) technical requirements, and NR NTN will target self-evaluation against all technical requirements (in bullets b) to e)).

This study shall start with evaluating features that are supported by Rel-17 NTN (NR NTN + IoT NTN), as relevant for the above aspects.

According to the original work plan [4] at RAN2#122, RAN2 to start discussion on evaluation assumptions for RAN2 related performance requirements, e.g. latency and mobility interruption time.

The following agreements have been adopted during RAN2#122 meeting:

Agreements:

- RAN2 will perform the evaluations of user plane latency, control plane latency, and mobility interruption time.

- Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED.

- Evaluate the control plane latency based on the 2-step RACH.

- For user plane latency evaluation, HARQ disabling should be assumed.

- Evaluate the mobility interruption in beam mobility

- Confirm 0 ms mobility interruption time is achieved by NR in beam mobility

- For RTD we consider the same scenario as considered by RAN1

# Defining evaluation assumptions

The following table is used in RAN1 as the starting point for defining evaluation assumptions:

| Reference number | Characteristic for evaluation | High-level assessment method | Requirement description in ITU-R M.2514 | Usage Scenario | Needed assumptions |
| --- | --- | --- | --- | --- | --- |
| #1 | Peak data rate | Analytical | § 7.2.1 | eMBB-s | Yes (modulation, #layers, etc) (NOTE 1) |
| #2 | Peak spectral efficiency | Analytical | § 7.2.2 | eMBB-s | Yes (modulation, #layers, etc) (NOTE 1) |
| #3 | User experienced data rate | Simulation and Analytical | § 7.2.3 | eMBB-s | Derived from #4 |
| #4 | 5th percentile spectral efficiency | Simulation | § 7.2.4 | eMBB-s | Yes |
| #5 | Average spectral efficiency | Simulation | § 7.2.5 | eMBB-s | Yes |
| #6 | Area traffic capacity | Simulation and Analytical | § 7.2.6 | eMBB-s | Derived from #5 (May need discussion on how to compute the area) |
| #7 | User plane latency | Analytical and Inspection | § 7.2.7.1 | eMBB-s | NOTE 2 |
| #8 | Control plane latency | Analytical and Inspection | § 7.2.7.2 | eMBB-s | NOTE 2 |
| #9 | Connection density | Simulation | § 7.2.8 | mMTC-s | Yes |
| #10 | Energy efficiency | Inspection | § 7.2.9 | eMBB-s | No |
| #11 | Reliability | Simulation | § 7.2.10 | HRC-s | Yes |
| #12 | Mobility | Simulation | § 7.2.11 | eMBB-s | Yes |
| #13 | Mobility interruption time | Analytical | § 7.2.12 | eMBB-s | NOTE 2 |
| #14 | Bandwidth | Inspection | § 7.2.13 | N/A | No |
| NOTE 1: How to determine the appropriate parameters (MCS, bandwidth, etc.) may be subject to evaluations. NOTE 2: To be evaluated by RAN2. RAN2 may need to develop assumptions for these metrics. RAN1 can provide input on aspects such as UE and gNB processing time. |  |  |  |  |  |

Table 1: IMT-2020 self-evaluation assumptions

Based on the above table and agreements from RAN2#122 meeting [2], the following technical performance requirements will be evaluated in RAN2 and discussed in this contribution:

- The latency

  - The control plane latency related to 2-steps RACH procedure

  - The user plane latency in both downlink and the uplink

- The mobility interruption time

  - The beam mobility interruption time

# Latency

Satellite-related latency

For both control and user planes, the space segment introduces latency due to the propagation and satellite processing delays.

Propagation delay

The distances between the satellite and the UE (i.e. the Service link distance) and between the satellite and the gateway (i.e. the feeder link) are hundreds or thousands kilometers according to the scenario (LEO, MEO or GEO) and the geometry. At the speed of light, the two links introduces several milliseconds of one-way delay, which is several times the TTI length (usually considered as propagation delay in terrestrial networks).

LEO 600 km with 90 degree elevation angle for the UE (service link) and for the Gateway (feeder link) is assumed as baseline scenario. These elevation angle minimize the distance from the ground to the satellite. For that scenario, the propagation Round-Time Delay (RTD) is equal to:

TRTDprop = 2 (TFL,UL + TFL,DL) + 2 (TSL,UL + TSL,DL)

Where:

TFL,UL : UL feeder link propagation delay, inversely, TFL,DL for DL with TFL,UL= TFL,DL

TSL,UL: UL feeder link propagation delay, inversely, TSL,DL for DL with TSL,UL = TSL,DL

t = d / cpath

Where:

t: the propagation delay in function of distance d and the average speed of light on the path cpath with cpath ≈ c in the atmosphere and near space environment.

In the scenario mentioned above, d = 600 km for both service and feeder links, therefore, t= TFL,UL= TFL,DL= TSL,UL= TSL,DL=2 ms.

Proposal 1: The propagation delay (RTD) due to service and feeder links is 8 ms for both control and user plane in the LEO-600 scenario (corresponding to 90° elevation angle).

Processing delay

In the Rel-17 architecture, the satellite is transparent, i.e. it forwards the signal without perform any processing on it. The processing delay related to the satellite corresponds to the routing of the signal from the feeder receiving antenna to the service link transceiver and inversely and its amplification.

Proposal 2: The satellite on-board processing delay in transparent mode can be considered negligible (i.e. << 1ms).

## Control plane latency

The CP latency is defined in [3] as follows:

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). Control plane latency should be equal to or less than 40 ms. This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the control plane latency. Table 2 in §7.2.5 of Report ITU-R M.2412 provides an example of the elements in the calculation of the control plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 1.15 s, and may operate with a range of relevant satellite orbits.

RAN2 evaluates control plane latency from RRC_INACTIVE to RRC_CONNECTED step based on the 2-step RACH.

Depending to the satellite altitude, the propagation RTD could be more than the requirement of 40 ms. Therefore, the propagation delays should not be included in the time budget to satisfy the control plane latency requirement.

Proposal 3:  Feeder and service links propagation delays are not included in the time budget to satisfy the control plane latency requirement.

For the analysis of NR CP latency, the following control plane procedure used for NR can be also used for NR NTN:

Figure 2: Control Plane procedure latency for 2-Step Random Access Procedure

By considering the different components of CP procedure in the above diagram, we have the following proposal:

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1 TTI) and L1 encoding of RRC Resume Request | 0 + NT,1+NT,2+0.5 ms |
| 2 | Transmission of Msg A (see NOTE 1, 2) | Length of the preamble according to the PRACH format as specified in TS 38.211 |
| 3 | Preamble detection and processing in gNB (L2 and RRC) | Tproc,2 (NOTE 3) |
| 4 | Transmission of Contention resolution response (see NOTE 1, 2) | Ts (the length of 1 slot) + link latency |
| 5 | UE Processing Delay (RRC Resume Complete) | NT,1+NT,2+0.5 ms |
|  | Total delay [ms] | 12.3 |
| NOTE 1: Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 1. NOTE 2: The link propagation delay depends on the satellite orbit as explained in §3.1. The satellite processing delay is negligible according to Proposal 2. NOTE 3: gNB processing delay Tproc,2 may vary depending to the implementation and load. Tproc,2 is defined in TS38.214-§6.4 |  |  |

Table 2: Control plane procedure latency

We made the following notes on the different steps/inputs in Table 2:

- Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 1.

- For step 1, the procedure for transition from a most “battery efficient” state haas yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. The latency of NT,1+NT,2+0.5ms is used according to Section 8.3 of TS 38.213. NT,1 is a time duration of N1 symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 or 2 when additional PDSCH DM-RS is configured; and NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1 or 2. The value of N1 and N2 are shown in Table 5.3-1 and Table 6.4-1 of TS38.214, respectively..

- For step 3, the processing delay in gNB (L2 and RRC) has been reduced to 3 ms. The delays due to inside-gNB or inter-gNB communication are not included in Step 7. Such delays may exist depending on deployment, but are not within the scope of this evaluation.

- For step 5 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0.

- For step 5 for DL data transfer, only the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of DL grant. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. The beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above

Observation 1: The control plane latency for 2-step RACH procedure is evaluated to be 12.3 ms (excluding the service and feeder link propagation delays) according to components in Table 2.

## User plane latency

The UP latency is defined in [3] as follows:

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. User plane latency should be equal to or less than 10 ms, for both downlink and uplink, assuming unloaded conditions (i.e. a single user) and small IP packets (e.g. 0 byte payload + IP header). This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the user plane latency, for both UL and DL. Table 3 in § 7.2.6 of Report ITU-R M.2412 provides an example of the elements in the calculation of the user plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 650 ms, and may operate with a range of relevant satellite orbits.

RAN2 assumes that HARQ is disabled. According to the satellite altitude, the propagation delay could exceed the user plane latency requirement. Therefore propagation delays should not be included in the time budget to satisfy the 10 ms user plane latency requirement.

Proposal 4: Feeder and service links propagation delays are not included in the time budget to satisfy the user plane latency requirement.

The evaluation of NR NTN user plane latency in Downlink (DL) and Uplink (UL) is based on the procedures illustrated in the Figures 2a and 2b.

The detailed assumptions of each step are provided in Table 3 and Table 4 for downlink and uplink, respectively.

Figure 2a: Downlink (DL) user plane (UP) data transfer procedure latency

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | DL data transfer | T1 = (TBS,tx + TFA,DL) + TDL,prop + TUE,rx |  |
| 1.1 | BS processing delay | TBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2 (NOTE 3) |
| 1.2 | DL Frame alignment (transmission alignment) | TFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA is the frame alignment time within the current DL slot, up to 1 ms in FDD |
| 1.3 | DL data packet transmission (see NOTE 1, 2) | TDL,prop | TDL,prop = TFL,DL+ TSL,DL= 4ms |
| 1.4 | UE processing delay | TUE,rx The time interval between the PDSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214) |
| - | Total one way user plane latency for DL [ms] | TDL | 2.5ms + TFA + TDL,prop |
| NOTE 1: Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 2. NOTE 2: The link propagation delay depends on the satellite orbit as explained in §3.1. The satellite processing delay is negligible according to Proposal 2. NOTE 3: gNB processing delay Tproc,2 may vary depending to the implementation and load. Tproc,2 is defined in TS38.214-§6.4 |  |  |  |

Table 3: User plane DL procedure latency

1. The value is used for evaluation only; gNB processing delay may vary depending on implementation.

2. For the above case, the value of this step is Tproc,1(  =15kHz) - Tproc,1(  =30kHz)/2.

3. For the above case, the value of this step is Tproc,2(  =15kHz)/2.

Observation 2: The DL user plane latency with HARQ and ARQ disabled is evaluated to be 3 ms (excluding the service and feeder link propagation delays) according to components in Table 3.

While the HARQ mechanism is disabled for retransmission handling during the evaluation and we put as assumption that the radio interface is error-free thanks to an ideal orientation and a robust transmission scheme for instance (p=0). But for a realistic p=0.1, the RLC ARQ mechanism could be used for residual loss after PHY and MAC as represented in Figure 2a. In that case, the RLC ARQ latency for loss recovery should be included in the total downlink user plane latency. For that, the error rate at the RLC layer and the RLC ARQ timer configurations should defined.

Proposal 5: FFS the impact of the RLC ARQ retransmission on the DL user plane latency.

Figure 2b: Uplink (UL) UP data transfer procedure latency

| Step | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | UL data transfer | T1 = (TUE,tx + TFA,UL) + TUL_duration + TBS,rx |  |
| 1.1 | UE processing delay (see NOTE 1) | TUE,tx The time interval between the data is arrived, and packet is generated; | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214) |
| 1.2 | UL Frame alignment (transmission alignment) | TFA,UL It includes frame alignment time, and the waiting time for next available UL slot | TFA,UL is the frame alignment time within the current UL slot, up to 1 ms in FDD |
| 1.3 | UL data packet transmission (see NOTE 2, 3) | TUL,prop | TUL,prop = TFL,UL+ TSL,UL= 4ms |
| 1.4 | BS processing delay | TBS,rx The time interval between the PUSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214) |
| - | Total one way user plane latency for UL [ms] | TUL= T1 | 2.5 ms + TFA,UL + TUL,prop |
| NOTE 1: It is assumed that UE has a grant to transmit data at this moment. Otherwise, SR transmission opportunity, SR transmission, BS SR and UL grant processing, and transmission of UL grant should be added. NOTE 2: Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 2. NOTE 3: The link propagation delay depends on the satellite orbit as explained in §3.1. The satellite processing delay is negligible according to Proposal 2. |  |  |  |

Table 4: User plane UL procedure latency

Observation 3: The UL user plane latency with HARQ and ARQ disabled is evaluated to be 3 ms (excluding the service and feeder link propagation delays) according to the components in Table 4.

Such as the downlink case above, the HARQ mechanism is and we put as assumption that the radio interface is error-free (p=0). But for a realistic p=0.1, the RLC ARQ mechanism could be used for residual loss after PHY and MAC as represented in Figure 2b. In that case, the RLC ARQ latency for loss recovery should be included in the total uplink user plane latency.

Proposal 6: FFS the impact of the RLC ARQ retransmission on the UL user plane latency.

Proposal 7: Adopt the following evaluation results for NR NTN user plane latency (in Table 5)

| UP Latency | RIT | Test environments | Req. (Rep ITU-R M.2514-0) | evaluation results (ms) (excluding propagation delays) | evaluation results LEO-600 90° elevation angle (ms) (including propagation delays) |
| --- | --- | --- | --- | --- | --- |
| DL | NR | Rural-eMBB-s | 10 ms | 3 | 7 |
| UL | NR | Rural-eMBB-s | 10 ms | 3 | 7 |

Table 5: User plane latency

# Mobility interruption time

The mobility interruption time is defined in [3] as follows:

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any satellite and/or gateway node during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

The requirement for mobility interruption time is 50 ms.

The procedure of exchanging user plane packets with base stations during transitions shall be described based on the proposed technology including the functions and the timing involved.

The requirement value above pertains to the eMBB-s usage scenario and applies to handheld devices.

Network controlled mobility applies to UEs in RRC_CONNECTED is categorized into two types of mobility: beam level mobility (i.e. intra-cell L1/L2 mobility procedure) and cell level mobility (i.e. inter-cell L3 handover procedure) and. RAN2 agreed to evaluate the beam-level mobility.

Beam switching is based on trigger condition for a beam and the configured beam switching algorithm. Unlike the cell-level mobility, beam-level mobility is handled internally within a cell, so it consists in PHY and MAC layers procedures, avoiding L3 RRC operation. Without RRC operations, the beam switching could be made without any interruption after the beam determination. RAN2 confirms 0ms mobility interruption is achieved by NR in the nominal case (the beam failure recovery after a beam misalignment event is not considered).

The beam level mobility evaluation is sufficient to comply with the ITU-R requirements. Thus, the cell level mobility interruption time is not evaluated in FFS.

Proposal 8: The inter cell level mobility interruption time is not evaluated for the IMT-2020 Satellite self-evaluation.

# Conclusion

In this contribution we made the following observations and proposals:

Proposal 1: The propagation delay (RTD) due to service and feeder links is 8 ms for both control and user plane in the LEO-600 scenario (corresponding to 90° elevation angle).

Proposal 2: The satellite on-board processing delay in transparent mode can be considered negligible (i.e. << 1ms).

Proposal 3:  Feeder and service links propagation delays are not included in the time budget to satisfy the control plane latency requirement.

Observation 1: The control plane latency for 2-step RACH procedure is evaluated to be 12.3 ms (excluding the service and feeder link propagation delays) according to components in Table 2.

Proposal 4: Feeder and service links propagation delays are not included in the time budget to satisfy the user plane latency requirement.

Observation 2: The DL user plane latency with HARQ and ARQ disabled is evaluated to be 3 ms (excluding the service and feeder link propagation delays) according to components in Table 3.

Proposal 5: FFS the impact of the RLC ARQ retransmission on the DL user plane latency.

Observation 3: The UL user plane latency with HARQ and ARQ disabled is evaluated to be 3 ms (excluding the service and feeder link propagation delays) according to the components in Table 4.

Proposal 6: FFS the impact of the RLC ARQ retransmission on the UL user plane latency.

Proposal 7: Adopt the following evaluation results for NR NTN user plane latency (in Table 5)

Proposal 8: The inter cell level mobility interruption time is not evaluated for the IMT-2020 Satellite self-evaluation.

# References

- RP-230736 - New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology, Ericsson, Qualcomm, Thales, MediaTek Inc.

- R2-2306541 Report from Break-out session on NR-NTN and IoT-NTN, 3GPP TSG-RAN WG2 Meeting #122

- Report ITU-R M.2514 (09/2022) - Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020 – PDF, Word

- R1-2302871: SI work plan for Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology, Rapporteur (Ericsson)

- R1-2306083: Feature lead summary #1 on evaluation methodology for IMT-2020 Satellite

# Agreements

RAN1#113

R1-2306083 Feature lead summary #1 on evaluation methodology for IMT-2020

Agreements:

The following working assumption is confirmed:

For peak spectral efficiency and peak data rate parameters:

- The parameters are chosen based on “ideal conditions”: 90degree elevation angle, 0dB atmospheric loss, 0dB shadow fading margin, 0dB scintillation loss, 0dB polarization loss, 0dB additional losses.

- Companies to provide in RAN1#113 realistic parameters, declaring the assumptions and evaluations leading to those parameters.

For uplink peak spectral efficiency and peak data rate:

- A bandwidth of 8 RBs is used.

- The peak data rate is [2.22-2.65]Mpbs

- The peak spectral efficiency is [1.54-1.84]bps/Hz

- This value is achieved with an uplink SNR of [7.05-7.08]dB

- FFS: Other assumptions to be included in the TR.

For downlink peak spectral efficiency and peak data rate:

A transmission bandwidth of 160 PRBs out of a channel bandwidth of 30 MHz is used. Channel bandwidth of 30 MHz is used as denominator for calculation of spectrum efficiency.

- The peak data rate is [104.58-118.9]Mbps

- The peak spectral efficiency is [3.67-4.13]bps/Hz

- This value is achieved with an uplink SNR of [16.91-16.95]dB

- FFS: Other assumptions to be included in the TR.

For evaluation parameters, the following table is agreed:

| Scenario | Parameter | Moderator proposal |
| --- | --- | --- |
| LLS connection density mMTC | TBS for NR | To be reported |
| SLS mMTC | Data transmission procedure | EDT/RRC Resume for eMTC/NBIOT, SDT/RRC Resume for NR |
| LLS reliability | Simulation bandwidth PUSCH | To be reported |
|  | Simulation bandwidth PDSCH | To be reported |
|  | MCS PUSCH | To be reported |
|  | MCS PDSCH | To be reported |
|  | # repetitions PUSCH | To be reported |
|  | # repetitions PDSCH | To be reported |
|  | Waveform | DFT-S-OFDM for UL, OFDM for DL. |
| LLS mobility | Simulation bandwidth | To be reported |
|  | TBS | 256 |
|  | # repetitions | To be reported |
| SLS eMBB | Number of UE receive antennas | (1,1,2) as baseline, (1,2,2) can be additionally provided |

For FRF3, 30MHz is the total maximum bandwidth (i.e., each beam will have 10MHz maximum bandwidth).

Companies are encouraged to provide calibration curves aligned with TR 38.821 calibration case 9 or case 10 (depending on whether frequency reuse factor one or three is used) for system-level simulation. Then there is no need for additional cross-company calibration.

RAN1 can further discuss the impact of simulating earth moving cell / earth fixed cell, impacts of feeder link switch, and assumptions on gateway density.

RAN1 can provide input to RAN2 (pending RAN2 progress) for the evaluation of control and user plane latency

RAN2#122

Agreements:

RAN2 will perform the evaluations of user plane latency, control plane latency, and mobility interruption time.

Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED.

Evaluate the control plane latency based on the 2-step RACH.

For user plane latency evaluation, HARQ disabling should be assumed.

Evaluate the mobility interruption in beam mobility

Confirm 0ms mobility interruption time is achieved by NR in beam mobility

For RTD we consider the same scenario as considered by RAN1