3GPP TSG-RAN WG2 Meeting #123		R2-2307347

Toulouse, France, 21st - 25th August, 2023

Source:	CATT

Title:	Discussion on power saving

Agenda Item:	7.5.3

Document for:	Discussion and Decision

Introduction

In the RAN2#122 meeting, the following agreements were made:

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications. Offline [205] (QC, Thursday CB): LS to RAN1/4 about this. Final LS in R2-2306564 Not use broadcast signalling for counter and reference SFN in XR.

In the document, we discuss the signaling details of using rational number DRX cycle and the solution of SFN wrap-around. Finally, we provide our observations and proposals.

Discussion

2.1 The signalling details of using rational number DRX cycles

In the RAN2#122 meeting, the solution of using rational number DRX cycles is approved.

The solution has the advantages of no mismatch with XR burst periods and little impact on specifications.

In [1], the companies proposed a MAC specification after applying rational number DRX cycles.

The necessary changes to TS 38.321 are shown in red in the following:

1> if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset: NOTE: If drx-ShortCycle or drx-LongCycle is not an integer, the modulo operation should be implemented by a method that does not introduce rounding errors. For example, (drx-StartOffset) modulo (drx-ShortCycle) can be implemented as drx-StartOffset – drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle). (The rest of the legacy text is omitted…)

In this TP, the same parameters drx-ShortCycle and drx-LongCycle are used for both integer and non-integer DRX cycles. That helps minimizing changes to the legacy text.

To enable that, in the field description of the new parameters we can specify that, when present, their rational number value shall be used for drx-ShortCycle and drx-LongCycle, respectively.

After converting modulo operation of the non-integer formula, the offset formula is shown as follows, which is the same as the formula in [2]:

drx-LongCycle:

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset

where n = [(SFN × 10) + subframe number]

drx-ShortCycle:

floor [n – [drx-ShortCycle × floor (n / drx-ShortCycle) ] ] =

floor [drx-StartOffset – [drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle) ] ]

where n = [(SFN × 10) + subframe number]

According to TR 38.835 [3], when XR cycle is not an integer, there are 7 kinds of frame rate, 15, 30, 45, 60, 72, 90, 120 fps corresponding to non-integer cycles of 66.66, 33.33, 22.22, 16.66, 13.88, 11.11, and 8.33 ms, respectively.

Therefore, after introducing the rational numbers for configuring DRX cycles, at least those 7 cases should be covered.

Proposal 1: The new characterization of the DRX cycle parameter in rational number should at least address the non-integer cycles corresponding to the burst rates of 15, 30, 45, 60, 72, 90, 120 fps.

In the current RRC specification [4] the characterization of drx-LongCycle is as follows:

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

A new field needs to be introduced, named drx-LongCycleStartOffset-r18 to allow configuring DRX periodicities in rational numbers.

To keep things simple, we suggest keeping the same structure for that field as in legacy: non-integer periodicity can be expressed as a fraction, and the unit is ms. For example, for a burst rate of 15 burst per second has a period of 200/3 ms, and the field drx-LongCycleStartOffset-r18 configures TwohundredThird, with unit in ms.

Proposal 2: If the DRX cycle is not an integer, the new fields drx-LongCycleStartOffset-r18 and shortDRX-r18 code for drx-LongCycle and drx-ShortCycle, respectively, a choice of ms in fractions.

And the corresponding offset value ranges up to the rounded-down value, in ms, of the DRX cycle. In other words it is coded as INTEGER(0..floor(drx-LongCycle)).

Proposal 3: When the DRX cycle is not an integer, the field drx-LongCycleStartOffset-r18 codes the drx-StartOffset as INTEGER(0..floor(drx-LongCycle)).

Therefore, the new drx-LongCycleStartOffset-r18 field could look as follows (only XR rates are captured below but the range of rates could be more generic):

drx-LongCycleStartOffset-r18            CHOICE {

msTwohundredThird                                INTEGER(0..66),

msOnehundredThird                                INTEGER(0..33),

msTwohundredNinth                                INTEGER(0..22),

msFiftyThird                                        INTEGER(0..16),

msOnehundredtwentyfiveNinth                        INTEGER(0..13),

msOnehundredNinth                                   INTEGER(0..11),

msTwentyfiveThird                                   INTEGER(0..8),

},

2.2 SFN wrap-around

The current DRX formulas to start the drx-onDurationTimer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle: [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle).

Hence, DRX on-duration start time is based on SFN and subframe number. The range of SFN is 0-1023 while the range of subframe number is 0-9. Hence, [(SFN × 10) + subframe number] is repeated every 10,240ms, i.e. every time SFN=0 again. However, the periodicities of XR traffic are not factors of 10240ms. With the above formulas, the time when the SFN value wraps around does not match the traffic arrival time.

There are two possible solutions to the SFN wrap-around issue, H-SFN and E-SFN.

The introduction of E-SFN requires many modifications to multiple specifications. H-SFN seems to be a straightforward method, it reuses existing H-SFN parameters which introduces less specification impact. However, this is based on the use of broadcast in SIB1 for eDRX, and considering using broadcast signalling was excluded, the H-SFN procedure would also need to be changed to use dedicated signaling, which would impact the specifications.

Therefore, both of H-SFN and E-SFN can impact the specification.

Observation 1: Both of H-SFN and E-SFN can impact the specification.

In addition, E-SFN can completely solve the issue of SFN wrap-around.

On the contrary the solution based on H-SFN could still have a wraparound issue in some cases, e.g., UE may be configured DRX when the value of H-SFN is near to the maximum value of H-SFN. The range of H-SFN is 0~1023. When the value of H-SFN reaches its maximum value, the mismatch from DRX cycle will cause H-SFN wraparound issue.

Considering that H-SFN may not be suitable for any DRX cycle, E-SFN is proposed. E-SFN is a newly defined variable whose value range can be defined by the frame rate. For example, the range can be defined as 0-99 or 0-999.

The DRX cycle is related to the frame rate of XR, as follows, fps is frames per second:

DRX cycle=1000fps(ms)

When the range of E-SFN is 0-99, the maximum value of E-SFN is 99. If a wrap-around occurs, the data transmission time need to be 10240*(99+1)=1024000ms. It is completely divisible by any DRX cycle.

data transmission timeDRX cycle=10240*the maximum value of E-SFN+1DRX cycle

If the maximum value of E-SFN is 99, the result as below:

data transmission timeDRX cycle=1024*fps

As can be seen from the formula, as long as (the maximum value of E-SFN +1) is a multiple of 25, the formula for E-SFN is compatible with any DRX cycle.

E-SFN will be increased by 1 whenever SFN wrapped-around occurs, and it has the maximum value. The network provides the reference SFN, via dedicated RRC signaling, in the RRC configuration for DRX. UE can determine the maximum value of E-SFN by the value of the reference SFN. E-SFN is suitable for all of DRX cycle, and it can solve the issue of SFN wrap-around.

Observation 2: E-SFN is suitable for all of DRX cycles, and it can solve the issue of SFN wrap-around.

Therefore, we can introduce E-SFN for DRX formulae to specification:

Long DRX: [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset

Short DRX: [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)

Proposal 4: To address SFN wrap around issue, RAN2 to discuss the introduction of E-SFN in the DRX formula.

Conclusion

In this document, we find the observations and proposals as following:

Proposal 1: The new characterization of the DRX cycle parameter in rational number should at least address the non-integer cycles corresponding to the burst rates of 15, 30, 45, 60, 72, 90, 120 fps.

Proposal 2: If the DRX cycle is not an integer, the new fields drx-LongCycleStartOffset-r18 and shortDRX-r18 code for drx-LongCycle and drx-ShortCycle, respectively, a choice of ms in fractions.

Proposal 3: When the DRX cycle is not an integer, the field drx-LongCycleStartOffset-r18 codes the drx-StartOffset as INTEGER(0..floor(drx-LongCycle)).

Observation 1: Both of H-SFN and E-SFN can impact the specification.

Observation 2: E-SFN is suitable for all of DRX cycles, and it can solve the issue of SFN wrap-around.

Proposal 4: To address SFN wrap around issue, RAN2 to discuss the introduction of E-SFN in the DRX formula.

Reference

- R2-2304709, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Meta.

- R2-2207999, C-DRX enhancements for XR, MediaTek Inc.

- TR 38.835 V2.0.0, Study on XR enhancements for NR, March 2023.

- TS 38.331 V17.5.0, NR Radio Resource Control (RRC) Protocol Specification, June 2023.