3GPP TSG-RAN WG2 Meeting #123		R2-2307421

Toulouse, France, August 21st–25th 2023

Source:	CATT

Title:	Discussion on IE structure for MSG1 repetition

Agenda Item:	7.21.2

Document for:	Discussion and Decision

Introduction

In previous RAN2 meetings, preliminary agreements on MSG1 repetition were agreed [1]-[2]. In this document, we continue to discuss IE structure for MSG1 repetition.

Discussion

In RAN2#121bis-e meeting, it was agreed that:

- Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE)

Based on the agreement above, RAN2 has discussed the IE structure for MSG1 repetition.

However, the fallback procedure for Msg1 repetition with lower number to Msg1 repetition with higher number which is discussed in email [3]also has impacts on the design of the IE structure.

In order to facilitate the discussion, the options provided in email discussion are copied below:

Option 1: No fallback.

Option 2: Allow fallback

- Option 2.1: Each repetition number is treated as a separate RACH type.

- Option 2.2: All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type.

- Option 2.3: Each repetition number is treated as a separate feature and we define fallback between features

For option 1 and option 2.3, each repetition number can be treated as a separate feature. Hence, there is no need to revert the agreements made in RAN2#121bis-e meeting. However, if Option 2.2 or Option 2.3 is chosen, Msg1 repetition will be treated as one feature while each repetition number will be treated as different RACH type.

Normally, allowing fallback from lower repetition number to higher repetition is beneficial for the UEs at the cell edge, especially the UE which has reached its maximum power limitation. For the solutions provided for fallback procedure, we think Option 2.2 is simple and has less MAC impacts. Hence, Option 2.2 can be adopted as one basic solution addressing fallback procedure.

Proposal 1: Option 2.2 (all repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type) can be used as the basic solution addressing fallback from lower repetition number to higher repetition number for MSG1 repetition.

If Proposal 2 is agreed, the Msg1 repetition can be designed as one single feature. Considering that eRedCap was also defined as one single feature which means that only 3 spare bits are left in FeatureCombination, this solution also decreases the impacts on FeatureCombination design. Hence, the IE structure for MSG1 repetition can be designed following the principles below:

- Define MSG1 repetition as one feature in FeatureCombination;

- Associate one specific MSG1 repetition number to the parameters, e.g. preamble index ranges and/or RO resources in FeatureCombinationPreambles.

Proposal 2: If Option 2.2 is adopted:

-	Msg1 repetition is defined as one feature in FeatureCombination;

-	Associate one specific MSG1 repetition number to the parameters, e.g. preamble index ranges and/or RO resources in FeatureCombinationPreambles.

Conclusion

In this document, we analyse CP issues for MSG1 repetition, and we find the proposls as following:

Proposal 1: Option 2.2 (all repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type) can be used as the basic solution addressing fallback from lower repetition number to higher repetition number for MSG1 repetition.

Proposal 2: If Option 2.2 is adopted:

-	Msg1 repetition is defined as one feature in FeatureCombination;

-	Associate one specific MSG1 repetition number to the parameters, e.g. preamble index ranges and/or RO resources in FeatureCombinationPreambles.

Reference

- RAN2#121bis-e meeting report;

- RAN2#122 meeting report;

- R2-23xxxxxx, [Post122][802][R18CEenh-UP] UP open issues (ZTE)