3GPP TSG-RAN WG2 #123	Tdoc R2-2307457

Toulouse, France, August 21-25, 2023

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Discussion on SI for ATG

Document for:	Discussion, Decision

# 1	Introduction

RAN2 received an LS from RAN4 in [1] which has the following content:

RAN4 has discussed the RRM requirements for ATG UE according to the work item objectives stipulated in RP-230279. For inter-frequency cell reselection in RRC IDLE and INACTIVE states, RAN4 has agreed to define two sets (set1 and set2) of cell-reselection requirements for ATG UEs (one based on Rel-15 NR reselection requirements and other based on Rel-17 HST requirements). RAN4 has agreed that the network may indicate via system information, whether set1 or set2 requirements apply for a particular cell based on cell configuration. Actions to RAN2: RAN4 would like to respectfully request RAN2 to take the above information into account and introduce necessary signaling to indicate whether set1 or set2 requirements apply for a particular cell.

In this paper we discuss an SIB for ATG, considering the above LS and an earlier LS from RAN4 regarding the applicability of SIB19 for ATG.

# 2	Discussion

In RAN2#121 meeting RAN2 already received LS from RAN4 in [2] where RAN4 asked for applicability of SIB19 to ATG cells. RAN2 discussed the issue based on two papers submitted to that meeting and agreed as follows:

R2-2303045 Discussion on the support of Air to ground access Qualcomm Incorporated discussion Rel-18 NR_ATG-Core R2-2304088 Discussion on applicability of SIB19 for NR ATG CMCC discussion Rel-18 Both noted DISCUSSION - Ericsson think SIB19 has many other things not needed for this case, and if using SIB19 for this, a lot of explanations in the RRC TS is needed. Think that the required IE is very simple and can even be put into another SIB. QC think all info in SIB19 is optional. - Chair asks why the RAN4 request is so specific, QC think that the purpose is to reduce R2 work, think that also cell specific offset is needed. - OPPO think only Ephemeris is needed. - Huawei think BS is on the ground and we only need the location, are worried about security aspects on. Would like to postpone. - intel is ok to have BS location in SIB19 or other SIB. Think it could be per BS. - QC think that Ncell info may be needed, but this is under discussion. - CMCC also think that BS location may be sensitive, would like to ask R4 about the required accuracy. - Chair think that RAN2 can do this work as alignment work, without TU allocation - the magnitude of the work seems ok (FFS on the security concerns). RAN2 will address this and intends to find a solution for Rel-18 (SIB19 or other SIB etc).

The main argument for not to reuse SIB19 is that it is NTN system specific SIB and it is specified that an NTN cell is recognized by the UE by the existence of SIB19.

SIB19 is an NTN system specific SIB and an NTN cell is recognized by the UE by the existence of SIB19, hence it is not advisable to reuse SIB19 to a non-NTN system.

RAN2 also responded to the LS in [3] where RAN2 indicated the agreement reached “RAN2 will address this and intends to find a solution for Rel-18 (SIB19 or other SIB etc).” and asked if there was any further information that needs to be broadcasted.

In RAN2#122 the issue was not treated.

The newest LS on this chain is in [4] and has the following content:

RAN4 would like to thank RAN2 for the reply LS on the applicability of the SIB19 for ATG. RAN4 has discussed the question from RAN2 on required ATG ground station reference location accuracy and had the following agreements: - Base station is allowed to not provide the exact base station location. The exact supported accuracy according to RAN4 requirements is still under discussion and it could be sent to RAN2 if it is needed by RAN2. - RAN4 assumes the BS location accuracy does not need to be specified. - UE performs the frequency/time pre-compensation based on the indicated BS location and the actual UE location, where the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location. In addition, RAN4 would like to respectfully inform RAN2 that RAN4 has agreed to introduce the mechanism of koffset in ATG system as NTN, whether and how to configure koffset is up to network implementation. RAN4 kindly asks RAN2 to take this information into account when considering the applicability of SIB19 for ATG and design corresponding signaling for it. RAN4 kindly asks RAN2 to take the above information into account.

Our preference is to introduce a new ATG SIB and to include all ATG specific system information in that SIB. For providing the base station location the IE EphemerisInfo could be reused. This IE enables network to provide the BS location with either in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. Since the ATG BS is a ground BS, it is not assumed that it moves, hence there may not be the need to provide the epoch time. The RAN4 LS also asks to provide the k_offset as in NTN systems. The current parameter k_offset is in IE NTN-config is as follows:

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

cellSpecificKoffset

Scheduling offset used for the timing relationships that are modified for NTN (see TS 38.213 [13]). The unit of the field K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

This parameter can be provided directly as an optional parameter in the new ATG SIB together with ephemerisInfo.

Finally, as requested in LS in [1], the ATG SIB should contain an indication whether set1 or set2 requirements of cell-reselection apply.

- RAN2 to introduce a new ATG SIB which contains ephemerisInfo, k_offset and an indication whether set1 or set2 requirements of cell-reselection apply.

- Adopt TP in the appendix as baseline

- Inform RAN4 about RAN2 agreements on ATG SIB and the content.

# Conclusion

In the previous sections we made the following observations:

Observation 1	SIB19 is an NTN system specific SIB and an NTN cell is recognized by the UE by the existence of SIB19, hence it is not advisable to reuse SIB19 to a non-NTN system.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to introduce a new ATG SIB which contains ephemerisInfo, k_offset and an indication whether set1 or set2 requirements of cell-reselection apply.

Proposal 2	Adopt TP in the appendix as baseline

Proposal 3	Inform RAN4 about RAN2 agreements on ATG SIB and the content.

# References

R4-2310152 LS on signaling for NR ATG

R2-2302438	LS on applicability of SIB19 for NR ATG (R4-2303684; contact: Qualcomm)	RAN4	LS in	Rel-18	NR_ATG-Core	To:RAN2

R2-2304565 Reply LS on applicability of SIB19 for NR ATG

R4-2310058 Reply LS on applicability of SIB19 for NR ATG

# Appendix

******************START TP****************************

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SystemInformation message

-- ASN1START

-- TAG-SYSTEMINFORMATION-START

SystemInformation ::=               SEQUENCE {

criticalExtensions                  CHOICE {

systemInformation                   SystemInformation-IEs,

criticalExtensionsFuture-r16    CHOICE {

posSystemInformation-r16        PosSystemInformation-r16-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

}

SystemInformation-IEs ::=           SEQUENCE {

sib-TypeAndInfo                     SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2                                SIB2,

sib3                                SIB3,

sib4                                SIB4,

sib5                                SIB5,

sib6                                SIB6,

sib7                                SIB7,

sib8                                SIB8,

sib9                                SIB9,

...,

sib10-v1610                         SIB10-r16,

sib11-v1610                         SIB11-r16,

sib12-v1610                         SIB12-r16,

sib13-v1610                         SIB13-r16,

sib14-v1610                         SIB14-r16,

sib15-v1700                         SIB15-r17,

sib16-v1700                         SIB16-r17,

sib17-v1700                         SIB17-r17,

sib18-v1700                         SIB18-r17,

sib19-v1700                         SIB19-r17,

sib20-v1700                         SIB20-r17,

sib21-v1700                         SIB21-r17,

sibxx-v1800                         SIBxx-r18

},

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-SYSTEMINFORMATION-STOP

-- ASN1STOP

*************end TP***********************

*************start TP***********************

#### –	SIBxx

SIBxx contains assistance information for air to ground access.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r18 ::= SEQUENCE {

ephemerisInfo-r18              EphemerisInfo-r18                  OPTIONAL,  -- Need R

cellSpecificKoffset-r18        INTEGER(1..1023)                   OPTIONAL,  -- Need R

cellReselectionRequirement-r18 ENUMERATED {set1, set2}            OPTIONAL,  -- Need R

...

}

-- TAG-SIBxx-STOP

-- ASN1STOP

SIBxx field descriptions

ephemerisInfo This field provides ephemeris of the ground station either in format of position and velocity state vector or in format of orbital parameters.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for ATG, following the definitions for NTN (see TS 38.213 [13]). The unit of the field K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

cellReselectionRequirement Indicates whether set1 or set2 cell selection requirements apply for a this cell, see TS XXX.

*************end TP***********************