3GPP TSG-RAN WG2 Meeting #123	 R2-2307460

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.3

Source: 	NEC

Title:  	Discussion on shared process for MBS broadcast and unicast

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#122 meeting [1], the following agreements are made:

No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed).

When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then.

FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII)

No additional information is added to MII on top of what has been already agreed.

- 2 Discussion

- 2.1 Background

In last RAN2#122 meeting, it is agreed that no additional information such as DRX configuration, modulation order, etc., is added to MII, which means the MII report content only includes the basic information, i.e., frequency, bandwidth, subcarrier spacing (SCS). And the UE shall report the whole information when transmission of MII is initiated.

- 2.2 Considerations on MII transmission procedure

- what exact parameters should be reported

Now RAN2 has agreed that frequency, bandwidth and subcarrier spacing are included in MII message. Furthermore, the exact parameter should be specified.

Regarding frequency, the value, which indicates the carrier frequency used by the UE to receive broadcast from non-serving cell, should be ARFCN-ValueNR.

Regarding subcarrier spacing, which to be used in the bandwidth where broadcast is received from non-serving cell, should be corresponded to all NR SCS value including FR1 and FR2, e.g., 15, 30, 60, 120 kHz, etc.

Regarding bandwidth, compared to LTE ROM report mechanism, CFR is introduced in NR broadcast. Thus it is worthwhile to discuss whether the whole bandwidth of that non-serving cell, or just CFR is considered as the reported content. To our understanding, reporting CFR is more precise compared to the whole bandwidth. Furthermore, the value of bandwidth should reuse the legacy NR CFR configuration, e.g., locationAndBandwidth.

Proposal 1: The exact parameter of frequency, subcarrier spacing, and bandwidth should be:

- ARFCN-ValueNR is used for frequency;

- NR SCS value is used for subcarrier spacing;

- CFR configuration is used for bandwidth.

- whether/how to address the case where additional information cannot be read by the UE from the non-serving cell

First of all, the additional information should be explained as subcarrier spacing and bandwidth. To our understanding, how and where the UE receives broadcast from non-serving cell is up to UE implementation, if the UE cannot acquire the subcarrier spacing and bandwidth from one non-serving cell, UE actually will not receive broadcast session from this non-serving cell, MII will not be reported for this non-serving cell then.

Furthermore, if the UE is interested to receive broadcast from one non-serving cell but cannot acquire the subcarrier spacing and bandwidth from this non-serving cell. If that is the case, UE will only report frequency in MII to serving cell, but still whether network would make some modifications and whether UE can start to receive broadcast form this non-serving cell, can be up to implementation. That is to say, if the UE later can acquire the additional information (to start broadcast session), the UE will re-initiate the MII transmission. In fact, any content change could make MII transmit, and no further spec impact is introduced.

Proposal 2: The UE cannot receive broadcast session from one non-serving cell if subcarrier spacing, bandwidth of this non-serving cell is not available.

Proposal 3: It is up to implementation that whether the UE only reports frequency in MII if the UE is interested to receive broadcast from non-serving cell but no additional information is available. And once the UE acquires subcarrier spacing and bandwidth later, the MII could be (re-)initiated without specification impact.

- whether any special handling is needed when the non-serving cell updates the configuration which is relevant for MII

It is assumed that the UE is already receiving broadcast from non-serving cell. Regarding MII transmission procedure, the principle should be as long as the content from the last transmitted MII is different from the current configuration, UE will initiate MII transmission. Thus there is no special handling needed but only an initiation condition, e.g., An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon the configuration concerning the content of MII from non-serving cell is changed.

Proposal 4: The UE will initiate the MII transmission procedure upon the configuration concerning the content of MII from non-serving cell is changed.

- 2.3 Considerations after MII transmission

For the whole general MII procedure, there is still some issues that whether the serving gNB allows UE to receive broadcast session from non-serving cell or not after MII is transmitted. That is to say, if the resource collision between unicast and broadcast happens, the serving cell may refuse UE’s MII request for broadcast reception from non-serving cell (e.g., the serving cell wants to guarantee unicast service transmission). Therefore, there should be a response to let UE be aware of this situation and control UE’s behaviour on broadcast reception from non-serving cell after MII reporting.

Fig.1 serving cell response to MII

Proposal 5: Introduce a network response to control the UE of whether it is allowed to receive broadcast service from other non-serving cell after MII reporting.

Another issue is that when the UE is allowed and is receiving broadcast session from non-serving cell, but later the serving cell needs to modify the current configuration for unicast service (e.g., the subsequent RRCReconfiguration). What if resource collision between unicast and broadcast happens again? Honestly in this situation, gNB has already knew whether the resource is collided or not since MII is transmitted, but from UE side, when the UE receives this new (updated) unicast configuration, the UE will be confused about whether it should stop broadcast reception or not due to resource collision happening. For this issue, some options can be considered:

1. From UE side, specify UE behaviours upon the reception of new (updated) unicast configuration (e.g., the UE shall stop receiving broadcast from non-serving cell if resource collision happens again).

2. From Network side, explicitly indicates the resource collision between unicast and broadcast happens when a new (updated) configuration for unicast is transmitted to the UE (e.g., expect UE to stop receiving broadcast service from other gNB).

Proposal 6: RAN2 is suggested to consider how to solve resource collision between unicast and broadcast happens again upon the reception of new (updated) configuration for unicast.

- 3 Conclusions

Proposal 1: The exact parameter of frequency, subcarrier spacing, and bandwidth should be:

- ARFCN-ValueNR is used for frequency;

- NR SCS value is used for subcarrier spacing;

- CFR configuration is used for bandwidth.

Proposal 2: The UE cannot receive broadcast session from one non-serving cell if subcarrier spacing, bandwidth of this non-serving cell is not available.

Proposal 3: It is up to implementation that whether the UE only reports frequency in MII if the UE is interested to receive broadcast from non-serving cell but no additional information is available. And once the UE acquires subcarrier spacing and bandwidth later, the MII could be (re-)initiated without specification impact.

Proposal 4: The UE will initiate the MII transmission procedure upon the configuration concerning the content of MII from non-serving cell is changed.

Proposal 5: Introduce a network response to control the UE of whether it is allowed to receive broadcast service from other non-serving cell after MII reporting.

Proposal 6: RAN2 is suggested to consider how to solve resource collision between unicast and broadcast happens again upon the reception of new (updated) configuration for unicast.

- 4 References

[1] R2-2304207	Report from MBS breakout session		Dawid