3GPP TSG-RAN WG2 Meeting #123	R2-2307578

Toulouse, France, 21st – 25th of August 2023

Agenda item:	7.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further details on CHO with CPAC in Rel-18

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The Post122 discussion 057 on FFSs of the RRC running CR has led to the following outcome:

Potential easy agreement

Proposal 2(12/12): UE keeps the configuration for CHO including target MCG and candidate SCG configuration when SCG is to be released.

Proposal 8(11/12): Source MN initiates the preparation of the R18 CHO with candidate SCG(s), e.g., S-MN tells the T-MN whether it is allowed to configure candidate SCG(s).

Proposal 9(12/12): candidate MN recommends the candidate PSCells to candidate SN.

Proposals for online discussion

Proposal 3(8/12): Legacy CHO recovery mechanism applies to the configuration for CHO with candidate SCG(s). It is up to UE implementation to select which CHO configuration to use if there are multiple CHO configurations for the selected PCell.

Proposal 4(10/12): UE continues conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure.

Proposal 10(9/12): Recommendation of the candidate PSCells is based on measurement results.

Proposal 11(10/12): The evaluation of the execution conditions continues once PSCell change is triggered.

Proposals for further discussion

Proposal 1: Discuss whether selectedCondRRCReconfig-r17 can be reused to indicate the selected target SCG to the target MN,

- Option 1(6/12): selectedCondRRCReconfig-r17 is not reused.

- Option 2(3/12): selectedCondRRCReconfig-r17 is reused, assuming RAN3 defines the inter-node coordination to align the CondReconfigId between the source MN and each candidate MN.

Proposal 5: Discuss whether to reuse maxNrofCondCells-r16 for CHO with candidate SCG(s).

- Option 1(7/12): reuse the legacy value (i.e., 8)

- Option 2(4/12): Need an extension, FFS the exact value.

- Option 3(1/12): Wait for S-CPAC discussion

Proposal 6: The max number limitation is on the number of conditional reconfigurations (i.e., condRRCReconfig). Assume RAN3 will discuss how to ensure the number of conditional reconfigurations is within the maximum limitation.

Proposal 7: Discuss whether to support condEventA3 or condEventA5 for the execution conditions for candidate PSCells.

# 2	Remaining issues

In this paper, we discuss some of the most important open topics. The discussion is structured in the following way.

- Related to proposal 7, we need to clarify for which scenario which event should be used. We discuss this in section 2.1.

- Related to proposal 10, it is expected that recommendation of the candidate PSCells will be based on measurements. It is unclear how the source is aware of the target PSCell measurements once it receives the measurement report that will trigger PCell change. We investigate this issue in section 2.2.

- Related to proposal 1, the indication of selected SCG is discussed in section 2.3.1.

- Related to proposal 3, the CHO recovery mechanism is discussed in section 2.3.2.

- Related to proposal 5, the number of conditional configurations is discussed in section 2.3.3.

## 2.1	Measurement and reporting configuration for execution of CHO with candidate SCG(s)

The conditional configuration is indicated with three elements: (1) conditional configuration id, (2) execution condition and (3) RRCReconfiguration. As the Obj. 4 in the WID aims at reusing as much as possible the Rel. 17 work/existing specification on CHO enhancements, we think the aspects related to id handling or the RRCReconfiguration do not need to be enhanced. The running TS 38.331 CR already captures the additional execution condition for the candidate PSCell. Thus, we focus only on the conditional event type.

Figure 1: The execution condition configuration for CHO with candidate SCG.

- Observation 1: The candidate PSCell can be retained conditionally, while it is the same cell as the serving PSCell, this might need an enhancement to reporting configuration.

The measurement object and the reporting configuration details related to the new measurement id is as follows. The measurement object is for the UE to do measurements on the frequency band of the target PSCell. The reporting configuration depends on the target PSCell and whether UE is dual connectivity or not. The below table captures different settings.

| Scenario | SC / DC at source | Target band | Target PSCell | CondEvent ID |
| --- | --- | --- | --- | --- |
| 1 | DC | Same as serving band | Source PSCell | New event |
| 2 | DC | Same as serving band | Another PSCell in the same frequency band | A3 to compare against serving PSCells or A4 |
| 3 | DC | Different band than the serving | Multiple cells in that band | A5 to compare the serving versus a threshold or A4 |
| 4 | SC |  | Any PSCell | A4 |

Table 1: Different event configuration for different target PSCell setting

The above table discusses different conditional event configuration for target PSCell access. The table has two categories that is the existence of dual connectivity at the source and the target PSCell with respect to source PSCell.  Furthermore, the table differentiates if the target frequency band is the same as the serving PSCell. Lastly, the table also differentiates if there are multiple candidate PSCells and if the target PSCell is the current serving cell.

In scenario 1, the source PSCell coverage also extends to target PSCell. While changing the PCell the target MN may configure the UE to keep the same PSCell. In this scenario, UE needs a measurement to detect that the source PSCell is better than other cells in the same frequency band, or above a certain threshold. Currently, the events A3, A4 and A5 are allowed as part of reporting configuration for the report type condTriggerConfig. None of these events allow the evaluation of the serving PSCell. This may require a new event configuration for report type condTriggerConfig.

Observation 2: For scenario 1, where the current serving PSCell of the UE is configured as the target PSCell, a new event is needed as the execution condition of the target PSCell.

The new event should consider comparing the serving cell versus a threshold.

The A3 and A5 event has the following text in TS 38.331 to cover this option.

if the eventA3 or eventA5 is configured in the corresponding reportConfig:

5>    if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

Which enables UE to consider the serving cell as a neighbour cell. However, the same is not defined for A4 event. The existing text in TS 38.331 can be appended to cover A4 as well to enable the evaluation of serving PSCell.  Or alternatively, the new event can be similar to event A1. Event A1 currently cannot be configured for conditional configuration execution as it is mainly used for secondary cell activation.

Proposal 1:For scenario 1 down-select between:

- Option 1: Event A4 is enhanced to consider serving cell as a neighbour cell.

- Option 2: The event A1 is enhanced to be configurable for report type condTriggerConfig.

In scenario 2, the target PSCell is another PSCell than the serving PSCell in the same frequency band as serving PSCell. The existence of the serving PSCell poses a potential interference source. This means that, if the target PSCell is compared to a threshold this may not be good enough. The target PSCell needs to be better than the serving PSCell as well to avoid any issue with interference.

Proposal 2: For scenario 2, where the target PSCell is another PSCell in the same band as serving PSCell, the event A3 or A4 is used as the execution condition of the target PSCell.

In scenario 3, the target PSCell is in another frequency band than the serving PSCell. There may be also multiple cells on the target frequency band. Event A4 can be used as UE only considers the best cell in that frequency band for the event A4. Event A5 can also be used in case the network would like to trigger the CHO execution only once the throughput to be provided by target PSCell is better than the serving PSCell.

Proposal 3: For scenario 3, where the target PSCell is in another band than the serving PSCell, the event A4 or A5 is used as the execution condition of the target PSCell.

In scenario 4, the UE is in single connectivity. There is no serving PSCell to consider any relevance. Thus, the execution condition can be indicated with event A4.

The events A3 and A5 refer to the SpCell of the cell group that the measurement configuration belongs to. Thus for MCG measurement configuration, this is the PCell for the MCG. As such the A3 and A5 event related to PSCell cannot be configured if the meas ID is linked to the MCG measurement configuration. This means that to support the A3 and A5 event for the execution condition of target PSCell the new Meas ID added to the CondReconfigToAddMod may refer to SCG measurement configuration.

Proposal 4: The measurement ID for the additional execution condition for target PSCell can be mapped to MCG measurement-ID or SCG-measurement-ID.

## 2.2	Preparation of CHO with candidate SCG(s)

This section 2.2 discusses the problem of the source MN obtaining measurements for candidate PCell and candidates PSCells with different measurements reports.

### 2.2.1	Configuration and report of A3/A5 measurements for CHO preparation

Prior to preparation of CHO with SCG, the source MN can initiate a single RRC Reconfiguration procedure [3] to configure eventA3 (or eventA5) triggered measurements for the UE to detect and report candidate PCells for CHO at the frequency of the serving PCell.

Figure 2. Configuration and report of A3/A5 measurements for preparation of CHO with SCG

Measurement configuration for event-based measurements on the PCell object will only report the target-cells on the PCell frequency. Once an event is triggered, UE will only report a list of candidate PCells on the configured frequency but no candidate PSCells, if no candidate PSCell is available on the frequency of the serving cells. Also, the current reporting is limited to report only the best cell on the serving frequencies on top of reporting a list of cells configured for the MeasObj that has triggered the report.

In order to obtain measurements for more than one candidate PSCell separate report configuration needs to be configured. Even if multiple measurement object is configured to obtain separate reports.It is less probable that both events (event for PCell frequency and PSCell frequency) are met simultaneously. Thus, the source MN will have to wait for the other report if one report is triggered to collect measurements for potential Target PCells and PSCells for DC-CHO. In such scenario, MN can wait until both measurements to be available, but this will lead to delay in preparing the DC-CHO configurations.

Observation 3: Report-configuration in current measurement configuration does not allow combining the measurement-reports of PCell and PSCell in single message.

Observation 4: Preparing CHO with candidate PSCells on candidate non-serving frequencies requires several measurement configurations and reporting. It is therefore hard to guarantee that UE will report measurements for candidate PCells and candidate PSCells at the same time for MN to initiate the CHO preparation with candidate SCGs.

Proposal 5: Measurement and reporting configuration is enhanced to report multiple candidate cells at candidate PSCell frequencies at the same time as reporting the candidate target PCells.

### 2.2.2	Stage 3 details for preparation enhancement of CHO with candidate SCGs

Currently the measurement report for candidate PCell involves the report of cell list on the candidate PCell frequency and the best cell on each serving cell frequency. This might not include any candidate PSCells as such the MN may need to wait other measurement reports from the UE to obtain measurements related to candidate PSCells. Delaying of the CHO procedure may cause RLFs and should be avoided. In this section, we propose a simple way to avoid such scenarios.

#### 2.2.2.1	Change in MeasResults to report neighbouring cells on candidate frequencies

Here we propose how to improve MeasResults to allow the UE to report the best neighbouring cells on candidate frequencies which are different from the frequencies of serving cells. It is proposed to add a new information element to indicate this new behavior to the UE. When the UE sends to the source MN a single MeasurementReport message (triggered by eventA3 or eventA5) to report the measured candidate PCells for an intra-frequency CHO at the PCell serving frequency, the UE further includes measurements available on candidate cells on non-serving frequencies. Annex A.1 proposes the ASN.1 change in MeasResults to report neighbouring cells on candidate non-serving frequencies.

Upon reception of a single MeasurementReport message, the source MN may initiate the preparation of CHO with by forwarding to the target MN the UE measurements. This requires the source MN to forward the UE measurements to the target MN. Then the target MN may initiate the SN preparation for target PSCells which operate not only on serving frequencies but also on non-serving frequencies.

Proposal 6: A new information element is added in MeasResults. With this IE, a single measurement (and measurement report) on a serving frequency allows the UE to report measurements on the candidate frequency(ies) different than the frequency of the serving cells.

## 2.3	Further discussion

### 2.3.1 Indicating the selected SCG to the target MN (Proposal 1)

Currently, during CHO execution, the UE does not indicate the selected CHO configuration to the target MN. The reasoning behind this is that there is only one configuration per PCell provided to the UE.

There can be multiple PCells prepared under the same target MN, however, the radio resources for these PCells are different and depending on the UE access over the radio resources (preambles) of one PCell or another the target MN can understand that UE applied which PCell configuration.

However, in the release 18 CHO with candidate SCGs, there are multiple configurations provided to the UE for the same PCell with different candidate SCGs. This will create an ambiguity at the target MN side.

This can be solved either by,

- providing different preambles for PCell access for different candidate PSCell configurations.

- indicating in the RRCReconfigurationComplete message to the target MN.

Using a new indication following the CPC logic instead of leaving it to preamble configuration to detect the related PSCell is a structured solution. The use of separate preambles can also exhaust the preambles in a target node quickly.

Proposal 7: UE indicates the selected candidate PSCell with the RRCReconfigurationComplete message to the target MN.

### 2.3.2 CHO recovery for CHO with candidate SCGs (Proposal 3)

CHO recovery has been specified in Rel. 16 to make the re-establishment of the radio link faster after the UE detects either a Radio Link Failure (RLF) on source PCell or Handover Failure (HoF) on target PCell. Herein, the UE performs cell selection and if the selected cell is a prepared one, the UE executes the stored CHO configuration. Otherwise, the UE performs re-establishment.

If the UE selects a prepared target PCell, the UE will execute the corresponding CHO configuration which might consist of MCG and SCG parts. That is, the UE will perform random access to target PCell and configured target PSCell only based on the measurements of target PCell that are used check if target PCell is suitable or not. Thus, in this case also, the UE will perform a blind access to target PSCell which can result in a random access failure.

Observation 5: During CHO recovery, the UE does not check the measurements of the target PSCell, if configured with target PCell, which can result in random access failure to target PSCell.

Given that the random access to target PSCell is based on separate CHO and CPAC conditions in Rel. 18 (if CHO+CPC framework is effectively pursued and both conditions are met), it is reasonable to adapt the CHO recovery procedure to consider as well the measurements of target PSCell.

Proposal 8: RAN2 to discuss adaptations to CHO recovery procedure where the measurements of target PSCells are also considered for deciding whether to use CHO recovery.

CHO with CPAC was agreed as base scenario, along with the alternative of CHO only scenario. In case the UE is configured with both CHO with CPAC and CHO only configurations, the recovery procedure needs to be also specified as it is not clear which CHO configuration the UE will select (whether DC or SC CHO config will be used). Selecting CHO with CPAC configuration for recovery will be a recovery option with possibly high throughput (if the recovery is successful) whereas CHO recovery via CHO only configuration provides a method for a recovery that is more likely to be successful as it only requires one cell access. Considering that, the network should provide its preferred recovery mechanism to the UE so that the UE will consider either CHO with CPAC or CHO only configurations for CHO recovery.

Proposal 9:  When both CHO and CHO+CPAC configuration are prepared for same PCell, network should configure whether to consider CHO+CPAC configuration for CHO recovery. FFS additional criteria to select between CHO and CHO+CPAC.

### 2.3.3 Number of conditional configuration maintenance (Proposal 5)

The RAN 122 meeting has the following agreement.

P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration).

This indicates that each CHO configuration with a new candidate PSCell, will be configured to the UE as a single CHO configuration.

The Rel. 18 CHO configuration keeps the same structure as the Rel. 17 CHO configuration. The Rel. 17 CHO configuration is counted as a single conditional configuration.

Proposal 10: Each Rel. 18 CHO configuration with a candidate cell is counted as a single conditional configuration.

The source MN is aware of the number of conditional configurations configured to the UE. Until Rel. 18, a conditional handover request would only return a single conditional configuration. For this reason, there was no reason to indicate total number of conditional configuration in the handover request. In Rel. 18 as target MN may provide multiple conditional configurations with candidate SCGs, the number of conditional configurations the target can provide should be indicated to target MN by the source MN.

Proposal 11: Source MN indicates the number of conditional configurations to be provided by the target MN, in the handover request.

# 3	Conclusion

This document has made the following observations and proposals:

- Observation 1: The candidate PSCell can be retained conditionally, while it is the same cell as the serving PSCell, this might need an enhancement to reporting configuration.

Observation 2: For scenario 1, where the current serving PSCell of the UE is configured as the target PSCell, a new event is needed as the execution condition of the target PSCell.

Proposal 1:For scenario 1 down-select between:

- Option 1: Event A4 is enhanced to consider serving cell as a neighbour cell.

- Option 2: The event A1 is enhanced to be configurable for report type condTriggerConfig.

Proposal 2: For scenario 2, where the target PSCell is another PSCell in the same band as serving PSCell, the event A3 or A4 is used as the execution condition of the target PSCell.

Proposal 3: For scenario 3, where the target PSCell is in another band than the serving PSCell, the event A4 or A5 is used as the execution condition of the target PSCell.

Proposal 4: The measurement ID for the additional execution condition for target PSCell can be mapped to MCG measurement-ID or SCG-measurement-ID.

Observation 3: Report-configuration in current measurement configuration does not allow combining the measurement-reports of PCell and PSCell in single message.

Observation 4: Preparing CHO with candidate PSCells on candidate non-serving frequencies requires several measurement configurations and reporting. It is therefore hard to guarantee that UE will report measurements for candidate PCells and candidate PSCells at the same time for MN to initiate the CHO preparation with candidate SCGs.

Proposal 5: Measurement and reporting configuration is enhanced to report multiple candidate cells at candidate PSCell frequencies at the same time as reporting the candidate target PCells.

Proposal 6: A new information element is added in MeasResults. With this IE, a single measurement (and measurement report) on a serving frequency allows the UE to report measurements on the candidate frequency(ies) different than the frequency of the serving cells.

Proposal 7: UE indicates the selected candidate PSCell with the RRCReconfigurationComplete message to the target MN.

Observation 5: During CHO recovery, the UE does not check the measurements of the target PSCell, if configured with target PCell, which can result in random access failure to target PSCell.

Proposal 8: RAN2 to discuss adaptations to CHO recovery procedure where the measurements of target PSCells are also considered for deciding whether to use CHO recovery.

Proposal 9:  When both CHO and CHO+CPAC configuration are prepared for same PCell, network should configure whether to consider CHO+CPAC configuration for CHO recovery. FFS additional criteria to select between CHO and CHO+CPAC.

Proposal 10: Each Rel. 18 CHO configuration with a candidate cell is counted as a single conditional configuration.

Proposal 11: Source MN indicates the number of conditional configurations to be provided by the target MN, in the handover request.