3GPP TSG-RAN WG2 Meeting #123                                   R2-2307592

Toulouse, France, August 21-25, 2023                     Revision of R2-2305961

Agenda item:		7.22.3

Source: 			ZTE Corporation, Sanechips

Title: 	RAN2 impacts of LP-WUS in connected mode

Document for: 	Discussion and Decision

- Introduction

In RAN#97, a study on low-power Wake-up Signal and Receiver for NR has been agreed in SID [1]. And the main objectives are as following:

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

- Discussion

In RAN1#113 meeting, the agreements about LP-WUS in connected mode have been reached as below:

Agreement - In RRC CONNECTED mode, study benefit of LP-WUS over existing Rel-15, R16, and R17 power saving techniques for following functionalities: - LP-WUS with similar functionality as R16 DCP. - LP-WUS activates/resumes PDCCH monitoring when LP-WUS is received. - interaction with legacy power saving techniques, if any - other functionalities are not precluded - for evaluation - companies to report - assumption on MR sleep state when LP-WUR is monitoring LP-WUS - deep sleep, - light sleep, - micro sleep - how to activate/deactivate LP-WUS monitoring and deactivate/activate PDCCH monitoring - LP-WUS waveform - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

In the following sections, based on RAN1 agreements, we will discuss the possible usages and the corresponding RAN2 impacts of supporting the LP-WUS for UE in connected mode.

2.1 Relaxed measurement

One of the possible usages of LP-WUS in connected mode is the relaxed RLM/BFD measurement. According to the TS 38.133, the current relaxed RLM/BFD measurement based on SSB could relax the required measurement periodicity to 15* DRX cycle. The RLM/BFD measurements based on LP-WUS could achieve the desired effect of power saving in UE only if the time/frequency tracking based on LP-WUS could be guaranteed. However, according to the above agreements, whether and how RLM/BFD measurements could be offloaded to LP-WUS in connected mode hasn’t been confirmed by RAN1 and RAN4. From RAN2 perspective, RAN2 can postpone this study till it is confirmed by RAN1 and RAN4.

Proposal 1: RAN2 can postpone the study for connected mode RLM/BFD measurements based on LP-WUS till its feasibility is confirmed by RAN1 and RAN4.

Moreover, another possible usage of LP-WUS is for RRM measurement. RRM measurement results are important metrics for handover, especially for cell edge user. But, the feasibility of monitoring LP-WUS for cell edge user, e.g., whether the coverage of LP-WUS could reach to the cell edge users, is still FFS in RAN1 and RAN4. Hence, RAN2 can also postpone the study for RRM measurements based on LP-WUS till its feasibility is confirmed by RAN1 and RAN4.

Proposal 2: RAN2 can postpone the study for connected mode RRM measurements based on LP-WUS till its feasibility is confirmed by RAN1 and RAN4.

2.2 PDCCH monitoring

In DCP, it could carry multiple block number, and each number could indicate whether or not a UE monitors the PDCCH during the next occurrence of the on-duration. Since monitoring LP-WUS needs less power consumption than DCP, if LP-WUS can be used to indicate whether a UE or multiple UEs monitor the PDCCH, more UE power consumption can be saved.

Besides, considering the compatibility with the legacy processes, we could consider to jointly use LP-WUS and DCP to indicate the next available DRX cycle. For example, LP-WUS could indicate a group of UEs with the similar traffic whether monitor DCP, and the corresponding DCP could indicate the UE whether or not monitor PDCCH for the next DRX cycle. In this way, the maximum of power saving in UE could be expected.

As above, RAN2 could consider the LP-WUS together with DCP to indicate the UE whether monitor PDCCH for the next DRX cycle.

Proposal 3: RAN2 can consider to jointly use the LP-WUS and DCP to indicate whether or not UE would monitor PDCCH for the next DRX cycle.

2.3 UE assistance information

In connected mode, gNB could activate LP-WUS based on UE’s traffic and the power saving preferences. Moreover, gNB needs to transmit the LP-WUS for the UE only if UE satisfies the condition of using LP-WUS. Hence, it is essential or beneficial for gNB to acquire kind of assistance information about LP-WUS from UE, for example, whether UE desires a LP-WUS, or whether UE satisfies the condition of using LP-WUS. Different from the case of UE in idle or inactive mode, it’s feasible or easy for eNB to acquire some assistance information from a UE in connected mode.

Proposal 4: RAN2 can consider to introduce UE assistance information about LP-WUS for applying LP-WUS to UE in connected mode.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: RAN2 can postpone the study for connected mode RLM/BFD measurements based on LP-WUS till its feasibility is confirmed by RAN1 and RAN4.

Proposal 2: RAN2 can postpone the study for connected mode RRM measurements based on LP-WUS till its feasibility is confirmed by RAN1 and RAN4.

Proposal 3: RAN2 can consider to jointly use the LP-WUS and DCP to indicate whether or not UE would monitor PDCCH for the next DRX cycle.

Proposal 4: RAN2 can consider to introduce UE assistance information about LP-WUS for applying LP-WUS to UE in connected mode.

- References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September, 2022