3GPP TSG-RAN WG2 Meeting #123															R2-2307594

Toulouse, France, August 21-25, 2023

Agenda item:		7.11.2.1

Source:				Samsung

Title:					CP aspects for Multicast reception in RRC_INACTIVE

Document for:		Discussion & Decision

# 1 Introduction

This contribution discusses the following open CP issues for multicast reception in RRC_INACTIVE:

- PTM configuration aspects: Signalling approach, Indication for deactivated session?

- State transition aspects: Reception quality based threshold, Cell selection, Inactivity handling

- Multicast reception and SDT aspects: Paging monitoring, SDT failure handling

- Notification and group paging aspects: MCCH change notification, Deactivation, Release, Group paging

- Mobility and service continuity aspects: NCL, Frequency prioritization/de-prioritization

# 2 Discussion

### 2.1 PTM Configuration Aspects

### 2.1.1 PTM configuration – signalling approaches

UEs may need to be transitioned to RRC_INACTIVE in overload/congestion situation, and can continue “certain” multicast session(s). These UEs are required to be configured to receive multicast in RRC_INACTIVE. Following approaches are considered in RAN2:

- Approach 1: RRC Release with suspendConfig

- Approach 2: SIB+MCCH

- Approach 3: MCCH configuration through dedicated signalling

There are some concerns with approach 3. UE receives multicast configuration through RRC Release with suspendConfig at RRC state transition. As multicast configuration would not change immediately after RRC state change, hence there is no benefit to early receive multicast MCCH configuration through dedicated signalling. That is, there is no urgency or benefit to receive early multicast MCCH configuration. Further, UE may or may not continue to be on the same serving cell (as transition to RRC_INACTIVE involves cell selection). There is no benefit either w.r.t. to authorization/security by receiving MCCH configuration through dedicated signaling. An UE that has joined a single session can also receive configuration for other sessions.

Observation 1: Multicast MCCH configuration through dedicated signaling does not provide any significant benefit (no urgency to receive MCCH early) and suffers from lack of authorized access per multicast session.

Proposal 1: RAN2 to agree that multicast MCCH configuration is not provided through dedicated signalling.

### 2.1.2 PTM configuration – deactivated session indication?

When UE is transitioned to RRC_INACTIVE, UE needs to monitor activation notification through group paging for deactivated session(s). There seems no need for specific indication or configuration for deactivated session(s) in RRCRelease with suspendConfig as UE just needs to monitor group paging for these deactivated session(s) in RRC_INACTIVE that UE has joined. When activated to stay in RRC_INACTIVE state, UE can receive the PTM configuration for these session(s) through multicast MCCH message or alternatively, when activated to move to RRC_CONNECTED state, UE receives the PTM configuration through dedicated signalling. This is applicable even when UE has only deactivated session(s).

Proposal 2: No PTM configuration or indication is provided in RRCRelease with suspendConfig for the deactivated multicast session(s) to the UE.

### 2.2 State Transition Aspects

### 2.2.1 Reception quality based state transition

Figure 1: Considerations for reception quality based state transition

R2#121bis made an agreement as follows:

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

With regard to the specifying details, we think signal strength/quality (RSRP or RSRQ) threshold configuration and measurement approach are significant aspects. As illustrated in Figure 1, it is important that the channel measurements need to be reliable, avoiding any false alarms while making the critical decision of RRC state transition for the UE. That is, a sustained channel quality measurement meeting the configured threshold for a configured state transition time ‘T’ needs be ensured. Lastly, it should also be considered that the serving cell is not congested and the session actually belongs to critical service that deserves the state transition. Network can control the criticality aspect by suitably configuring the threshold to only relevant UEs and services via RRCRelease with suspendConfig. Whereas, network may need to make UE aware about “congestion situation”, lest all the cell-edge UEs should transit to the RRC_CONNECTED just because they just meet the threshold conditions! Note that it is not efficient that network may be forced to reject these UEs to enter RRC_CONNECTED.

Proposal 3: To provide a robust and effective approach for channel quality based state transition:

- Signal strength/quality (RSRP or RSRQ) and state transition time ‘T’ are considered for threshold configuration in the RRCRelease with suspendConfig

- Sustained channel quality measurements meeting the configured threshold for a configured state transition time ‘T’ are considered to prevent false alarms

- Network congestion status is also considered as part of UE’s decision for transition to RRC_CONNECTED

One important cause for UE’s multicast reception in RRC_INACTIVE is network congestion. However, UE is quite unaware about congestion status. Without congestion status awareness, it is quite uncertain situation for a UE whether it should attempt to go back to RRC_CONNECTED (e.g. when reception quality has degraded in RRC_INACTIVE) or maintain service continuity with performing a cell reselection. New multicast SIB can provide a very effective way to broadcast a congestion status indication by the network to all UEs residing in the cell.

Proposal 4: RAN2 considers a congestion status indication to be provided in the new multicast SIB.

### 2.5.2 Cell selection at transition to RRC_INACTIVE

- As agreed in R2#121, serving cell will not provide the PTM configuration of neighbour cells from other gNBs. Given this, it is required that UE transitioning from RRC_CONNECTED to RRC_INACTIVE should continue to camp on the same serving cell where and for which UE has received PTM configuration in RRC_CONNECTED. However, as per TS 38.331 sec. 5.3.8.3, UE undertakes ‘cell selection’ upon receiving RRCRelease with suspendConfig and therefore, there is a need to ensure that UE camps to the same serving cell while doing so.

Proposal 5: Upon receiving RRCRelease with suspendConfig with multicast PTM configuration, while performing ‘cell selection’ at transition to RRC_INACTIVE, UE prioritizes the serving cell to camp where and for which UE has received the multicast PTM configuration.

### 2.2.3 Multicast inactivity handling in RRC_INACTIVE

It is expected that the UE receiving multicast in RRC_INACTIVE may face similar issue as it were for RRC_CONNECTED UEs e.g. multicast data inactivity. In some case, UE may miss session activation or session deactivation notification (e.g. temporary out of coverage) and this may cause loss of multicast data or unnecessary decoding/power consumption respectively, and bad user experience in either case. Thereore, it is required that the UE in RRC_INACTIVE should maintain a multicast inactivity timer for G-RNTI monitoring.

Multicast inactivity timer can be started/ restarted when a MTCH is received by the G-RNTI or Rel-18 Group paging is received for G-RNTI i.e. activation for session reception in RRC_INACTIVE. UE stops multicast inactivity timer for G-RNTI monitoring when session deactivation is received (e.g. in multicast MCCH message). When multicast inactivity timer expires, UE may stop monitoring the G-RNTI. Further, UE may request network to transit to RRC_CONNECTED as the reason for staying for the RRC_INACTIVE (i.e. reception of multicast session) is no longer valid. UE may formally release the multicast session, if UE is not interested in multicast session, or could avail the service through unicast.

Proposal 6: UE receiving multicast in the RRC_INACTIVE, maintains a multicast Inactivity Timer per G-RNTI monitoring.

Proposal 7: UE receiving multicast in the RRC_INACTIVE, starts/restarts multicast Inactivity Timer for G-RNTI monitoring when a MTCH is received by the G-RNTI or Rel-18 Group paging is received for G-RNTI i.e. activation for session reception in RRC_INACTIVE. UE stops multicast Inactivity Timer for G-RNTI monitoring when session deactivation is received.

Proposal 8: UE receiving multicast in the RRC_INACTIVE, upon expiry of multicast Inactivity Timer, stops monitoring the relevant G-RNTI. Further, UE may request network to transit to RRC_CONNECTED.

### 2.3 Multicast Reception and SDT Aspects

### 2.3.1 Group paging and SDT

In legacy, while SDT procedure is ongoing, UE does not monitor a paging channel for paging using TMGI. We think same approach is workable for UE configured for multicast reception in RRC_INACTIVE. However, as this UE may be notified to either receive multicast session in RRC_INACTIVE or to move to RRC_CONNECTED in the group paging; there is a need to have similar provisions for the UE, while SDT procedure is ongoing. To support this behaviour, gNB may send either RRCRelease with suspendConfig message or RRC Resume message to the UE when SDT procedure is ongoing and UE needs to be informed for session activation. As RRCRelease with suspendConfig can carry multicast PTM configuration for activated session(s), UE can infer this for session activation of the related TMGI(s) as well as consider it as the latest PTM configuration i.e. UE is not required to receive and/or validate PTM configuration through multicast MCCH (unlike activation through group paging).

Proposal 9: UE which is configured for multicast reception in RRC_INACTIVE, does not monitor group paging while SDT procedure is ongoing.

Proposal 10: UE which is configured for multicast reception in RRC_INACTIVE, considers the multicast PTM configuration received in RRCRelease with suspendConfig while SDT procedure is ongoing, as the activation notification of the related TMGI(s) and as the latest PTM configuration.

### 2.3.2 Multicast reception and SDT failure handling

In legacy, UE transits to RRC_IDLE upon unsuccessful completion of the SDT procedure. The unsuccessful completion of the SDT procedure can be due to many reasons including cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over Configured Grant (CG) and the UE has not received a response from the network after the initial PUSCH transmission.

However, for UE configured for multicast reception in RRC_INACTIVE, this is drastic as the UE would lose the multicast session(s). As the SDT failure would cause the transition to RRC_IDLE, UE should inform upper layers about TMGI(s) for multicast session(s) that were configured for reception in RRC_INACTIVE, to enble quick recovery by transiting to RRC_CONNECTED.

Proposal 11: Upon unsuccessful completion of the SDT procedure:

- UE which is configured for multicast reception in RRC_INACTIVE, transits to RRC_IDLE (legacy specification).

- UE informs upper layers about TMGI(s) for multicast session(s) that were configured for reception in RRC_INACTIVE, to enable quick recovery by transiting to RRC_CONNECTED.

### 2.4 Notification and Group Paging Aspects

### 2.4.1 Multicast MCCH change notification

To efficiently monitor multicast MCCH, there is a need for MCCH change notification, quite similar to that is used in MBS broadcast. The PDCCH that schedules multicast MCCH can carry Multicast MCCH change notification. At the minimum, notification for session configuration change and for neighbor cell information change is needed. Though an individual UE may have only a few multicast sessions to be received in RRC_INACTIVE, however, the multicast MCCH would cater to many sessions for the serving cell. Specifying specific bit for each session configuration change notification would be an overkill. Also, it is quite rare for configuration to change. So we think, a common bit for all multicast sessions is sufficient. Similarly, one bit for neighbor cell information change notification would suffice.

Proposal 12: Define multicast MCCH change notification fields - one bit for multicast session configuration change and one bit for neighbour cell information change.

### 2.4.2 Session deactivation notification

R2#122 made the following agreements:

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

It is quite evident the deactivation indication cannot be self-contained in the PDCCH DCI owing to unreasonable need of precious DCI bits to cater to each and every multicast session notification and no TU for RAN1 work. To keep things simple, it is preferable to have a multicast MCCH message based deactivation notification. Also, instead of interpreting session deactivation from an absence of related PTM configuration, explicit indication for the multicast session deactivation is added in the multicast MCCH message. Notably, network may not remove the PTM configuration just due to deactivation, it may be more relevant when the session is actually released.

Proposal 13: Multicast MCCH message carries explicit indication for the multicast session deactivation (i.e. UE needs not interpret session deactivation from an absence of related PTM configuration in multicast MCCH message)

### 2.4.3 Session release – implicit notification

- RAN2 agreed that “No additional enhancement (with regard to enhancements made for ‘deactivation/temp no data’) is needed specifically for enabling UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release”.

That is, release notification is not provided in Group notification and as in legacy, UE may perform NAS level release whenever it goes to RRC_CONNECTED. Without release notification, UE would continuously and indefinitely monitor multicast channel in RRC_INACTIVE, which is power consuming and is not desired. However, as the session is released, it seems not reasonable that multicast MCCH would carry the configuration for such session. This removal of configuration from MCCH can serve as a timely notification for session release for the UE. UE can release the multicast configuration. This does not require any additional enhancement and is aligned with the RAN2 agreement.

Proposal 14: UE determines about session release from the removal of multicast session configuration in multicast MCCH. UE can release the PTM configuration and stop monitoring for the multicast channel for the session in RRC_INACTIVE.

### 2.4.4 Group Paging – common/separate TMGI list

During [Post122][607]-offline discussion, an issue was raised that whether a common or a separate TMGI list be considered for Rel-18 group paging. Further, an EN is added.

Editor’s note: FFS whether there is any issue with the current signalling structure related to the per TMGI indication of inactiveReceptionAllowed-r18.

If we pursue a common TMGI list (i.e. referring to the TMGI list in the Rel-17 group paging part), there may be some limitation. For example, activated Multicast session is always required to be enabled “together” for reception in RRC_CONNECTED as well as RRC_INACTIVE. That is, network cannot do multicast session transmission only in RRC_INACTIVE. This is quite restricting situation, as there may be a possible case that the cell is congested and it cannot allow UEs enter into RRC_CONNECTED but it is feasible to provide the activated multicast session in RRC_INACTIVE. Notably, the resources used for RRC_INACTIVE UEs may not be same as that for RRC_CONNECTED UEs. Comparatively, common TMGI list approach may or may not be more signalling efficient than separate TMGI list based on the scenarios.

Observation 2: Common TMGI list imposes restriction that network cannot pursue activated multicast session transmission only in RRC_INACTIVE. For example, there may be a possible case that the cell is congested and it cannot allow UEs enter into RRC_CONNECTED but it is still feasible to provide the activated multicast session in RRC_INACTIVE.

Proposal 15: RAN2 to discuss and decide on one approach among:

- Common TMGI list i.e. reusing the TMGI list in Rel-17 Group Paging along with indication of inactiveReceptionAllowed-r18 in Rel-18 Group Paging

- Separate TMGI list is used in Rel-18 Group Paging which implies the activated session reception in RRC_INACTIVE

### 2.5 Mobility and Service Continuity Aspects

### 2.5.1 Neighbour cell information

- To ensure UEs can move (reselect) across cells, neighbour cell information including the ‘applicability’ of the existing PTM configuration along with multicast session support across cells should be provided through multicast MCCH. Based on the information about applicability of the PTM configuration and support of the session on neighbour cell(s), UE can make a judicious and prompt decision to support mobility and service continuity e.g. whether to go or not to go to RRC_CONNECTED.

- Proposal 16: RAN2 to agree that neighbour cell information (NCL) in multicast MCCH provides the PTM configuration ‘applicability’ along with multicast session support for the neighbour cells to support mobility and service continuity.

### 2.5.2 Frequency prioritization and de-prioritization for multicast

R2#121bis made an agreement as follows:

                - Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

Multicast differs from the broadcast in terms of coverage and service provisioning, and it is not prudent to extend the SAI/USD based solution for multicast reception in RRC_INACTIVE. Instead, frequency prioritization information for multicast reception in the RRC_INACTIVE can be directly provided through the dedicated signalling (i.e. RRCRelease with SuspendConfig) as in the legacy mechanism.

Proposal 17: Dedicated frequency prioritization information for multicast reception in the RRC_INACTIVE is provided to the UE through the RRCRelease with SuspendConfig.

Another important issue to consider is the RAN overload due to multicast. It should be feasible to reuse the RAN overload control mechanism of de-prioritization specifically for multicast. In legacy LTE and NR, network can determine RAN overload situation and sends to the UE, a de-prioritization request for the current frequency (or RAT) along with RRCRelease message. This ensures that the UE considers the indicated frequency (or all frequencies of the RAT) to be the lowest priority for cell reselection for a configured duration.

We think this well-established and proven mechanism of de-prioritization seems a good approach to address the congestion / overload scenario for multicast. Network can indicate de-prioritization request for multicast frequency along with RRCRelease with suspendConfig and only multicast UE can consider the indicated frequency to be the lowest priority for cell reselection for configured time duration while UE is in RRC_INACTIVE.

Proposal 18: UE can be provided with the de-prioritization request for multicast in RRCRelease with suspendConfig to control RAN overload on specific frequency.

# 3 Conclusion

Request RAN2 to discuss and agree to the following observations and proposals:

PTM Configuration Aspects

Observation 1: Multicast MCCH configuration through dedicated signaling does not provide any significant benefit (no urgency to receive MCCH early) and suffers from lack of authorized access per multicast session.

Proposal 1: RAN2 to agree that multicast MCCH configuration is not provided through dedicated signalling.

Proposal 2: No PTM configuration or indication is provided in RRCRelease with suspendConfig for the deactivated multicast session(s) to the UE.

State Transition Aspects

Proposal 3: To provide a robust and effective approach for channel quality based state transition:

- Signal strength/quality (RSRP or RSRQ) and state transition time ‘T’ are considered for threshold configuration in the RRCRelease with suspendConfig

- Sustained channel quality measurements meeting the configured threshold for a configured state transition time ‘T’ are considered to prevent false alarms.

- Network congestion status is also considered as part of UE’s decision for transition to RRC_CONNECTED

Proposal 4: RAN2 considers a congestion status indication to be provided in the new multicast SIB.

Proposal 5: Upon receiving RRCRelease with suspendConfig with multicast PTM configuration, while performing ‘cell selection’ at transition to RRC_INACTIVE, UE prioritizes the serving cell to camp where and for which UE has received the multicast PTM configuration.

Proposal 6: UE receiving multicast in the RRC_INACTIVE, maintains a multicast Inactivity Timer per G-RNTI monitoring.

Proposal 7: UE receiving multicast in the RRC_INACTIVE, starts/restarts multicast Inactivity Timer for G-RNTI monitoring when a MTCH is received by the G-RNTI or Rel-18 Group paging is received for G-RNTI i.e. activation for session reception in RRC_INACTIVE. UE stops multicast Inactivity Timer for G-RNTI monitoring when session deactivation is received.

Proposal 8: UE receiving multicast in the RRC_INACTIVE, upon expiry of multicast Inactivity Timer, stops monitoring the relevant G-RNTI. Further, UE may request network to transit to RRC_CONNECTED.

Multicast Reception and SDT Aspects

Proposal 9: UE which is configured for multicast reception in RRC_INACTIVE, does not monitor group paging while SDT procedure is ongoing.

Proposal 10: UE which is configured for multicast reception in RRC_INACTIVE, considers the multicast PTM configuration received in RRCRelease with suspendConfig while SDT procedure is ongoing, as the activation notification of the related TMGI(s) and as the latest PTM configuration.

Proposal 11: Upon unsuccessful completion of the SDT procedure:

- UE which is configured for multicast reception in RRC_INACTIVE, transits to RRC_IDLE (legacy specification).

- UE informs upper layers about TMGI(s) for multicast session(s) that were configured for reception in RRC_INACTIVE, to enable quick recovery by transiting to RRC_CONNECTED.

Notification and Group Paging Aspects

Proposal 12: Define multicast MCCH change notification fields - one bit for multicast session configuration change and one bit for neighbour cell information change.

Proposal 13: Multicast MCCH message carries explicit indication for the multicast session deactivation (i.e. UE needs not interpret session deactivation from an absence of related PTM configuration in multicast MCCH message)

Proposal 14: UE determines about session release from the removal of multicast session configuration in multicast MCCH. UE can release the PTM configuration and stop monitoring for the multicast channel for the session in RRC_INACTIVE.

Observation 2: Common TMGI list imposes restriction that network cannot pursue activated multicast session transmission only in RRC_INACTIVE. For example, there may be a possible case that the cell is congested and it cannot allow UEs enter into RRC_CONNECTED but it is still feasible to provide the activated multicast session in RRC_INACTIVE.

Proposal 15: RAN2 to discuss and decide on one approach among:

- Common TMGI list i.e. reusing the TMGI list in Rel-17 Group Paging along with indication of inactiveReceptionAllowed-r18 in Rel-18 Group Paging

- Separate TMGI list is used in Rel-18 Group Paging which implies the activated session reception in RRC_INACTIVE

Mobility and Service Continuity Aspects

- Proposal 16: RAN2 to agree that neighbour cell information (NCL) in multicast MCCH provides the PTM configuration ‘applicability’ along with multicast session support for the neighbour cells to support mobility and service continuity.

Proposal 17: Dedicated frequency prioritization information for multicast reception in the RRC_INACTIVE is provided to the UE through the RRCRelease with SuspendConfig.

Proposal 18: UE can be provided with the de-prioritization request for multicast in RRCRelease with suspendConfig to control RAN overload on specific frequency.