3GPP TSG-RAN WG2 Meeting #123	R2-2307615

Toulouse, France, 21 - 25 August 2023

Agenda Item:	7.20.3

Source:	Huawei, HiSilicon

Title:	Extension of unified TCI framework for mTRP

Document for:	Discussion and decision

# 1. Introduction

In the Rel-18 MIMO WID [1], one of the objectives with RAN2 impact is the following:

2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

RAN1 have already made a number of agreements on extension of unified TCI framework for mTRP. Based on RAN1 agreements, RAN2 discussed the MAC CE design of unified TCI framework for mTRP. In this contribution, we further discuss the RAN2 impacts taking into account RAN1 agreements.

# 2. Discussion

## 2.1 MAC

### 2.1.1 mDCI mTRP

According to the RAN1 agreements below, a new MAC CE for TCI state activation/deactivation needs to be introduced for mDCI mTRP. In RAN2 #122 meeting, RAN2 agreed to revise the existing unified TCI state activation/deactivation MAC CE (in Clause 6.1.3.47 of TS38.321) by adding a “CORESET Pool ID” field to support mDCI based mTRP operation. Since there are seven reserved bits in this legacy MAC CE, one reserved bit can be used for indicating a “CORESET Pool ID”.

RAN2 #121bis-e Working assumption: Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation. RAN2 #122 RAN2 confirm the following working assumption as an agreement: - Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

For mDCI mTRP operation, one reserved bit in the legacy unified TCI state activation/deactivation MAC CE (in Clause 6.1.3.47 of TS 38.321) can be used for indicating a “CORESET Pool ID”.

### 2.1.2 sDCI mTRP

In sDCI mTRP operation, RAN2 agreed a general design rule for this MAC CE, as extracted below.

RAN2 #122 For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) - FFS for the separate DL/UL TCI mode.

The basic principle of Rel-18 TCI state activation MAC-CE was discussed in previous meetings. For each TCI codepoint, the MAC-CE should indicate the following information:

- Info 1: The activated TCI states associated with the TCI codepoint are for one or two TRPs;

- Info 2: For each TRP among the one or two TRPs, the number of activated TCI states is one (e.g., one DL TCI) or two (e.g., one DL TCI plus one UL TCI);

- Info 3: For each TCI state among the activated TCI states associated with the TCI codepoint, the corresponding TRP of the TCI state is the first TRP or the second TRP;

- Info 4: For each TCI state among the activated TCI state associated with the TCI codepoint, the type of the TCI state is DL TCI state or UL TCI state;

- Info 5: For each TCI state among the activated TCI state associated with the TCI codepoint, the index of the TCI state;

To minimize standardization effort, Rel-17 TCI state activation MAC-CE can be considered as a starting point. In Rel-17 TCI activation MAC-CE, TCI state ID and TCI state type are carried in an octet. Such design can be reused to indicate Info 4 and Info 5.

For sDCI mTRP operation, reuse the legacy mechanism to indicate TCI state ID and TCI state type (DL or UL), i.e. the ID and type of a TCI state are indicated in the same octet.

- TS38.321: Clause 6.1.3.47 Unified TCI States Activation/Deactivation MAC CE

In Rel-17 MAC-CE, in addition to TCI state ID and TCI state type, one additional field is used to indicate the information (i.e. number) of the activated TCI states of a TCI codepoint. Such kind of design principle can be reused for indication of Info 1, 2 and 3. For indication of Info 1 (number of TRPs), 1 bit is needed. For indication of Info 2 (number of TCI states for each TRP), 1 bit (if the activated TCI states are for one TRP) or 2 bits (if the activated TCI states are for two TRPs) are needed. For indication of info 3 (corresponding TRP of TCI state), 1 bit (if the activated TCI states are for one TRP) or 2 bits (if the activated TCI states are for two TRPs) are needed. So, if Info 1, 2 and 3 are indicated independently, 1+2+2=5 bits may be needed.

To minimize the number of bits for indication of Info 1, 2 and 3, we suggest introducing two additional fields for each TCI codepoint i.

- Field Ti (1 bit): this field is used to indicate Info 1. In particular,

  - if the value of Ti is ‘0’, the activated TCI states associated with TCI codepoint i are for one TRP;

  - if the value of Ti is ‘1’, the activated TCI states associated with TCI codepoint i are for two TRPs;

- Field Ni (2 bits): this field is used to indicate Info 2 and 3 together for overhead reduction. To further reduce the overhead, the interpretation of Ni can be based on the value of Field Ti, so that some invalid cases can be precluded. In particular,

  - if the value of Ti is ‘0’, the four candidate values (00, 01, 10 and 11) of Ni are interpreted as follows.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP;

    - 01： TCI codepoint i is associated with one TCI state of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP;

    - 11： TCI codepoint i is associated with two TCI states of the second TRP;

  - if the value of Ti is ‘1’, the four candidate values (00, 01, 10 and 11) of Ni are interpreted as follows. To reduce overhead, it is assumed that the TCI states of the first TRP are located before the TCI states of the second TRP, so that the corresponding TRP of each TCI state does not need to be indicated.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP and one TCI state of the second TRP;

    - 01： TCI codepoint i is associated with one TCI state of the first TRP and two TCI states of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP and one TCI state of the second TRP;

    - 11： TCI codepoint i is associated with two TCI states of the first TRP and two TCI states of the second TRP;

With the above two fields (Ti and Ni), Info 1, 2 and 3 can be indicated with minimized overhead. The reduction of overhead comes from the design that the interpretation of Ni is based on the value of Ti and hence the invalid cases are precluded. For example, when Ti is ‘0’ (i.e., the activated TCI states associated with TCI codepoint i are for one TRP), the four cases given in the bullet with Ti is ‘1’ as above do not need to be indicated. The whole Rel-18 TCI state activation MAC-CE is given as below.

| R | Serving Cell ID |  |  |  |  | DL BWP ID |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R | R | R | R | R | R | UL BWP ID |  |
| T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 |
| N1 |  | N2 |  | N3 |  | N4 |  |
| N5 |  | N6 |  | N7 |  | N8 |  |
| D/U | TCI state ID 1 |  |  |  |  |  |  |
| D/U | TCI state ID 2 |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |
| D/U | TCI state ID N |  |  |  |  |  |  |

For sDCI mTRP operation, use the above TCI state activation MAC-CE format. Compared to the Rel-17 TCI activation MAC-CE, it contains two new fields, i.e., Ti (1 bit) and Ni (2 bit), for each TCI codepoint i. The interpretation of these two fields is given as follows.

- Field Ti (1 bit):

  - 0: The activated TCI states associated with TCI codepoint i are for one TRP;

  - 1: The activated TCI states associated with TCI codepoint i are for two TRPs;

- Field Ni (2 bits):

  - if Ti = 0, Ni is interpreted as follows.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP;

    - 01： TCI codepoint i is associated with one TCI state of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP;

    - 11： TCI codepoint i is associated with two TCI states of the second TRP;

  - if Ti = ‘1’, Ni is interpreted as follows. Note that TCI state(s) of the first TRP are located before the TCI state(s) of the second TRP by default.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP and one TCI state of the second TRP;

    - 01： TCI codepoint i is associated with one TCI state of the first TRP and two TCI states of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP and one TCI state of the second TRP;

    - 11： TCI codepoint i is associated with two TCI states of the first TRP and two TCI states of the second TRP;

## 2.2 RRC

Using MAC CE and DCI, the network can indicate TCI states from RRC configured TCI lists for two TRPs. The dynamically scheduled PUSCH and PDSCH can follow the indicated TCI state(s). However, there are some RRC configured transmission, e.g., CG PUSCH, PUCCH, SRS, etc. For those physical channels or RS, RRC configuration to indicate which TCI state the UE should use is necessary.

### 2.2.1 PUCCH and CG PUSCH

- According to the RRC parameter list [2] provided by RAN1, RRC indication applyIndicatedTCIState-r18 is needed for CG PUSCH and PUCCH.

PUCCH-Resource and PUCCH-ResourceGroup-r16; applyIndicatedTCIState-r18; This parameter is used to inform the UE shall apply the first, the second, or both of the indicated joint/UL TCI states to PUCCH transmission(s) corresponding to a PUCCH resource or PUCCH resource group. For M-DCI based MTRP operation, the candidate values can be {the first, the second}, and the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.. ConfiguredGrantConfig; applyIndicatedTCIState-r18; This parameter is used to inform the UE shall apply the first, the second, or both of the indicated joint/UL TCI states to PUSCH transmission(s) corresponding a Type1-CG configuration. For M-DCI based MTRP operation, the candidate values can be {the first, the second}, and the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively..

The main difference between sDCI mTRP and mDCI mTRP is that the gNB can configure {the first, the second, both} for sDCI mTRP, while the gNB can only configure {the first, the second} for mDCI mTRP. We have the following proposals for CG PUSCH and PUCCH according to the information from RAN1:

Introduce an RRC indication per PUCCH resource/resource group. The value range is {the first, the second, both}.

- For mDCI mTRP operation, the gNB does not configure the value “both”.

- For sDCI mTRP operation, the gNB can configure either the first, the second, or both.

Introduce an RRC indication per Type1 CG configuration. The value range is {the first, the second, both}.

- For mDCI mTRP operation, the gNB does not configure the value “both”.

- For sDCI mTRP operation, the gNB can configure either the first, the second, or both.

### 2.2.2 PDCCH

For PDCCH reception, the network can configure zero, one or two TCI states to a CORESET. So the UE knows how to monitor PDCCH in this CORESET. The information from RAN1 is extracted as follows. Note that this indication is only needed for sDCI mTRP case, because in mDCI mTRP case, different CORESETPoolIndex will have different indicated TCI state.

ControlResourceSet; applyIndicatedTCIState-r18; This parameter is used to inform the UE shall apply the first, the second, both, or none of the indicated joint/DL TCI states to PDCCH reception(s) a CORESET for S-DCI based MTRP operation.

For sDCI mTRP operation, introduce an RRC indication per CORESET. The value range is {the first, the second, both, none}.

### 2.2.3 SRS and CSI-RS

According to the RRC parameter list [2], for SRS transmission and aperiodic CSI-RS reception, a RRC indication is needed as well to tell the UE to use the first or the second indicated TCI state.

CSI-AssociatedReportConfigInfo; applyIndicatedTCIState-r18; This parameter is used to inform the UE shall apply the first or the second indicated joint/DL TCI states to an aperiodic CSI-RS resource or an aperiodic CSI-RS resource set. For M-DCI based MTRP operation, the first and the second indicated joint/DL TCI states correspond to the indicated joint/DL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.. SRS-ResourceSet; applyIndicatedTCIState-r18; This parameter is used to inform the UE shall apply the first or the second indicated joint/UL TCI states to an SRS resource set. For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively..

We have the following proposal based on the information from RAN1.

Introduce an RRC indication for SRS and (aperiodic) CSI-RS. The value range is {the first, the second}.

# 3. Conclusion

Based on the discussion in this contribution, we have the following proposals:

MAC for multi-TRP operation

- For mDCI mTRP operation, one reserved bit in the legacy unified TCI state activation/deactivation MAC CE (in Clause 6.1.3.47 of TS 38.321) can be used for indicating a “CORESET Pool ID”.

- For sDCI mTRP operation, reuse the legacy mechanism to indicate TCI state ID and TCI state type (DL or UL), i.e. the ID and type of a TCI state are indicated in the same octet.

For sDCI mTRP operation, use the above TCI state activation MAC-CE format. Compared to the Rel-17 TCI activation MAC-CE, it contains two new fields, i.e. Ti (1 bit) and Ni (2 bit), for each TCI codepoint i. The interpretation of these two fields is given as follows:

- Field Ti (1 bit):

  - 0: The activated TCI states associated with TCI codepoint i are for one TRP;

  - 1: The activated TCI states associated with TCI codepoint i are for two TRPs;

- Field Ni (2 bits):

  - if Ti = 0, Ni is interpreted as follows.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP;

    - 01： TCI codepoint i is associated with one TCI state of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP;

    - 11： TCI codepoint i is associated with two TCI states of the second TRP;

  - if Ti = ‘1’, Ni is interpreted as follows. Note that TCI state(s) of the first TRP are located before the TCI state(s) of the second TRP by default.

    - 00： TCI codepoint i is associated with one TCI state of the first TRP and one TCI state of the second TRP;

    - 01： TCI codepoint i is associated with one TCI state of the first TRP and two TCI states of the second TRP;

    - 10： TCI codepoint i is associated with two TCI states of the first TRP and one TCI state of the second TRP;

    - 11： TCI codepoint i is associated with two TCI states of the first TRP and two TCI states of the second TRP;

RRC for PUCCH and CG PUSCH

Introduce an RRC indication per PUCCH resource/resource group. The value range is {the first, the second, both}.

- For mDCI mTRP operation, the gNB does not configure the value “both”.

- For sDCI mTRP operation, the gNB can configure either the first, the second, or both.

Introduce an RRC indication per Type1 CG configuration. The value range is {the first, the second, both}.

- For mDCI mTRP operation, the gNB does not configure the value “both”.

- For sDCI mTRP operation, the gNB can configure either the first, the second, or both.

RRC for PDCCH

For sDCI mTRP operation, introduce an RRC indication per CORESET. The value range is {the first, the second, both, none}.

RRC for SRS and CSI-RS

Introduce an RRC indication for SRS and (aperiodic) CSI-RS. The value range is {the first, the second}.

# 4. Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

[2] R1-2306271, Consolidated higher layers parameter list for Rel-18.