3GPP TSG-RAN WG2 Meeting #123	   R2-2307801

Toulouse, France, August 21-25, 2023

Agenda item:		7.21.3

Source:	NEC

Title:	Discussion on UP issues of Multiple PRACH Transmissions

Document for:		Discussion and Decision

1 Introduction

In the previous RAN2 meeting, the following agreements are achieved for Msg1 repetition [1][2].

Agreements RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS Agreements RAN2 to further discuss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure. Agreements - RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers - The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP

In RAN1 #113 [3], regarding RA-RNTI, the following agreement was reached.

Agreement RA-RNTI is calculated based on the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note 1: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note 2: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not.

In this contribution, we would like to further discuss the potential RAN2 impacts for UP, including fallback and threshold, and provide our views.

2 Discussion

Fallback

In the current email discussion of UP [4], whether to support fallback from lower number of Msg1 repetition to higher number is being discussed. In our understanding, it is not necessary to support it. Firstly, Msg1 repetition number is selected based on the current channel condition and it should be suitable during the whole RACH procedure considering the channel cannot change so fast. We think fallback is just a corner case. If RACH fails based on a selected repetition number, random access problem can be reported to the upper layer. The next RACH procedure can be triggered, then the UE may select an appropriate repetition number. Secondly, based on the current agreement, the fallback from legacy RA to Msg1 repetition will not be supported. Then for fallback from lower number to higher number, it is just the difference of repetition number. Hence, since the fallback from legacy RA to Msg1 repetition is not supported, similarly, the fallback from lower number to higher number should not be supported either. Thirdly, based on the current discussion for this fallback mechanism, there is a big impact on specification regardless of which option of the followings is selected.

- Option 2.2: All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type

- Option 2.3: Each repetition number is treated as a separate feature and we define fallback between features

Therefore, based on the analysis above, we think the fallback from lower number of Msg1 repetition to higher number should not be supported.

Proposal 1: Fallback from lower repetition number to higher repetition number is not supported for Msg1 repetition.

Multiple RSRP thresholds

Multiple RSRP thresholds for the different repetition numbers may be configured. Once configured, the UE selects the suitable repetition number based on the following procedure and the selection procedure may be placed in the section of selection of the set of random access resources similar to Msg3 repetition.

The MAC entity shall:

-	if the RSRP threshold of Msg1 repetition number 8 is configured and the measured RSRP of DL pathloss reference is less than the RSRP threshold of Msg1 repetition number 8:

-	assume Msg1 repetition number 8 is applicable for the current random access procedure;

-	else if the RSRP threshold of Msg1 repetition number 4 is configured and the measured RSRP of DL pathloss reference is less than the RSRP threshold of Msg1 repetition number 4:

-	assume Msg1 repetition number 4 is applicable for the current random access procedure;

-	else if the RSRP threshold of Msg1 repetition number 2 is configured and the measured RSRP of DL pathloss reference is less than the RSRP threshold of Msg1 repetition number 2:

-	assume Msg1 repetition number 2 is applicable for the current random access procedure;

-	else:

-	assume Msg1 repetition is not applicable for the current random access procedure;

Proposal 2: When RSRP threshold for Msg1 repetition number is configured, the selection procedure above is adopted.

Multiple PRACH occasions

The UE can select the number of PRACH transmissions at least for the first RACH attempt based on the configured RSRP threshold. When the number of PRACH transmissions is selected, the UE shall select one SSB. In the following, the UE selects the available PRACH occasion(s) based on the selected number of PRACH transmission and SSB. In the current TS 38.321 [5], only one PRACH occasion is selected and informed to the lower layer. Then for Msg1 repetition, a simple way is to select multiple PRACH occasions once in the procedure of random access resource selection. For multiple PRACH occasions, how to inform the lower layer, the following two options may be considered:

Option 1: All PRACH occasions within one RACH attempt are indicated to the lower layer together.

Option 2: All PRACH occasions are separately indicated to the lower layer.

For option 1, since multiple PRACH occasions are indicated to the lower layer together, the procedure below is performed once. Then for power ramping, it is not applied within one RACH attempt according to the current agreement. In our understanding, the current SPEC may be reused. For RA-RNTI, some clarifications are needed based on RAN1’s agreement, for example, “compute the RA-RNTI associated with the last valid PRACH occasion in which the Random Access Preamble is transmitted within one Msg1 repetition transmission”. While for the calculation of PREAMBLE_RECEIVED_TARGET_POWER, the current specification may be updated based on the future discussion. So option 1 seems to have a smaller impact on specification.

The MAC entity shall, for each Random Access Preamble: 1> if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and 1> if the notification of suspending power ramping counter has not been received from lower layers; and 1> if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and 1> if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. 1> select the value of DELTA_PREAMBLE according to clause 7.3; 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA; 1> except for contention-free Random Access Preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; 1> instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

For option 2, since each PRACH occasion is separately indicated, whether PRACH resource selection and preamble transmission procedure can be separately performed for each Msg1 repetition needs to be considered. If so, there will be a big impact on specification.

Therefore, in our understanding, it is necessary to discuss how to inform multiple PRACH occasions.

Proposal 3: RAN2 to discuss the following ways of indicating all PRACH occasions within one attempt:

- All PRACH occasions within one RACH attempt are indicated to the lower layer together

- All PRACH occasions are separately indicated to the lower layer

3 Conclusion

In this contribution, we have further discussed the potential RAN2 impacts of multiple PRACH transmissions from UP perspective, all the proposals are summarized below:

Proposal 1: Fallback from lower repetition number to higher repetition number is not supported for Msg1 repetition.

Proposal 2: When RSRP threshold for Msg1 repetition number is configured, the selection procedure above is adopted.

Proposal 3: RAN2 to discuss the following ways of indicating all PRACH occasions within one RACH attempt:

- All PRACH occasions in one RACH attempt are indicated to the lower layer together

- All PRACH occasions are separately indicated to the lower layer

4 References

[1] Draft_R2_121bis_meeting_report_v2.

[2] Draft_R2_122_meeting_report_v1.

[3] Draft_Minutes_report_RAN1#113_v020.

[4] R2-230xxxx UP open issues on CE.

[5] TS 38.321 h50.