3GPP TSG-RAN2#123	R2-2307804

Toulouse, 21th Aug – 25th Aug, 2023

Agenda item:		7.18.3 (NR_MT_SDT-Core)

Source:	LG Electronics Inc.

Title: 	Remaining UP issues on MT-SDT

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made agreements for MT-SDT as follows:

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

Based on above agreements, RAN2 had an e-mail discussion [1], but some issues were not clearly concluded.

In this contribution, we present our views on remaining UP issues.

2.	Discussion

In the last meeting, RAN2 agreed that for both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first. For this, in the running CR [2], it is captured that the UE performs CG-SDT only if the time gap between SDT initiation and the next CG occasion is less than the threshold, i.e. cg-SDT-MaxDurationToNext-CG-Occasion.

2> if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and 2> if the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than cg-SDT-MaxDurationToNext-CG-Occasion, if configured; and 2> if the SDT procedure is initiated for MO-SDT as in TS 38.331 [5], for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel; and 2> if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available: 3> indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled; 3> perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

During e-mail discussion [1], one issue was raised that even if RA occasion is farther than CG occasion, the RA occasion is selected if only the time gap and the threshold are compared. However, it was concluded to postpone this issue because this issue has not been discussed before.

According to [2], the UE selects CG-SDT or RA procedure as follows:

- If CG occasion for SDT is not farther than the threshold from SDT initiation, the UE selects CG-SDT.

- If CG occasion for SDT is farther than the threshold from SDT initiation, the UE selects RA procedure.

The intention of comparing the time gap and the threshold is to prevent the delay of transmission of CCCH message, i.e. RRCResumeRequest message, due to the long CG periodicity.

However, there may be a case where the time gap is larger than the threshold but there is no RA occasion until the next CG occasion. In this case, based on [2], the UE always selects RA procedure even though RA occasion is farther than the available CG occasion, so that RRCResumeRequest message is transmitted later than when CG-SDT is selected. This is against to the intention of comparing the time gap and the threshold. Thus, we think it is not desirable to compare only the time gap and the threshold.

Figure 1. Example of problematic case

In order to prevent the delay from selecting farther RA occasion, it should also be considered whether there is an available RA occasion until the next CG occasion. In other words, if the time gap between SDT initiation and the next CG occasion is larger than the threshold, the UE needs to check whether there is an available RA occasion until the next CG occasion. If there is an available RA occasion until the next CG occasion, the UE selects RA procedure, and if there is no available RA occasion until the next CG occasion, the UE selects CG-SDT. In this way, the UE can select earliest one between CG occasion and RA occasion.

Figure 2. Selection CG-SDT and RA procedure

Proposal 1. If the time gap between SDT initiation and the next CG occasion is larger than the threshold, the UE should check whether there is an available RA occasion until the next CG occasion.

During e-mail discussion [1], the other issues was raised whether UL data volume is checked or not for MT-SDT initiation. Some companies think that it has been already agreed not to check data volume for MT-SDT initiation because RAN2 agreed to specify RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data.

However, we have a different understanding on this. In Rel-17 SDT, the availability of UL data is checked in RRC and the UL data volume is checked in MAC. These two checking procedure are separate, so we think whether to check DVT or not in MAC should be discussed and a clear agreement is needed.

We think that the legacy DVT check should be applied to MT-SDT procedure. There may be a case where UL data of SDT RB, which is larger than DVT, is generated before MT-SDT initiation after receiving the paging message for MT-SDT. In this case, if DVT is not checked, MT-SDT would be initiated and the large UL SDT data is transmitted during SDT procedure. This may lead to data loss or data transmission delay, so it would be desirable to transmit the large UL data after transition to RRC_CONNECTED.

Therefore, we think that DVT condition should be checked in MAC in MT-SDT procedure.

Proposal 2. DVT condition should be checked in MAC in MT-SDT procedure, same as legacy.

In addition, in [2], configuredGrantType1Allowed is considered only for MO-SDT initiation. This is because RAN2 agreed that CCCH message can be transmitted on CG-SDT without data when triggered by MT-SDT.

2> if the SDT procedure is initiated for MO-SDT as in TS 38.331 [5], for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel; and 2> if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available: 3> indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled; 3> perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

However, RAN2 has not been agreed that CCCH message is always transmitted on CG-SDT without data or configuredGrantType1Allowed is not considered for MT-SDT.

In addition, in the last meeting, RAN2 agreed to check LCH restrictions for DRBs as in MO-SDT if UL data is available during SDT procedure. Thus, if UL data of SDT DRB is generated before MT-SDT initiation after receiving the paging message for MT-SDT, LCH restriction should be checked at MT-SDT initiation.

Therefore, configuredGrantType1Allowed should be considered for the condition of MT-SDT initiation.

Proposal 3. configuredGrantType1Allowed should be considered for the condition of MT-SDT initiation.

From Proposal 1, 2 and 3, we provide TP in Annex.

Proposal 4. Adopt TP as in Annex.

3.	Conclusion

In this document, we present our view on remaining UP issues, and make proposals as follows.

Proposal 1. If the time gap between SDT initiation and the next CG occasion is larger than the threshold, the UE should check whether there is an available RA occasion until the next CG occasion.

Proposal 2. DVT condition should be checked in MAC in MT-SDT procedure, same as legacy.

Proposal 3. configuredGrantType1Allowed should be considered for the condition of MT-SDT initiation.

Proposal 4. Adopt TP as in Annex.

4.	Reference

- R2-2306958	[Post122][309][MT-SDT] 38.321 Running CR (Huawei)	Huawei, HiSilicon

- R2-2306965	Introduction of MT-SDT to MAC spec	Huawei, HiSilicon

5.	Annex

## [MAC running CR]

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

Editor’s NOTE:	It needs to be further discussed whether the MT-SDT RSRP threshold can be a separate RSRP threshold when MO-SDT is configured according to the following agreement: 9.	A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

-	cg-SDT-MaxDurationToNext-CG-Occasion: time threshold for the UE to determine whether to perform CG-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

Editor’s NOTE:	Whether the above condition is needed is dependent on the further discussion on FFS 3a: Check for DVT (if UL data becomes available in UL).

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than cg-SDT-MaxDurationToNext-CG-Occasion, if configured, or if the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is larger than cg-SDT-MaxDurationToNext-CG-Occasion and there is no RA occasion available until the first available CG occasion; and

2>	for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.