3GPP TSG-RAN WG2 Meeting #123	R2-2307838

Toulouse, France, 21 Aug – 25 Aug 2023

Agenda item:	7.23.2

Source:	Apple

Title:	UE Access for 5GS Network Timing Synchronization

WID/SID:	NR_TRS_URLLC

Document for:	Discussion / Decision

# Introduction

SA2 has studied architectural enhancements for supporting 5G Timing Resiliency and URLLC in the 5G system. To address the RAN impacts out of this work, RAN approved a new Rel-18 WI in the last RAN plenary at RAN#99.

One of the Rel-18 WI objectives is about 5GS network timing synchronization status and reporting [1].

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

Enhancements in this WI allow the 5G System to remain time resilient after a clock synchronization incident and to act as a backup and offer wireless and indoor-capable time synchronization service for other applications.

This paper provides some of our views on 5GS Network Timing Synchronization status and reporting from a UE perspective.

# Discussion

## Background

Use cases related to the NR_TRS_URLLC work item target network customers like smart grids and audio/video production. Rel-18 timing resiliency is meant to improve the robustness of services with a strong need for time synchronization. In the event of a clock/time synchronization degradation or failure, the 5G system can assist the end user to switch to an alternative time source. Accordingly, the 5G system will support reporting network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs and AFs. Likewise, the RAN will be able to observe 5GS network timing synchronization status and inform UEs and AFs.

In order to notify UEs of a clock quality information update, the gNB includes in SIB9 an event ID when the network timing synchronization status exceeds the threshold or when the time system meets the thresholds again (following an earlier event). Either event serves as a notification to the UEs reading the SIB9 that there is new clock quality information available. The UE in RRC_INACTIVE or RRC_IDLE compares the event ID in SIB9 with the locally stored ID to determine if the last available clock quality information was already retrieved. If the UE is instructed to reconnect to the network in the case when the UE determines that event ID has changed, the UE in RRC_INACTIVE or RRC_IDLE state reconnects to the network. After the UE has reconnected to the network, the gNB uses unicast RRC signalling to provision the clock quality information to the UEs.

## UE Access Randomization

Following the LS in [4], RAN2 identified that it is feasible to randomize the UE(s) re-connection to the network for clock quality information retrieval based on the current UAC framework [5].

The use of the UAC framework for randomization of UE(s) re-connection attempts may require a new access category. Accordingly, RAN2 had consulted CT1/SA1 to clarify if an existing access category can be used for the access attempt to retrieve the latest available clock quality information (e.g., the one for MO signalling), or if it would require a new one since it should not block access triggered by other MO signalling [5]. Meanwhile SA1 has responded in [6], indicating that there are no stage 1 requirements to have a separate access category for TRS_URLLC.

S1-231285:

[SA1 answer]: SA1 would like to inform that there are no stage 1 requirements to have a separate access category for TRS.

Given the response from SA1 and considering that CT1 had already envisioned an update of TS 24.501 as per the editor’s note in [7], if a new access category is not required then access attempts related to clock quality information retrieval can be associated with a new Access Identity. Otherwise, UAC would not allow for differentiation between normal MO signalling and signalling initiated for clock quality information retrieval.

Observation 1: With the current UAC method, there would be no differentiation between normal MO signalling and signalling initiated for clock quality information updating.

We see two options to address this issue.

- Option 1: Introduce a new Access Identity for clock quality information retrieval.

- Option 2: Configure a dedicated set of RA-Prioritization for an Access Identity.

Option 1 - New Access Identity

To control UE access attempts for clock quality information retrieval in a cell, the RAN can broadcast access control parameters in SIB1 to UEs supporting NR timing resiliency, which are used in conjunction with the clock quality information in SIB9. To achieve UE access differentiation using UAC, access attempts related to clock quality information retrieval can be associated with a new Access Identity (e.g., AI 4). The network sets the access control parameters for Access Identity 4 (in SIB 1) so that access attempts for clock quality information retrieval (from relevant UEs) are more likely to be barred (or can be controlled) compared to other types of UEs or other Access Identities.

In summary, to achieve a UE access differentiation access attempts related to clock quality information retrieval can be associated with a new Access Identity (e.g., AI 4). The mechanism controls the UE access based on their assigned Access Identity. Optionally, the Access Identity can be part of subscription data and/or stored user data on USIM.

Proposal 1: Randomize UE accesses through a new UAC Access Identity associated with UAC parameters configured by the network to spread UE connection requests in time.

A text proposal for the associated changes is provided in the annex.

Proposal 2: RAN2 to adopt the TP in the annex as a baseline.

Option 2 - Random Access Prioritization

By including the RA-Prioritization IE in the ASN.1 for different configuration scenarios it is possible to parameterize the random access procedure with multiple sets of powerRampingStepHighPriority and scalingFactorBI, in order to allow for prioritized random access. The configuration would also have to be associated with an Access Identity. It can provide a different ‘weight’ for the prioritization of this Access Identity on a given carrier compared to random accesses for other types of traffic. For example, access requests for clock quality information retrieval could get a lower (or even higher) priority than other access requests. However, it should be noted that this approach addresses network overload only indirectly, rather, it is a relative parameterization. We therefore think that option 1 is preferred.

Proposal 3: If specific RACH prioritization is deemed necessary for an Access Identity, sets of scalingFactorBI and powerRampingStepHighPriority can be configured in system information.

# Conclusions

This contribution provides initial views on 5GS Network Timing Synchronization status and reporting. We have following observations and proposals:

Observation 1: With the current UAC method, there would be no differentiation between normal MO signalling and signalling initiated for clock quality information updating.

Proposal 1: Randomize UE accesses through a new UAC Access Identity associated with UAC parameters configured by the network to spread UE connection requests in time.

Proposal 2: RAN2 to adopt the TP in the annex as a baseline.

Proposal 3: If specific RACH prioritization is deemed necessary for an Access Identity, sets of scalingFactorBI and powerRampingStepHighPriority can be configured in system information.

# References

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements, Nokia, RAN#99

- TR 23.700-25, v18.1.0, Study on timing resiliency and TSC and URLLC enhancements, Rel-18

- S2-2301461, TR 23.700 KI#1 conclusion update, CR 0002, Nokia, Qualcomm, Huawei, ZTE, Samsung

- S2-2301463, Proposed method for Time Synchronization status reporting to UE(s), Nokia

- R2-2302106, Reply LS on Proposed method for Time Synchronization status reporting to UE(s), Nokia

- S1-231285, Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s), Nokia

- C1-232839, TS 24.501 version 18.3.1, CR 5191, UAC for RAN timing synchronization status change - RRC inactive UE, Ericsson, Nokia

- TS 38.331, version 18.5.0

# Annex with Text Proposals

This section contains a text proposal for the enhancement proposed in Option 1. At this stage the indicated changes are for TS 38.331 with the intention to show the principle.

## TP for TS 38.331

The text proposal in this section is based on the current version of TS 38.331 in [8].

START OF CHANGES

#### 5.3.14.5	Access barring check

The UE shall:

1> if one or more Access Identities equal to 1, 2, 11, 12, 13, 14, or 15 are indicated according to TS 24.501 [23], and

1> if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity contained in "UAC barring parameter" is set to zero:

2> consider the access attempt as allowed;

1> else:

2> if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN); and

2> if the bit corresponding to Access Identity 1 in the uac-BarringForAccessIdentity contained in the "UAC barring parameter" is set to zero:

3> consider the access attempt as allowed;

2> else if Access Identity 3 is indicated:

3> draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3> if 'rand' is lower than the value indicated by uac-BarringFactorForAI3 included in "UAC barring parameter":

4> consider the access attempt as allowed;

3> else:

4> consider the access attempt as barred;

2> else if Access Identity 4 is indicated:

3> draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3> if 'rand' is lower than the value indicated by uac-BarringFactorForAI4 included in "UAC barring parameter":

4> consider the access attempt as allowed;

3> else:

4> consider the access attempt as barred;

2> else:

3> draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3> if 'rand' is lower than the value indicated by uac-BarringFactor included in "UAC barring parameter":

4> consider the access attempt as allowed;

3> else:

4> consider the access attempt as barred;

1> if the access attempt is considered as barred:

2> if Access Identity 4 is indicated:

3> draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

3> start timer T390 for the Access Category with the timer value calculated as follows, using the uac-BarringTimeForAI4 included in "UAC barring parameter":

T390 = (0.7+ 0.6 * rand) * uac-BarringTimeForAI4.

2> else:

23> draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

23> start timer T390 for the Access Category with the timer value calculated as follows, using the uac-BarringTime included in "UAC barring parameter":

T390 = (0.7+ 0.6 * rand) * uac-BarringTime.

START OF NEXT CHANGES

This change extends the UAC BarringInfo for Rel-18, so that it can be extended with a UAS BarringInfoSet dedicated to, e.g., Access Identity 4, which is conditional on the timing resilience feature, allowing for a special configuration.

### 6.2.2	Message definitions

Unchanged parts omitted

END OF CHANGES