3GPP TSG-RAN WG2 #123 meeting	R2- 2307902

21st Aug–25th Aug 2023, Toulouse

Source:		NEC

Agenda item:		7.5.4.1	BSR enhancements for XR

Title:		Discussion on delay status reporting for XR

Document for:		Discussion and decision

# 1. Introduction

In RAN2#122 meeting, agreements [1] on BSR enhancements were made as below:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this. Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In this contribution, we will discuss the potential BSR enhancements for delay status reporting to improve the XR-specific capacity.

# 2. Discussion

  - 2.1 Delay status reporting (DSR)

In the previous meetings, it was agreed that data volume information associated with delay information (e.g. remaining time) should be reported in a MAC CE. Since both delayed data volume and total buffer size are useful for UL scheduling, the most straightforward way is to extend the current BSR MAC CE format to support delay status reporting (DSR), and the following information could be included in the new MAC CE:

- Remaining delay budget: The delay information of the buffered UL packet with the shortest remaining delay budget can be reported. How to derive the delay information of a UL packet shall be discussed in section 2.2.

- Delayed buffer size: the total amount of available data that is identified as delayed data (e.g., the size of buffered data whose remaining delay budget is lower than a threshold). These data should be scheduled as soon as possible to avoid the corresponding PDCP discard timer is expired. How to calculate the delayed buffer size shall be discussed in section 2.3.

- Total buffer size (i.e., legacy buffer size): the total size of all available data that includes both delayed and non-delayed data.

Proposal 1: A new BSR MAC CE format is introduced for delay status reporting, which includes delay information, delayed buffer size, total buffer size, etc.

  - 2.2 Remaining delay budget reporting

In RAN2#122 meeting, it was agreed that the remaining time is based on the PDCP discard timer and the reference time for the remaining time is determined from the point of the first transmission of the information.

Figure 1: delay information of a UL packet (Assuming that DSR is within a BSR)

Since both UE and the network know when the delay information (e.g., in a BSR) is initially transmitted based on the UL grant (i.e.,t3), UE could report “remaining delay budget” with value “t PDB – (t 3 – t 0)”, then the network can derive the exact remaining delay value as “t PDB – (t 3 – t 0) – (t 4 – t 3)”, where

- t 0 is the arrival time of packet from upper layer, or the start time of discard timer for the corresponding SDU.

- t 4 is the time when the delay information (e.g., BSR) is received by the network.

- t PDB is equal to the value of PDCP discard timer.

- “t 4 – t 3” is derived by the network itself.

Furthermore, if a PDU Set needs to be decoded/handled as a whole, all SDU/PDUs belonging to the PDU set should share the same arrival time, which is equal to the arrival time of the first arriving SDU/PDU belonging to the PDU set, and hence have the same remaining delay budget.

Proposal 2: RAN2 to report remaining delay budget with value “t PDB – (t 3 – t 0)”, where t PDB is equal to the value of PDCP discard timer.

To trade off the BSR reporting overhead versus the desired precision, the following report granularities can be considered when reporting delay information.

- Logical channel (LCH).

- Logical channel group (LCG).

Although per-LCH reporting has higher precision, it may bring more signalling overhead, especially when delay information of multiple LCHs need to be reported. Due to the fact that LCHs with similar QoS requirements (e.g., delay requirements) can be mapped to the same LCG, we think per-LCG reporting may be a better choice.

Proposal 3: RAN2 agrees to report delay status with the granularity of LCG.

With regard to per-LCG reporting, the network and UE should only care about LCHs for delay-sensitive traffic. When more than one LCHs are mapped to an LCG, the reported delay information should correspond to the data from the delay-sensitive LCH that has the shortest remaining delay budget.

Proposal 4: Delay information reported by UE should correspond to the data from the delay-sensitive LCH that has the shortest remaining delay budget.

  - 2.3 Delayed buffer size calculation

If delayed buffer size is reported with the granularity of LCG, UE needs to determine whether the size of a packet should be included when calculating delayed buffer size. For this purpose, a delay threshold can be configured per delay-sensitive LCH or LCG. If the remaining delay budget of the packet is lower than the threshold, then the packet is considered as delayed data and shall be counted when calculating delayed buffer size for an LCG. Otherwise, the packet is considered as non-delayed data.

Proposal 5: A threshold for distinguishing delayed data and non-delayed data can be configured per delay-sensitive LCH or LCG for delayed buffer size calculation.

  - 2.4 New trigger for delayed BSR

According to the specification TS 38.321, A BSR shall be triggered if any of the events below occur:

A BSR shall be triggered if any of the following events occur for activated cell group: - UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either - this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or - none of the logical channels which belong to an LCG contains any available UL data. in which case the BSR is referred below to as 'Regular BSR'; - UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR'; - retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR'; - periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'. NOTE 1: When Regular BSR triggering events occur for multiple logical channels simultaneously, each logical channel triggers one separate Regular BSR.

As we can see, there is no delay-related BSR trigger mechanism at present. In order to let the network know the delay information as soon as possible, we propose a new BSR trigger event, which can be referred to as ‘Delayed BSR’, is introduced. The Delayed BSR should be triggered when the remaining delay budget of a packet is less than a configured threshold.

Furthermore, since multiple events may trigger a BSR, and a MAC PDU shall contain at most one BSR MAC CE, we propose that the delayed BSR shall have precedence over the Regular and the Periodic BSR, and hence over the padding BSR.

Proposal 6: “Delayed BSR” is triggered when the remaining delay budget of a packet is less than a configured threshold.

Proposal 7: Delayed BSR has precedence over the Regular and the Periodic BSR.

# 3. Conclusion

In this paper, we provide our views on BSR enhancement for XR traffic, and have the following proposals:

Proposal 1: A new BSR MAC CE format is introduced for delay status reporting, which includes delay information, delayed buffer size, total buffer size, etc.

Proposal 2: RAN2 to report remaining delay budget with value “t PDB – (t 3 – t 0)”, where t PDB is equal to the value of PDCP discard timer.

Proposal 3: RAN2 agrees to report delay status with the granularity of LCG.

Proposal 4: Delay information reported by UE should correspond to the data from the delay-sensitive LCH that has the shortest remaining delay budget.

Proposal 5: A threshold for distinguishing delayed data and non-delayed data can be configured per delay-sensitive LCH or LCG for delayed buffer size calculation.

Proposal 6: “Delayed BSR” is triggered when the remaining delay budget of a packet is less than a configured threshold.

Proposal 7: Delayed BSR has precedence over the Regular and the Periodic BSR.

# Reference

[1]. Report of 3GPP TSG RAN WG2 meeting #122

[2]. TS 38.321, Medium Access Control (MAC) protocol specification, v17.5.0