3GPP TSG-RAN WG2 #123 meeting	R2- 2307910

21st Aug–25th Aug 2023, Toulouse

Source:		NEC

Agenda item:		7.12.2.1 Connected mode

Title:		Discussion on mobility enhancements for mobile IAB

Document for:		Discussion and decision

# 1. Introduction

In RAN2#122 meeting, agreements [1] on connected mode for mobile IAB were made as below:

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). for the UL grant and HO completion in RACH-less HO: - 1. Both type-1 configured grant and dynamic grant are supported - 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

In this contribution, we will provide our views on mobility enhancement for mobile IAB.

# 2. Discussion

2.1 RACH-less HO

In the last meeting, explicit beam indication has been agreed for beam management during RACH-less HO. Since the relative position between on-board UE and IAB-node is relatively stationary, the indication could only focus on the case that beam information is not changed during the handover, which is similar to the TA solution which covers only the case of the same TA. If the indication is set, the latest beam information for the corresponding signal or channel shall be maintained and applied for the target cell of handover until new beam info is specified.

Proposal 1: RAN2 agrees that the beam indication only indicates whether beam information shall be maintained during RACH-less HO.

According to the agreements of the last meeting, both type-1 configured grant and dynamic grant can be used to transmit RRCReconfigurationComplete message. If the pre-allocated uplink grant is configured, UE shall use it to transmit the confirmation message. Otherwise, UE shall monitor PDCCH in the target cell for the dynamic grant.

With regard to the pre-allocated uplink grant, we think LTE RACH-less mechanism can be used as a baseline, for which per-allocated uplink grant is indicated in RACH-Skip-r14.

RACH-Skip-r14 ::=					SEQUENCE {

...

ul-ConfigInfo-r14				SEQUENCE {

numberOfConfUL-Processes-r14	INTEGER (1..8),

ul-SchedInterval-r14			ENUMERATED {sf2, sf5, sf10},

ul-StartSubframe-r14			INTEGER (0..9),

ul-Grant-r14					BIT STRING (SIZE (16))

}																OPTIONAL	-- Need OR

Similarly, the number of configured HARQ processes, time reference slot, scheduling interval (or periodicity in the unit of slot), and UL grant information are helpful for RACH-less HO in NR. With regard to the UL grant information, the size of the field should be 27 bits which is equal to the size of RAR UL grant. In addition, repetition may be considered for the PUSCH transmission to improve the reliability of confirmation message.

Proposal 2: For pre-allocated UL grant, the following information can be indicated to UE via RRC signalling,

- The number of configured HARQ processes;

- Time reference slot (i.e, start slot);

- Scheduling interval or periodicity;

- UL grant with the size of 27 bits;

- Repetition.

According to the design of LTE RACH-less HO, the handover procedure is completed upon reception of UE contention resolution identity MAC CE. If the MAC CE is received, UE shall stop T304 timer and release the RACH-less HO configuration (i.e., rach-Skip). However, there is a small drawback here. Even if there is no buffered DL data, the network needs to schedule a PDSCH resource for the MAC CE.

A similar issue has been discussed in NTN and the following agreements were reached during RAN2#122 meeting.

2. Release pre-allocated UL grant after RACH-less HO completion 3. LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

From our perspective, PDCCH addressed to the C-RNTI is a better choice to overcome the issue. If a PDCCH addressed to the C-RNTI is received from the target cell, the RACH-less HO is successfully completed and pre-allocated resources shall be released.

Proposal 3: PDCCH addressed to the C-RNTI can be used to indicate the successful completion of RACH-less HO.

2.2 CHO

To control the time window of CHO in mIAB, condEvent T1, which is only supported by NTN currently, was discussed in the last meeting. In this way, the CHO execution timing across UEs can be distributed to mitigate the signalling congestion.

However, condEvent T1 is based on absolute UTC time and is always configured together with events A3/A4/A5. To support CHO in mIAB, the execution requirements of condEventT1 need to be relaxed. Since there is a minor change needed for the specification (i.e., to make condEvent T1 applicable in TN), RAN2 may consider applying condEvent T1 for TN if CHO is supported.

Furthermore, time-based CHO is under discussion in the NES topic, we may wait for the progress of NES to make the final decision.

Proposal 4: RAN2 considers applying condEvent T1 for TN, or waiting for the progress of NES WI.

# Summary

This contribution provided our analysis on mobility enhancement for mobile IAB, and we propose to list all these possible solutions and the issues each solution can solve:

Proposal 1: RAN2 agrees that the beam indication only indicates whether beam information shall be maintained during RACH-less HO.

Proposal 2: For pre-allocated UL grant, the following information can be indicated to UE via RRC signalling,

- The number of configured HARQ processes;

- Time reference slot (i.e, start slot);

- Scheduling interval or periodicity;

- UL grant with the size of 27 bits;

- Repetition.

Proposal 3: PDCCH addressed to the C-RNTI can be used to indicate the successful completion of RACH-less HO.

Proposal 4: RAN2 considers applying condEvent T1 for TN, or waiting for the progress of NES WI.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #122

[2] TS 38.321, Medium Access Control (MAC) protocol specification, V17.5.0