3GPP TSG-RAN WG2 Meeting #123	R2- 2308028

Toulouse, France, August 21-25, 2023                       revision of R2- 2305719

Agenda item:		7.20.2

Source:		Lenovo

Title:		Discussion on the impacts of Two TAs for multi-DCI multi-TRP operation

Document for:		Discussion and Decision

# Introduction

The related discussion has been kicked off and some agreements have been achieved in RAN2#122 [1].

Agreement

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

RAN2 confirm the following working assumption as an agreement:

- Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles:

    - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs).

- The following information can be indicated by the MAC CE (for joint DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s). FFS for the separate DL/UL TCI mode.

In this contribution, we discuss RAN2 impacts of two TAs for multi-DCI multi-TRP operation in New Radio (NR).

# Discussion

In order to support the transmission in multi-DCI multi-TRP operation, the architecture shall be discussed. Referring to the existing architecture, one TA for one TAG is managed by the MAC entity. And a Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

Considering the multi-TRP operation, it will be complex when one of the TAG of SpCell expires if the TAG model is defined that only one TAG of SpCell is PTAG and the other TAG of SpCell is STAG. While the 2-PTAG model i.e., both TAGs of SpCell are PTAGs, can be simpler for this case.

Proposal 1: 2-PTAG model, i.e., both TAGs of SpCell can be supported.

In legacy, the MAC entity maintains only one TA for a TAG. The gNB provides the TA to the UE by the TAC MAC CE or by the RAR message. The received Timing Advance Command is applied for the indicated TAG. With the introduction of two TAs, it could be possible that the two TAs need to be updated especially for the intra-cell case. It may work if the UE receives two updated TAC one by one. But it introduces more signaling overhead comparing with two updated TACs being received one time.

Proposal 2: RAN2 to discuss whether to support two TAs are updated via a single TAC.

With the introduction of two TAs, it is not clear what is criteria on the resource validity of a cell. For example, CG #1 is associated with TRP #1 and CG #2 is associated with TRP #2, while TAT #1 is associated with TRP #1 and TAT #2 is associated with TRP #2. Then should the configured uplink grants #1 and #2 be cleared if only one TAT expires?

Considering the UE can keep UL transmission with only one TA valid, also considering it is possible that the unsynchronized TRP can be recovered by the PDCCH ordered RACH procedure, the UL resource associated with the synchronized TRP can be considered as valid. The UL is considered as unsynchronized only when both of the TATs corresponding to the multi-TRP expire.

Proposal 3: The configured grants are considered as invalid when uplink time is not aligned any longer.

In handover case, the configuration for handover is generated by candidate cell. If multi-TRPs with two TAs can be configured for candidate cell e.g PCell or PSCell, UE is expected to synchronize to candidate cell via two TAs. That means UE need to get two TAs when T304 is running via two RACH procedures. Another way is that only one TA for candidate Pcell or PSCell is configured by candidate cell. The second TRP associated with a second TA can be configured after successful handover. The latter will have additional delay.

Proposal 4: RAN2 to discuss whether multi-TRP with two TAs can be configured to UE in handover case.

# Conclusion

In this contribution, we discuss RAN2 impacts of two TAs for multi-DCI multi-TRP operation in New Radio (NR) and provide the following proposals:

Proposal 1: 2-PTAG model, i.e., both TAGs of SpCell can be supported.

Proposal 2: RAN2 to discuss whether to support two TAs are updated via a single TAC.

Proposal 3: The configured grants are considered as invalid when uplink time is not aligned any longer.

Proposal 4: RAN2 to discuss whether multi-TRP with two TAs can be configured to UE in handover case.

# References

RAN2#122 chairman notes