3GPP TSG-RAN WG2 Meeting #123                                    R2-2308042

Toulouse, France, August 21-25, 2023

Agenda Item:	7.4.3

Source: 	OPPO

Title:  	Open issues for subsequent CPAC in NR-DC

Document for:	Discussion and Decision

Introduction

In last meeting, the following agreements have been reached for subsequent CPAC regarding to candidate cell configuration:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

In this contribution, we would like to keep discussing the open issues for subsequent CPAC, mainly including reference configuration, candidate cell configurations and security update.

Discussion

Candidate configurations

In R16/17 CPAC, conditional reconfiguration is applied for candidate cell configuration. subsequent CPAC may reuse legacy conditional reconfiguration way to provide the candidate cells configuration. One potential issue is that how to differentiate the candidate cell for subsequent CPAC and legacy conditional PScell change/addition. Additional information to indicate the candidate cell type may be required in this manner. To be more flexible, the indication can be provided per candidate cell.

Explicit indication is introduced to differentiate whether the candidate cell is a subsequent CPAC candidate or R16/R17 CPAC candidate. And the indication is provided per candidate cell.

With pre-configured candidate cells, how to maintain the candidate configurations for subsequent CPAC should be studied. In general, we understand that it is NW responsibility to guarantee the validity of candidate cell configuration, for example, NW can modify/release the candidate cell configuration if it is invalid. And RAN2 also reached consensus on releasing CPC/CPA configurations by explicitly NW indication for inter-MN Pcell change scenario.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

Further study is required for other cases to release the configuration for subsequent CPAC, we put the cases into two categories as shown below:

NW controlled:

- upon SCG is released

- upon reception of RRC release

- upon intra-MN Pcell change

UE triggered:

- upon RRC re-establishment

- upon going to RRC_IDLE

For NW controlled case, we understand it can leave to NW on whether to release the subsequent CPAC configuration. For example, NW can indicate UE to release subsequent CPAC configuration together with MR-DC release message if NW prefers to keep UE in single connection mode, otherwise, subsequent CPAC configuration can be maintained to assist UE perform fast DC setup. For UE triggered case, UE can autonomous release the subsequent CPAC configurations accordingly.

In previous meeting, we agreed that the subsequent CPAC configuration can be released by NW through legacy RRC reconfiguration procedure, for example, by using condReconfigToRemoveList-r16 to release the subsequent CPAC configuration. While additional signalling overheads may be introduced especially for the cases that NW wants to remove the whole subsequent CPAC configuration. In that case, we should further study whether the subsequent CPAC configuration is released by 1-bit indicator or legacy condReconfigToRemoveList.

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

UE autonomously releases the subsequent CPAC configurations in the following cases: upon RRC re-establishment and upon going to RRC_IDLE.

UE relies on NW indication to release the subsequent CPAC configuration in the following cases: upon SCG is released, upon reception of RRC release and upon intra-MN Pcell change.

RAN2 further study whether the subsequent CPAC configuration is released by 1-bit indicator or legacy condReconfigToRemoveList.

One more issue is about the configuration type differentiation. For efficient use of candidate configurations, RAN2 assumes that a CPA conditional configuration can be used for CPC considering that CPA configuration always provides a complete RRC reconfiguration message to UE.

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

And for subsequent CPAC, as we already reach consensus on using delta configuration on top of reference configuration to form a complete candidate cell configuration, we see no difference for CPA candidate configuration and CPC candidate configuration since all candidate configuration will be formed as a complete configuration.

CPC conditional configuration can be used for CPA.

CPA/CPC can be used for subsequent CPC/CPA by default, i.e., no additional indication is introduced.

UE follow legacy to perform compliance check, i.e. it is up to UE implementation to perform compliance check upon reception of the subsequent CPAC configuration or upon CPA/CPC execution.

Reference configuration

Reference configuration is employed for subsequent CPAC to support delta candidate cell configuration, which is aiming to reduce the signalling overheads. The granularity of reference configuration has been discussed, and RAN2 assume single reference configuration is supported. There are still open issues on reference configuration handling.

- The first issue is about the RRC model for reference configuration. As reference configuration is used to assist UE to form a complete candidate configuration, and the reference configuration may also be a complete configuration. The RRC reconfiguration message can be used to provide the reference configuration.

RRC reconfiguration message is used to provide reference configuration. And the reference configuration can include MCG config.

- Regarding to the reference configuration generation, as we have agreed that both MN-initiated and SN-initiated subsequent CPAC are supported for Rel-18, the reference configuration maybe generated by different node. For SN- initiated case, source SN can initially generate the reference configuration, and provide it to other candidates. For MN-initiated case, as there is no serving SN available for reference configuration generation, MN can generate the reference configuration and deliver it to candidate SNs for candidate cell configuration preparation.

Reference configuration is initially generated by the node that triggers the subsequent CPAC procedure.

- Another question is about the reference configuration modification/update. As we know, reference configuration is used for all candidate to form a complete configuration for the subsequent CPAC execution. Modification of the reference configuration may result in reconfiguration of all candidate cell configuration; thus, frequently reference update should be prohibited.

- How to release the reference configuration is also FFS. Since only one reference config will be maintained by UE, if new reference configuration is received, UE will replace the stored reference configuration with the newly received one. And if the whole subsequent CPAC config is released, e.g., released by NW or UE autonomously released, the corresponding reference configuration should be released accordingly.

Reference configuration update based on delta config is not supported.

The reference configuration is released when the subsequent CPAC configuration is released.

And RAN2 has reached consensus on following similar design on reference configuration as in LTM.

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

Assume for now that there is only one reference configuration.

In previous meeting, LTM has agreed the following agreements:

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Whether the Reference configuration is a complete configuration or not is up to the network implementation.

Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

Reference config can be empty

In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check.

In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration.

As we agreed to follow similar design as LTM, we understand the related agreements can be confirmed for subsequent CPAC.

RAN2 confirm the following agreements for subsequent CPAC:

- The reference configuration is provided to all candidates involved in preparation;

- Whether the Reference configuration is a complete configuration or not is up to the network implementation;

- Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration;

- The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

- Reference config can be empty

- In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the subsequent CPAC configuration (before the CPAC execution). UE implementation can postpone that step to the CPAC execution;

- In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch);

Security update

Regarding the security update issue for inter-SN subsequent CPAC, SA3 has provided the solution on how to provide SN counter values as well as how to derive the key for SN in the LS reply.

SA3 wants to update on the state of the security analysis on selective SCG.

SA3 has considered different alternative solutions to address the issue and concluded to have a SN Counter based solution for fresh key derivation for selective SCG activation.

Following is the solution details from security perspective:

1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC.

2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

According to the solution provided by SA3, we understand the following information can be obtained and confirmed in RAN2:

Multiple sk counters associated with each SN are provided to UE in subsequent CPAC configuration.

Upon inter-SN subsequent CPAC, UE performs key derivation with an unused SN counter.

There are still open issues related to SN counter maintenance and key derivation:

- How UE maintains the preconfigured SK counters?

MN provides the SN counter values for subsequent inter-SN CPAC procedure key derivation, with the configured SN counters, UE shall store it along with subsequent CPAC configurations.

UE maintains the preconfigured SK counters for subsequent CPAC configuration.

- How to select the SK counter for key derivation among multiple pre-configured SK counter values?

According to the principle of legacy SN counter maintenance at MN, SN counter shall be increased monotonically:

The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

For subsequent CPAC, UE can follow similar way on SK counter selection. If multiple sk_counter is preconfigured for subsequent CPAC, UE select the SK counter with increasing order for key derivation.

UE selects the SK counter value with increasing order for security key update for subsequent CPAC.

- How UE handles the used SK counter value?

As mentioned in SA3 LS, the UE derives the S-KgNB using the KgNB together with an unused SN Counter value. For those SN counter values that have been used in previous CPAC procedures, UE shall consider them as invalid.

After sk_counter has been applied, UE considers the sk_counter as invalid.

- How to differentiate inter-SN and intra-SN scenario?

As we know, UE is required to update the security key if PDCP anchor changes, i.e. inter-SN CPAC. With the UE movement, the relationship (i.e., inter-SN and intra-SN) between source pscell and target pscell is unfixed. Considering UE has no knowledge of SN deployment, and NW cannot predict UE movement, inclusion of the inter-SN or intra-SN indication in candidate cell configuration is necessary. The indication may be also needed to configuration sk counters. For example, the indication can work as the pointer between candidate cells and the sk counters that can be used.

Introduce“inter-SN/intra-SN”indication to determine whether to perform key update  for subsequent CPAC.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	Explicit indication is introduced to differentiate whether the candidate cell is a subsequent CPAC candidate or R16/R17 CPAC candidate. And the indication is provided per candidate cell.

Proposal 2	UE autonomously releases the subsequent CPAC configurations in the following cases: upon RRC re-establishment and upon going to RRC_IDLE.

Proposal 3	UE relies on NW indication to release the subsequent CPAC configuration in the following cases: upon SCG is released, upon reception of RRC release and upon intra-MN Pcell change.

Proposal 4	RAN2 further study whether the subsequent CPAC configuration is released by 1-bit indicator or legacy condReconfigToRemoveList.

Proposal 5	CPC conditional configuration can be used for CPA.

Proposal 6	CPA/CPC can be used for subsequent CPC/CPA by default, i.e., no additional indication is introduced.

Proposal 7	UE follow legacy to perform compliance check, i.e. it is up to UE implementation to perform compliance check upon reception of the subsequent CPAC configuration or upon CPA/CPC execution.

Proposal 8	RRC reconfiguration message is used to provide reference configuration. And the reference configuration can include MCG config.

Proposal 9	Reference configuration is initially generated by the node that triggers the subsequent CPAC procedure.

Proposal 10	Reference configuration update based on delta config is not supported.

Proposal 11	The reference configuration is released when the subsequent CPAC configuration is released.

Proposal 12	RAN2 confirm the following agreements for subsequent CPAC:

	The reference configuration is provided to all candidates involved in preparation;

	Whether the Reference configuration is a complete configuration or not is up to the network implementation;

	Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration;

	In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the subsequent CPAC configuration (before the CPAC execution). UE implementation can postpone that step to the CPAC execution;

	In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch);

Proposal 13	Multiple sk counters associated with each SN are provided to UE in subsequent CPAC configuration.

Proposal 14	Upon inter-SN subsequent CPAC, UE performs key derivation with an unused SN counter.

Proposal 15	UE maintains the preconfigured SK counters for subsequent CPAC configuration.

Proposal 16	UE selects the SK counter value with increasing order for security key update for subsequent CPAC.

Proposal 17	After sk_counter has been applied, UE considers the sk_counter as invalid.

Proposal 18	Introduce“inter-SN/intra-SN”indication to determine whether to perform key update  for subsequent CPAC.

Reference

- RP-222332 Further NR Mobility Enhancements    MediaTek Inc