3GPP TSG RAN WG2 Meeting #123                                                         R2-2308073

# Toulouse, France, August 21-25, 2023

Agenda item:	7.5.5

Source:	Intel Corporation

Title:	UE Capabilities for Rel-18 XR WI

Document for:	Discussion and decision

# Introduction

UE radio capabilities for Rel-18 XR are discussed here considering the WI objectives [1] and RAN2 agreements. For RAN1 led topics, the suggestion is to wait until the details are added in RAN1 feature list.

# Discussion

## XR awanress in RAN

### UE identificaiton of XR related information to PDU Set and Data Burst

From the beginning of Rel-18 XR SI phase, RAN2 understanding is that the same PDU set related information is available at the UE for UL. This expected UE behaviour was informed by RAN2 to SA2 and SA4 in multiple LS as shown below:

“RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions.” [2]

”Furthermore, RAN2 has also agreed that the UE needs to be able to identify PDU Sets / Bursts but that in-band marking of PDUs in the uplink over Uu (the equivalent of downlink marking in GTP headers) is not needed.” [3]

“RAN2 considers that the PDU set concept is applicable to both UL and DL.” [4]

Neither SA2 or SA4 has raised any concern on this regard when responding to the corresponding LSs. Therefore, RAN2 assumption on the applicability of PDU set related information to the UL XR traffic seems reasonable.

During Rel-18 XR SI phase, RAN2 informed SA2 and SA4 multiple times the assumption that PDU set concept is applicable to UL side and UE is able to identify the corresponding PDU set related information. By not responding to this, RAN2 understands that there is no concern/issue identified by SA2 and SA4 on this regard.

The following is an agreement captured in last RAN2#122 meeting on this topic:

- On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

Running CR to 38.300 [5] introduces the term of XR-Awareness as follows:

16.X.2	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information (see TS 23.501 [3]). Optional PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow:

-	PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). A QoS Flow is associated with only one PSDB, and when available, it applies to both DL and UL and supersedes the PDB of the QoS flow.

-	PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. A QoS Flow is associated with only one PSER, and when available, it applies to both DL and UL and supersedes the PER of the QoS flow.

NOTE 1:	In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

-	PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

NOTE 2:	The PDU Set QoS parameters are common for all PDU Sets within a QoS flow.

In addition, the UPF can identify PDUs that belong to PDU Sets, and may determine the following PDU Set Information which it sends to the gNB in the GTP-U header:

-	PDU Set Sequence Number;

-	Indication of End PDU of the PDU Set;

-	PDU Sequence Number within a PDU Set;

-	PDU Set Size in bytes;

-	PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

Traffic assistance information may also be provided by 5GC to the gNB:

-	Via TSCAI:

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

-	Indication of End of Data Burst in the GTP-U header of the last PDU in downlink.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation.

From AS point of view, RAN2 does not need to discuss whether or how this XR awareness is visible to the UE. This kind of discussion should be left up to SA2 and SA4, e.g. whether similar upper layers add PDU set related information in each packet vs leaving these details up to UE implementation. Each new mechanism defined as part of Rel-18 WI would have its own specific new UE capability (as discussed in next sections of this document). In addition, it seems preferable if another new capability is defined to indicate whether UE has visibility (or not) to the XR awareness including PDU set information/concept as this might not always be required for the applicable feature(s) as discussed below. The proposed section on XR awareness for TS 38.300 covers both PDU Set concept/information as well as data burst. Therefore it does not seem essential to have a separate UE capability for this.

UE AS layer has visibility to XR awareness (including PDU set concept/information and data burst) for UL as it is currently defined by SA2 for DL traffic.

- A new optional radio capability (e.g., supportOfXR-Awareness) is defined to identify UE supporting XR awareness for UL traffic as it is defined in TS 38.300 §16.x.2 (which includes the PDU set concept/QoS parameters/information and data burst one).

### UE assistance of XR related information to gNB

The following are RAN2 agreements related to the new UE assistance information for XR UL traffic:

RAN2#122

- UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

- Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

- UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

- RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

- Do not use PIN delay budget request for jitter reporting for XR services.

- Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN.

RAN2#121bis

- UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL).

RAN2 agreed that UE can provide XR related information associated to jitter and burst arrival time (leaving FFS whether to also provide periodicity) as part of UE Assistance Information (UAI).

Running CR to 38.300 [5] introduces the new UAI as follows:

Reporting of uplink assistance information (jitter range and burst arrival time) per QoS flow by the UE via UE Assistance Information.

For all of this, at least a new UE capability seems required as the UE could provide this new assistance information only if it is configured to do so, and network should only configure it if the UE supports it. The open question is whether a single capability indication is sufficient or different ones are preferable for each of the different new UE assistance information. This capability could be tied with the XR-awareness explained in previous 0, however this requirement does not seems strictly required from functionality point of view.

RAN2 agreed for UE to provide UL jitter and bust arrival information to RAN and it is FFS whether UE might also be defined other ones such as periodicity.

- A new optional UE capability signaling is defined for the new UE assistance information related to XR traffic requires (e.g., supportOfXR-AssistanceInfo).

  - To discuss whether different capabilities are required to indicate UE’s support for each of the new XR related assistance information, e.g., one for the support of providing UL jitter and another for burst arrival time.

  - To discuss whether option (a) supportXR-AssistanceInfo is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness) or option (b) this supportXR-AssistanceInfo should be defined as an optional capability only if UE also supports supportOfXR-Awareness.

## Capacity related enhancements

### Discard operation of PDU Set

RAN2 agreements to enhance discard operation for XR traffic considering the PDU set concept are as follows:

RAN2 #122

- PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

- Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

- FFS how/whether to minimize additional UL signalling after this indication.

- FFS if the NW indication is a one-shot or also subsequent packets.

RAN2 #121bis-e

- PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

RAN2 #121

- RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion.

RAN2 #120

- RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set.

A new UE capability would be required to identify UEs that can perform this new discard enhancement. Further granularity does not seem essential. However, it seems necessary that UE is able to identify PDU set related concept/information (i.e., UE shall support supportOfXR-Awareness explained in previous Proposal 1).

- A new optional UE capability signaling (e.g., supportOfDiscardPDU-Set) is defined to identify Rel-18 UEs supporting discard operation associated with the PDU Set concept.

  - UE supporting supportOfDiscardPDU-Set shall also support supportOfXR-Awareness (i.e.,  supportOfDiscardPDU-Set is optional capability only if UE also supports supportOfXR-Awarenessi).

### BSR enhancements (including new BS table(s))

RAN2 agreements to enhance BSR defining new BS table(s) for XR traffic are as follows:

RAN2 #122

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

- We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

RAN2 #121bis-e

- Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

  - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs.

  - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- At least linear distribution is used for generating code points in new BSR table(s).  FFS whether exponential distribution can be considered too.  FFS if piecewise linear distribution is supported.

  - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size.

RAN2 #121

- New BSR tables are fixed (=specified) or semi-static (RRC-based).

- FFS how many BSR tables are defined.

RAN2 #120

- RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase.

- RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

Running CR to 38.300 [5] introduces the new BSR table as follows:

-	One additional BS table to reduce the quantisation errors in BSR reporting (e.g. for high bit rates):

-	The code points of this new table follow a linear distribution;

Editor's Notes: can consider piecewise linearity when discussing how the BSR table values are defined.

-	The gNB configures the BS table(s) that an LCG is eligible to use, and when there is more than one, the UE selects the table (criteria FFS).

-	Additional BSR triggering conditions to allow timely availability of buffer status information can be investigated further.

A new UE capability would be required to identify UEs that can use this new BSR enhancement. It is also FFS whether other BSR related enhancements are defined (e.g., new BSR trigger conditions) therefore this is not considered for now. In addition, it does not seem required from functionality to tie the support of this feature with the support of XR awareness previously discussed.

- A new optional UE capability signaling (e.g., supportOfNewBS-Table) is defined to identify Rel-18 UEs supporting BSR enhancements associated with the new BS tables.

  - This supportOfNewBS-Table is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

### Delay reporting of buffered data

RAN2 agreement to enhance delay reporting of buffered data is as follows:

RAN2 #122

- UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

RAN2 #120

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

This new reporting may happen via BSR however there is no functional need to tie its capability support with the other ones. Therefore, UE’s reporting of delay related information could have its own capability. However, this feature would require that UE also supports XR awareness.

- A new optional UE capability signaling (e.g., supportOfDelayReporting) is defined to identify Rel-18 UEs supporting the delay reporting of the buffered data.

  - UE supporting supportOfDelayReporting shall also support supportOfXR-Awareness (i.e.,  supportOfDiscardPDU-Set is optional capability only if UE also supports supportOfXR-Awarenessi).

### Retransmission less CG enhancement

RAN2 agreement to enable retransmission less CG enhancement is as follows:

RAN2 #122

- For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced:

- A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration;

- Changes in the procedural text of DRX operations for CG in the MAC specification;

- A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.

RAN2 still needs to discuss whether retransmission less CG enhancement is associated or not to any of the other features defined for XR. In our understanding there is no need to tie this feature with other ones defined in this WI and could be defined as a standalone feature.

- A new optional UE capability signaling (e.g., supportOfDisableHARQ-RTT-CG) is defined to identify UE supporting retransmission-less CG enhancement (which allows disabling the HARQ RTT timer per CG configuration).

  - This supportOfDisableHARQ-RTT-CG is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

### RAN1 lead topics: Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and Dynamic indication of unused CG PUSCH occasion(s) based on UCI

Most likely two new UE capabilities will be needed for the two features currently under discussion by RAN1, however we suggest waiting for RAN1 input on its associated requirements to be provided within ongoing RAN1 feature list.

- Wait for RAN1 input on new UE capabilities for RAN1 lead objectives (i.e., Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and Dynamic indication of unused CG PUSCH occasion(s) based on UCI). NOTE: The corresponding TP would be included in the mega capability CR.

## Power saving related enhancements

### C-DRX enhancements for XR traffic with non-integer periodicities

RAN2 agreements to enhance C-DRX in order to better support XR frame rates with non-integer periodicities are as follows:

RAN2 #122

- Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

- Not use broadcast signalling for counter and reference SFN in XR.

RAN2 #121bis-e

- RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

RAN2 #121

- Companies should evaluate the RAN2 specification impacts and any other RAN2 aspects of their proposals for XR DRX.

- Companies should evaluate the (high-level) impacts to RAN1/4 specification from their proposals for XR DRX.

- Companies should try to coordinate with each other offline and bring joint proposals to next meeting. RAN2 aims to exclude proposals with least support in the next meeting.

- Companies should evaluate the RAN2 specification impacts and any other RAN2 aspects of their proposals for SFN wrap-around.

- Same as for DRX solutions, companies should try to coordinate with each other offline and bring joint proposals to next meeting. RAN2 aims to exclude proposals with least support in the next meeting.

RAN2 #120

- RAN2 aims to allow XR frame rates that correspond to non-integer periodicities in at least semi-static manner (e.g. RRC). Details can be left to WI phase.

RAN2 #119bis-e

- At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

A new UE capability would be required to identify UEs that can perform this new C-DRX enhancements for XR traffic. Further granularity does not seem essential and could be defined as a standalone feature.

- A new optional UE capability signaling (e.g., supportOfRationalDRX) is defined to identify Rel-18 UEs supporting C-DRX enhancements targeting any traffic with non-integer periodicity.

  - This supportOfRationalDRX is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

# Conclusion

The observations captured are the following:

Observation 1.	During Rel-18 XR SI phase, RAN2 informed SA2 and SA4 multiple times the assumption that PDU set concept is applicable to UL side and UE is able to identify the corresponding PDU set related information. By not responding to this, RAN2 understands that there is no concern/issue identified by SA2 and SA4 on this regard.

Observation 2.	UE AS layer has visibility to XR awareness (including PDU set concept/information and data burst) for UL as it is currently defined by SA2 for DL traffic.

Observation 3.	RAN2 agreed for UE to provide UL jitter and bust arrival information to RAN and it is FFS whether UE might also be defined other ones such as periodicity.

The proposals captured are the following:

Proposal 1.	A new optional radio capability (e.g., supportOfXR-Awareness) is defined to identify UE supporting XR awareness for UL traffic as it is defined in TS 38.300 §16.x.2 (which includes the PDU set concept/QoS parameters/information and data burst one).

Proposal 2.	A new optional UE capability signaling is defined for the new UE assistance information related to XR traffic requires (e.g., supportOfXR-AssistanceInfo).

Proposal 2.1.	To discuss whether different capabilities are required to indicate UE’s support for each of the new XR related assistance information, e.g., one for the support of providing UL jitter and another for burst arrival time.

Proposal 2.2.	To discuss whether option (a) supportXR-AssistanceInfo is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness) or option (b) this supportXR-AssistanceInfo should be defined as an optional capability only if UE also supports supportOfXR-Awareness.

Proposal 3.	A new optional UE capability signaling (e.g., supportOfDiscardPDU-Set) is defined to identify Rel-18 UEs supporting discard operation associated with the PDU Set concept.

Proposal 3.1.	UE supporting supportOfDiscardPDU-Set shall also support supportOfXR-Awareness (i.e.,  supportOfDiscardPDU-Set is optional capability only if UE also supports supportOfXR-Awarenessi).

Proposal 4.	A new optional UE capability signaling (e.g., supportOfNewBS-Table) is defined to identify Rel-18 UEs supporting BSR enhancements associated with the new BS tables.

Proposal 4.1.	This supportOfNewBS-Table is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

Proposal 5.	A new optional UE capability signaling (e.g., supportOfDelayReporting) is defined to identify Rel-18 UEs supporting the delay reporting of the buffered data.

Proposal 5.1.	UE supporting supportOfDelayReporting shall also support supportOfXR-Awareness (i.e.,  supportOfDiscardPDU-Set is optional capability only if UE also supports supportOfXR-Awarenessi).

Proposal 6.	A new optional UE capability signaling (e.g., supportOfDisableHARQ-RTT-CG) is defined to identify UE supporting retransmission-less CG enhancement (which allows disabling the HARQ RTT timer per CG configuration).

Proposal 6.1.	This supportOfDisableHARQ-RTT-CG is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

Proposal 7.	Wait for RAN1 input on new UE capabilities for RAN1 lead objectives (i.e., Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and Dynamic indication of unused CG PUSCH occasion(s) based on UCI). NOTE: The corresponding TP would be included in the mega capability CR.

Proposal 8.	A new optional UE capability signaling (e.g., supportOfRationalDRX) is defined to identify Rel-18 UEs supporting C-DRX enhancements targeting any traffic with non-integer periodicity.

Proposal 8.1.	This supportOfRationalDRX is defined as a standalone feature (i.e., it does not require that UE also supports supportOfXR-Awareness).

# Reference

- RP-230786, Updated WID on XR Enhancements for NR, Rel-18, March 2023.

- R2- 2209215, LS on XR-awareness in RAN, LS, From RAN2, To SA2, August 2022

- R2-2213225, LS on PDU Set Handling, LS, From RAN2, To SA2, SA4, November 2022.

- R2-2302010, Reply LS on PDU Set Handling, From RAN2, To SA2, CCing SA4, RAN3, March 2023.

- R2-2306920, XR Enhancements, running CR to TS 38.300, May 2023.