3GPP TSG RAN WG2 Meeting #123                                                           R2-2308092

# Toulouse, France, August 21-25, 2023

Agenda item:	7.7.1

Source:	Intel Corporation

Title:	UE Capability Discussion for Rel-18 NR NTN Enhancements

Document for:	Discussion and decision

# Introduction

For Rel-18 NR NTN enhancements [1], some new features have been agreed, e.g., network verified UE location, location-based measurement initiation in Earth-moving cells, and RACH-less handover. This document discuss which UE capabilities can be defined accordingly and also takes into consideration RAN1 inputs in LS [2] about the updated RAN1 feature list [3] and RAN4 latest agreements [4].

# Discussion

## RAN1-lead features

RAN1 sent an LS [2] with the updated RAN1 feature list [3] and the following information was provided for Rel-18 eRedCap. It is highlighted in yellow those that are still under discussion by RAN1. The TP associated to any capability included in RAN1 feature list [3] will be included in the Mega-Capability CR, however it is important that the working groups discuss any unclear point, if any, to easy the CR process.

| # | 44-1 | 44-2 | 44-3 |
| --- | --- | --- | --- |
| Group | PUCCH repetition for Msg4 HARQ-ACK | NTN DMRS bundling enhancement for PUSCH | UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN |
| Components | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource 2. Support receiving repetition factor in system information [3. Support receiving repetition factor in DCI scheduling Msg4 PDSCH] [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] [5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration] | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit [3. Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.] | 1. Support UE Rx-Tx Measurement and Report for Multi-RTT positioning with single satellite in NTN [2. Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN] |
| Pre-req. |  | 30-4a/b | 13-4, 13-8 |
| Need for gNB to know if feature is supported | Yes | Yes | Yes |
| Applicable to signaling exchange btwn UEs (Sidelink) | No | No | No |
| Consequence if the feature is not supported by the UE | UE does not support PUCCH repetition for Msg4 HARQ-ACK | UE does not support DM-RS bundling enhancement for PUSCH in NTN | UE does not support Multi-RTT positioning with single satellite in NTN |
| Type | Per Band | Per Band | Per Band |
| FDD/ TDD diff | N/A | N/A | N/A |
| FR1/ FR2 diff | N/A | N/A | N/A |
| Mixed of FDD/TDD and/or FR1/FR2 | N/A | N/A | N/A |
| Note | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 [Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] |
| Mandatory/ Optional | Optional without capability signaling | Optional with capability signaling | Optional with capability signaling |

The following observations could be highlighted from RAN1 feature list:

RAN1 feature list have already included three new capabilities associated to Rel-18 NR NR NTN Enhancements: (FG 44-1) PUCCH repetition for Msg4 HARQ-ACK, (FG 44-2) NTN DMRS bundling enhancement for PUSCH and (FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN.

On network verified UE location, it is also important to point that RAN2 also took a working assumption which is aligned to RAN1 agreement as shown below:

RAN2#122 agreement:

Working Assumption:  1.	A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network.

RAN1 feature list is aligned to RAN2 working assumption that a new UE capability is used to indicate UE support of network verified location via (FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN.

The following proposals could be discussed for confirmation and checking on open points, if any:

- To check whether RAN2 needs any discussion on RAN1-centric capabilities included in their latest feature list:

  - (FG 44-1) PUCCH repetition for Msg4 HARQ-ACK defined as optional without capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223)..

  - (FG 44-2) NTN DMRS bundling enhancement for PUSCH defined as optional with capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223)..

  - (FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN defined as optional with capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223).

## TN-NTN cell reselection

Regarding the enhancements to TN-NTN cell reselection, the following agreement was made:

RAN2#119bis-e agreement:

UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

The intention is to save idle/inactive UE power consumption on TN neighbour cell measurements. And network needs to provide assistance information to help UE identify the TN area as agreed below:

RAN2#121 agreements:

- TN coverage area information will be associated to the frequency information.

- RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information):

Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

RAN2#121bis-e Agreements:

1.	For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

RAN2#122 Agreements:

1.	An RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency, regardless of the frequency priority

2.	We don’t introduce RRC dedicated signalling to provide more accurate TN coverage information

3	We no longer consider option 3 alone for signaling the frequency information for TN coverage area (in case option 3 should be combined with option 1). Come back in the next meeting to decide between option 2 (plus possible fixes if needed) and option 1+3.

To sum up, a UE supporting this feature needs to decode the corresponding assistance info. to identify the TN area, and the UE can skip the neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. RAN2#122 captured “if configured” however in our understanding this only refers to the broadcast signaling required for the UE to decide when skipping can be performed. Therefore, since this feature is for idle/inactive, it could be optional UE capability without signaling.

- To define an optional without signalling UE capability to indicate the support of skipping neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

## NTN-NTN cell reselection

Regarding NTN-NTN cell reselection enhancements, location based measurement initiation is supported in Earth-moving cell in R18 NR NTN, while location and time based measurement initiation in Quasi-earth fixed cell was supported in R17 NR NTN.

RAN2#121 agreement:

1.	For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell

Similar to R17 UE capability of location-based measurement initiation in Quasi-earth fixed cell, an optional UE capability without signalling can be defined for location based measurement initiation in Earth-moving cell.

- To define an optional without signalling UE capability for location-based measurement initiation in Earth-moving cell for cell selection/reselection.

The time-based measurement initiation is also agreed as below, and the corresponding UE capability can also be defined.

RAN2#121bis agreement:

For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

- To define an optional without signalling UE capability for time-based measurement initiation in Earth-moving cell for cell selection/reselection.

## RACH-less handover

Regarding R18 NR NTN handover enhancements, one confirmed new feature is RACH-less handover.

RAN2#121 Agreements:

Support RACH-less Handover in Rel-18.

And considering in RAN2#116bis-e meeting, a general agreement on FRX/XDD differentiation was made as follows.

RAN2#116bis-e agreement: From Rel-17 onwards, at least for new capabilities, if a UE capability requires at least FRx or at least xDD differentiation, it is defined with both FRx and xDD differentiation in per band signaling, i.e. no new UE capabilities will be defined in the FRX and XDD capability signaling branches.

Similar to CHO UE capabilities which has been defined as a per band capability, the RACH-less handover UE capability should be a per band capability. In addition to per band capability, whether the per UE capabilities need to be defined (like condHandoverFDD-TDD-r16 and condHandoverFR1-FR2-r16 for CHO), which indicate whether RACH-less handover between FR1 and FR2 is supported, and whether RACH-less handover between TDD and FDD is supported. The support of these cases would rely RAN4 input on the NTN bands above 10GHz. The latest RAN4 agreements [4] are as shown below which captures how RAN4 plans to define requirements for RACH-less (C)HO for Rel-18 NR NTN:

Issue 5-1: RACH-less (C)HO – RAN4 Agreements:

- RAN4 to define RACH-less HO requirements for intra-/inter-satellite handover with and without gateway/gNB switch.

  - Known and Unknown target cell condition:

    - Reuse the R17 known/unknown condition applied in NTN HO requirements.

    - If any other new aspects to be additionally added are identified based on RAN1/2 progress, those aspects will be further discussed in RAN4.

  - Whether to define the requirements for both known and unknown target cells

    - Consider both known and unknown target cells.

  - Whether to include fine time tracking latency in the HO latency requirement,

    - Fine time tracking latency will be included even when target cell is known.

  - Additional latency can be added, if identified and needed, based on the detailed design of the feature to be made by RAN1 and RAN2

- The detailed formula and exact values in the requirements, e.g. T_IU, T_SI-NR, T_RACH, T_delta, T_processing, T_margin, T_search, etc. if relevant, will be defined in the next meeting accordingly.

RAN4 has agreed to enable requirements for RACH-less handover requirements for intra-/inter-satellite handover with and without gateway/gNB switch.

- To define an optional with signalling UE capability per band for NTN RACH-less handover.

# Conclusion

The observations captured are the following:

Observation 1.	RAN1 feature list have already included three new capabilities associated to Rel-18 NR NR NTN Enhancements: (FG 44-1) PUCCH repetition for Msg4 HARQ-ACK, (FG 44-2) NTN DMRS bundling enhancement for PUSCH and (FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN.

Observation 2.	RAN1 feature list is aligned to RAN2 working assumption that a new UE capability is used to indicate UE support of network verified location via (FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN.

Observation 3.	RAN4 has agreed to enable requirements for RACH-less handover requirements for intra-/inter-satellite handover with and without gateway/gNB switch.

The proposals captured are the following:

Proposal 1.	To check whether RAN2 needs any discussion on RAN1-centric capabilities included in their latest feature list:

Proposal 1.1.	(FG 44-1) PUCCH repetition for Msg4 HARQ-ACK defined as optional without capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223)..

Proposal 1.2.	(FG 44-2) NTN DMRS bundling enhancement for PUSCH defined as optional with capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223)..

Proposal 1.3.	(FG 44-3) UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN defined as optional with capability signaling with the corresponding details explained in RAN1 feature list (R1-2306223).

Proposal 2.	To define an optional without signalling UE capability to indicate the support of skipping neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

Proposal 3.	To define an optional without signalling UE capability for location-based measurement initiation in Earth-moving cell for cell selection/reselection.

Proposal 4.	To define an optional without signalling UE capability for time-based measurement initiation in Earth-moving cell for cell selection/reselection.

Proposal 5.	To define an optional with signalling UE capability per band for NTN RACH-less handover.

# Reference

- RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales.

- R1-2306225, LS on Rel-18 RAN1 UE features list for NR after RAN1#113, From RAN1, To RAN2, Ccin RAN4, May 2023.

- R1-2306223, Updated RAN1 UE features list for Rel-18 NR after RAN1#113, May 2023.

- R4-2310168, WF on NR NTN enhancements RRM requirements, May 2024.