3GPP TSG-RAN WG2 Meeting #123								              R2-2308170

Toulouse, France, August 21-25, 2023

Agenda Item:	7.18.2

Source: 	SONY

Title:	Control plane aspects for MT-SDT

Document for:	Discussion and Decision

# Introduction

In the past few meetings, RAN2 discussed the Mobile Terminated-Small Data Transmission (MT-SDT) for Rel-18 UEs in RRC_INACTIVE state, and the following agreements were reached: RAN2#120 [2], RAN2#121 [3] and RAN2#122 [4]:

Agreements at RAN2#120:

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message. FFS if more information for MT-SDT are needed FFS what the indication will be called. FFS signalling details.

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline. The detailed triggers will be discussed on case by case. FFS on resources used for access.

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer. The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e., implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT. FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT. Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure. FFS stage 3 details.

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication.

Agreement at RAN2#121:

- Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE. FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS.

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6.  RBs configured for SDT are common for MO-SDT and MT-SDT.

7.  If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long.

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements at RAN2#121:

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e., if MT-SDT is initiated first the resume cause will be set to MT-SDT).

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initiation RRCResumeRequest for MT-SDT) using separate procedures.

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e., RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e., like legacy).

Agreement at RAN2#122:

- Allow support of only MT-SDT in a cell. A separate SIB configuration will be introduced. FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non-critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e., MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message.

- MT-SDT is only applicable to the legacy MT-Access use case (i.e., it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e., similar to MO-SDT).

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT.

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

In this contribution, we discuss some remaining issues of control plane aspects for MT-SDT procedures.

## Control plane aspects for MT-SDT

RAN2 agreed that to allow support of only MT-SDT in a cell, and in this case a separate SIB configuration will be introduced. For this scenario, because MO-SDT is not configured/supported, the UE will employ non-SDT random access resources for accessing the network so that the network can transfer MT-SDT to the UE.

In addition, even if MO-SDT is configured within the cell, when UL data is not available at the UE (i.e., MO-SDT is not initiated) and a paging message triggering MT-SDT is received, it is agreed that common RACH resources can be used to initiate/respond MT-SDT.

Based on the above scenarios, there could be two different methods to apply non-SDT random access resources:

- The UE employs legacy contention-based RA procedure (CBRA), but UE includes a new resume cause in RRC resume request and network will know that this is for MT-SDT transfer.

- The UE receives a dedicated random access preamble (CFRA) in the paging message to respond in the uplink. In this case, the gNB includes the dedicated random access preamble ID in the paging message.

We think for MT-SDT, a non-SDT random access resources for accessing the network should be indicated to the UE in the paging message where the random-access resource is based on either legacy contention-based RA procedure (CBRA) or dedicated random access preamble (CFRA).

Proposal 1: For MT-SDT, a non-SDT random access resources for accessing the network should be indicated to the UE in the paging message where the random-access resource is based on either legacy contention-based RA procedure (CBRA) or dedicated random access preamble (CFRA).

In Rel-17, the duration of SDT transmission is supervised by a timer (T319a), named as “SDT failure detection timer” which is configured by the network. If this timer expires, the UE goes to IDLE state. This timer ensures that SDT transmission/reception does not go on for ever and to recover in case of a connection failure. Hence, similarly for MT-SDT, the UE should apply the same timer to supervise the duration of SDT transmission/reception. In this case, after receiving a paging message indicating MT-SDT and upon first transmission of the CCCH message, i.e., when conditions for initiating MT-SDT are fulfilled, the UE should start timer T319a.

Proposal 2: When a UE receives a paging message indicating MT-SDT and upon first transmission of the CCCH message (i.e., when conditions for initiating MT-SDT are fulfilled), the UE should start timer T319a.

## Conclusions

In this contribution, we have discussed some remaining issues of control plane aspects for MT-SDT procedures. We have the following proposals:

Proposal 1: For MT-SDT, a non-SDT random access resources for accessing the network should be indicated to the UE in the paging message where the random-access resource is based on either legacy contention-based RA procedure (CBRA) or dedicated random access preamble (CFRA).

Proposal 2: When a UE receives a paging message indicating MT-SDT and upon first transmission of the CCCH message (i.e., when conditions for initiating MT-SDT are fulfilled), the UE should start timer T319a.

## References

[1] RP-213583, “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR,” RAN#94e.

[2] Minutes RAN2#120

[3] Minutes RAN2#121

[4] Minutes RAN2#122